| Document Title | Specification of CAN Interface |
| --- | --- |
| Document Owner | AUTOSAR |
| Document Responsibility | AUTOSAR |
| Document Identification No | 12 |

| Document Status | published |
| --- | --- |
| Part of AUTOSAR Standard | Classic Platform |
| Part of Standard Release | R21-11 |

| Document Change History |  |  |  |
| --- | --- | --- | --- |
| Date | Release | Changed by | Description |
| 2021-11-25 | R21-11 | AUTOSAR Release Management | - Support hardware-based timestamping - Minor corrections/clarifications - Editorial changes |
| 2020-11-30 | R20-11 | AUTOSAR Release Management | - Introduction of Security Events - Rework of Error classification chapters - Removal of pretended networking - Minor corrections/clarifications |
| 2019-11-28 | R19-11 | AUTOSAR Release Management | - Update reference to ISO 11898-1:2015 - Minor corrections - Editorial changes - Changed Document Status from Final to published |
| 2018-10-31 | 4.4.0 | AUTOSAR Release Management | - BusMirroring (CONC_634) - Receive Data Length Check per Pdu - Remove dummy implementations for Cancel Transmit APIs - Header File Cleanup |
| 2017-12-08 | 4.3.1 | AUTOSAR Release Management | - Introduction of Runtime errors - Replace Can_ReturnType with Std_ReturnType overlay - Minor corrections - Editorial changes |
| 2016-11-30 | 4.3.0 | AUTOSAR Release Management | - Remove CCMSM - Rework MetaData handling - Reliable TxConfirmation - Error Active/Passive State API |
| 2015-07-31 | 4.2.2 | AUTOSAR Release Management | - Clarified wakeup, buffering, transmit, and variants - Removed deprecated APIs - Editorial changes |
| 2014-10-31 | 4.2.1 | AUTOSAR Release Management | - Full CAN FD Support - Global Time Synchronization over CAN - Removed CanIf_CancelTxConfirmation - Small improvements |
| 2014-03-31 | 4.1.3 | AUTOSAR Release Management | - Removed BSW Exclusive areas - Set ICOM support to optional - Can_IdType handling - Small improvements |
| 2013-10-31 | 4.1.2 | AUTOSAR Release Management | - Restricted PDU mode changes - Removed critical section handling description in chapter 9 - Set CanIfInitRefCfgSet oboslete - Pretended Networking section - Small improvements |
| 2013-03-15 | 4.1.1 | AUTOSAR Administration | - CAN FD (without DLC extension) - Pretended Networking (ICOM) - Heavy Duty Vehicle (J1939) support - PduModes and PnTxFilter for clean wake-up - Relation between PDUs & HOHs - Post-build loadable concept |
| 2011-12-22 | 4.0.3 | AUTOSAR Administration | - Partial Networking Support - Improved Transmit Buffering - Improved Error Detection |
| 2009-12-18 | 4.0.1 | AUTOSAR Administration | - Updated chapters "Version Checking" and "Published Information" - Multiple CAN IDs could optionally be assigned to one I-PDU - Wake-up validation optionally only via NM PDUs - Asynch. mode indication call-backs instead of synch. mode changes - No automatic PDU channel mode change when CC mode changes - TxConfirmation state entered for BusOff Recovery - WakeupSourceRefIn and WakeupSourceRefOut - PduInfoPtr instead of SduDataPtr - Introduction of Can_GeneralTypes.h and Can_HwHandleType - Transceiver types of chapter 8. shifted to transceiver SWS |
| 2010-02-02 | 3.1.4 | AUTOSAR Administration | - HOH definition - abstracted ControllerId and TransceiverId - No changing of baudrate via CanIf and CanIf_ControllerInit - Dispatcher adapted because of CDD - TxBuffering: only one buffer per L-PDU - Wake up mechanism adapted to environment behavior (network -> controller/transceiver; wakeupSource) - Mode changes made asynchronous - no complete state machine in CanIf, just buffered states per controller - Legal disclaimer revised |
| 2008-08-13 | 3.1.1 | AUTOSAR Administration | Legal disclaimer revised |
| 2008-02-01 | 3.0.2 | AUTOSAR Administration | - Replaced chapter 10 content with generated tables from AUTOSAR MetaModel. |
| 2008-02-01 | 3.0.2 | AUTOSAR Administration | - Interface abstraction: network related interface changed into a controller related one - Wakeup mechanism completely reworked, APIs added & changed for Wakeup - Initialization changed (flat initialization) - Scheduled main functions skipped due to changed BSW Scheduler responsibility - Document meta information extended - Small layout adaptations made |
| 2007-12-21 | 3.0.1 | AUTOSAR Administration | - Header file structure changed - Support of mixed mode operation (Standard CAN & Extended CAN in parallel on one network) added - Support of CAN Transceiver API <User>_DlcErrorNotification deleted - Pre-compile/Link-Time/Post-Built definition for configuration parameters partly changed - Re-entrant interface call allowed for certain APIs - Support of AUTOSAR BSW Scheduler added - Support of memory mapping added - Configuration container structure reworked - Various of clarification extensions and corrections |
| 2006-05-16 | 2.0.0 | AUTOSAR Administration | Second Release |
| 2005-05-31 | 1.0.0 | AUTOSAR Administration | Initial Release |

## Disclaimer

This work (specification and/or software implementation) and the material contained in it, as released by AUTOSAR, is for the purpose of information only. AUTOSAR and the companies that have contributed to it shall not be liable for any use of the work.

The material contained in this work is protected by copyright and other types of intel- lectual property rights. The commercial exploitation of the material contained in this work requires a license to such intellectual property rights.

This work may be utilized or reproduced without any modification, in any form or by any means, for informational purposes only. For any other purpose, no part of the work may be utilized or reproduced, in any form or by any means, without permission in writing from the publisher.

The work has been developed for automotive applications only. It has neither been developed, nor tested for non-automotive applications.

The word AUTOSAR and the AUTOSAR logo are registered trademarks.

## Table of Contents

Disclaimer	5

Table of Contents	6

1. Introduction and functional overview	9

2. Acronyms and Abbreviations	11

3. Related documentation	14

3.1. Input documents & related standards and norms	14

3.2. Related specification	14

4. Constraints and assumptions	15

4.1. Limitations	15

4.2. Applicability to car domains	15

5. Dependencies to other modules	16

5.1. Upper Protocol Layers	17

5.2. Initialization: Ecu State Manager	17

5.3. Mode Control: CAN State Manager	17

5.4. Lower layers: CAN Driver	17

5.5. Lower layers: CAN Transceiver Driver	18

5.6. Configuration	18

5.7. File structure	20

6. Requirements Tracing	21

7. Functional specification	25

7.1. General Functionality	25

7.2. Hardware object handles	25

7.3. Static L-PDUs	28

7.4. Dynamic L-PDUs	29

7.5. Physical channel view	30

7.6. CAN Hardware Unit	33

7.7. BasicCAN and FullCAN reception	34

7.8. Initialization	35

7.9. Transmit request	36

7.10. Transmit data flow	36

7.11. Transmit buffering	39

7.12. Transmit confirmation	42

7.13. Receive data flow	42

7.14. Receive indication	46

7.15. Read received data	47

7.16. Read Tx/Rx notification status	48

7.17. Data integrity	48

7.18. CAN Controller Mode	49

7.19. PDU channel mode control	54

7.20. Software receive filter	58

7.21. Data Length Check	60

7.22. L-SDU dispatcher to upper layers	60

7.23. Polling mode	61

7.24. Multiple CAN Driver support	61

7.25. Partial Networking	67

7.26. CAN FD Support	67

7.27. Security Events	69

7.28. Error classification	71

8. API specification	74

8.1. Imported types	74

8.2. Type definitions	75

8.3. Function definitions	78

8.4. Callback notifications	106

8.5. Scheduled functions	115

8.6. Expected interfaces	115

9. Sequence diagrams	125

9.1. Transmit request (single CAN Driver)	125

9.2. Transmit request (multiple CAN Drivers)	126

9.3. Transmit confirmation (interrupt mode)	128

9.4. Transmit confirmation (polling mode)	129

9.5. Transmit confirmation (with buffering)	131

9.6. Trigger Transmit Request	133

9.7. Receive indication (interrupt mode)	135

9.8. Receive indication (polling mode)	138

9.9. Read received data	140

9.10. Start CAN network	144

9.11. BusOff notification	146

9.12. BusOff recovery	147

10. Configuration specification	149

10.1. Containers and configuration parameters	149

A - Not applicable requirements	1

### Introduction and functional overview

Tài liệu này mô tả chức năng, API và cấu hình cho module AUTOSAR Basic Software CAN Interface.

Như minh họa trong Figure 1.1, mô-đun CAN Interface nằm giữa các thiết bị CAN drivers (CAN Driver [1] and Transceiver Driver [2]) và các lớp dịch vụ giao tiếp trên như (i.e. CAN State Manager [3], CAN Network Management [4], CAN Transport Protocol [5], PDU Router [6]).. Nó đại diện cho giao diện với các dịch vụ của CAN Driver cho các lớp giao tiếp trên cùng.

Mô-đun CAN Interface cung cấp một giao diện duy nhất để quản lý các loại thiết bị phần cứng CAN khác nhau như CAN Controllers và CAN Transceivers được sử dụng bởi bố trí ECU hardware được xác định. Do đó, nhiều CAN Controller- s/CAN Transceivers nội bộ và bên ngoài có thể được điều khiển bởi mô-đun CAN State Managers dựa trên quan điểm liên quan đến kênh CAN vật lý.

Tóm tắt 1. **CAN Driver và CAN Transceiver Driver**: - CAN Driver giao tiếp trực tiếp với phần cứng bộ điều khiển CAN. - CAN Transceiver Driver giao tiếp với phần cứng bộ chuyển đổi CAN. - Các trình điều khiển này cung cấp các API cơ bản để đọc/ghi thanh ghi, cấu hình phần cứng, gửi/nhận khung dữ liệu. 2. **CAN Interface (CanIf)**: - CanIf sử dụng các API của CAN Driver và CAN Transceiver Driver để giao tiếp với phần cứng CAN. - CanIf cung cấp API cao cấp hơn cho các lớp trên như gửi yêu cầu truyền dữ liệu, nhận dữ liệu, xử lý thông báo lỗi. - Các lớp trên như CAN Transport Protocol, PDU Router trong CAN COM Stack gọi các API của CanIf để gửi/nhận dữ liệu qua mạng CAN. 3. **CAN State Manager (CanSM)**: - CanSM quản lý trạng thái của các kênh CAN bằng cách gọi các API của CanIf để khởi động, dừng, đánh thức bộ điều khiển CAN. - CanSM giao tiếp với CAN Network Management (CanNM) để đồng bộ hóa trạng thái mạng CAN. - CanSM cũng giao tiếp với ECU State Manager (EcuM) để điều phối khởi động và dừng mạng CAN với trạng thái của ECU. 4. **CAN Network Management (CanNM)**: - CanNM sử dụng các dịch vụ truyền/nhận của CanIf để trao đổi thông tin quản lý mạng với các nút khác trên mạng CAN. - CanNM giao tiếp với CanSM để báo cáo trạng thái mạng CAN và yêu cầu thay đổi trạng thái khi cần thiết. - CanNM cũng giao tiếp với Communication Manager (ComM) để quản lý tình trạng kết nối mạng CAN. 5. **Diagnostic Communication Manager (DCM)**: - DCM sử dụng các dịch vụ của CanIf và CanTrcv để gửi và nhận dữ liệu chẩn đoán qua mạng CAN. - DCM giao tiếp với các mô-đun khác trong hệ thống để thực hiện các chức năng chẩn đoán liên quan đến mạng CAN. 6. **Watchdog Manager (WdgM)**: - WdgM giám sát hoạt động của CAN Driver và CanIf thông qua các watchdog timer. - Nếu phát hiện lỗi, WdgM có thể khởi tạo lại hoặc khởi động lại các thành phần CAN bị lỗi. 7. **End-to-End Communication Protection (E2E)**: - E2E sử dụng các dịch vụ của CanIf để gửi và nhận dữ liệu qua mạng CAN. - E2E cung cấp cơ chế bảo vệ dữ liệu như kiểm tra tính toàn vẹn, phát hiện mất dữ liệu, đảm bảo dữ liệu được truyền đầy đủ từ đầu đến đầu. Tóm lại, các thành phần cấp thấp như CAN Driver và Transceiver Driver giao tiếp trực tiếp với phần cứng. CanIf cung cấp giao diện thống nhất cho các lớp trên gửi/nhận dữ liệu. CanSM, CanNM quản lý trạng thái và kết nối mạng CAN, giao tiếp với EcuM, ComM. DCM quản lý giao tiếp chẩn đoán, WdgM giám sát và E2E bảo vệ dữ liệu. Tất cả các thành phần này liên kết chặt chẽ để tạo nên một hệ thống giao tiếp CAN đầy đủ trong AUTOSAR. ECU State Manager (EcuM): Quản lý trạng thái của các ECU (Electronic Control Unit) trong hệ thống. EcuM phối hợp với CanSM để đảm bảo ECU được khởi động và dừng đúng cách, bao gồm cả việc khởi tạo và dừng các mạng giao tiếp như CAN.  Communication Manager (ComM): Quản lý giao thức giao tiếp trên các mạng khác nhau, bao gồm cả CAN. ComM phối hợp với CanNM để quản lý tình trạng kết nối mạng CAN và điều khiển việc tham gia hoặc rời khỏi mạng.  Diagnostic Communication Manager (DCM): Quản lý giao thức giao tiếp chẩn đoán, bao gồm giao thức chẩn đoán trên mạng CAN như UDS (Unified Diagnostic Services). DCM sử dụng các dịch vụ của CanIf và CanTrcv để gửi và nhận dữ liệu chẩn đoán qua mạng CAN.  Watchdog Manager (WdgM): Quản lý các watchdog timer dùng để giám sát hoạt động của hệ thống, bao gồm cả việc giám sát hoạt động của CAN Driver và CanIf.  End-to-End Communication Protection (E2E): Cung cấp các cơ chế bảo vệ dữ liệu truyền từ đầu đến đầu, bao gồm cả dữ liệu truyền qua mạng CAN. E2E sử dụng các dịch vụ của CanIf để gửi và nhận dữ liệu. Được rồi, tôi sẽ giải thích chi tiết hơn về mối liên hệ và cách các thành phần khác nhau trong hệ thống AUTOSAR CAN giao tiếp với nhau: 1. **CAN Driver và CAN Transceiver Driver**: - CAN Driver giao tiếp trực tiếp với phần cứng bộ điều khiển CAN. - CAN Transceiver Driver giao tiếp với phần cứng bộ chuyển đổi CAN. - Các trình điều khiển này cung cấp các API cơ bản để đọc/ghi thanh ghi, cấu hình phần cứng, gửi/nhận khung dữ liệu. 2. **CAN Interface (CanIf)**: - CanIf sử dụng các API của CAN Driver và CAN Transceiver Driver để giao tiếp với phần cứng CAN. - CanIf cung cấp API cao cấp hơn cho các lớp trên như gửi yêu cầu truyền dữ liệu, nhận dữ liệu, xử lý thông báo lỗi. - Các lớp trên như CAN Transport Protocol, PDU Router trong CAN COM Stack gọi các API của CanIf để gửi/nhận dữ liệu qua mạng CAN. 3. **CAN State Manager (CanSM)**: - CanSM quản lý trạng thái của các kênh CAN bằng cách gọi các API của CanIf để khởi động, dừng, đánh thức bộ điều khiển CAN. - CanSM giao tiếp với CAN Network Management (CanNM) để đồng bộ hóa trạng thái mạng CAN. - CanSM cũng giao tiếp với ECU State Manager (EcuM) để điều phối khởi động và dừng mạng CAN với trạng thái của ECU. 4. **CAN Network Management (CanNM)**: - CanNM sử dụng các dịch vụ truyền/nhận của CanIf để trao đổi thông tin quản lý mạng với các nút khác trên mạng CAN. - CanNM giao tiếp với CanSM để báo cáo trạng thái mạng CAN và yêu cầu thay đổi trạng thái khi cần thiết. - CanNM cũng giao tiếp với Communication Manager (ComM) để quản lý tình trạng kết nối mạng CAN. 5. **Diagnostic Communication Manager (DCM)**: - DCM sử dụng các dịch vụ của CanIf và CanTrcv để gửi và nhận dữ liệu chẩn đoán qua mạng CAN. - DCM giao tiếp với các mô-đun khác trong hệ thống để thực hiện các chức năng chẩn đoán liên quan đến mạng CAN. 6. **Watchdog Manager (WdgM)**: - WdgM giám sát hoạt động của CAN Driver và CanIf thông qua các watchdog timer. - Nếu phát hiện lỗi, WdgM có thể khởi tạo lại hoặc khởi động lại các thành phần CAN bị lỗi. 7. **End-to-End Communication Protection (E2E)**: - E2E sử dụng các dịch vụ của CanIf để gửi và nhận dữ liệu qua mạng CAN. - E2E cung cấp cơ chế bảo vệ dữ liệu như kiểm tra tính toàn vẹn, phát hiện mất dữ liệu, đảm bảo dữ liệu được truyền đầy đủ từ đầu đến đầu. Tóm lại, các thành phần cấp thấp như CAN Driver và Transceiver Driver giao tiếp trực tiếp với phần cứng. CanIf cung cấp giao diện thống nhất cho các lớp trên gửi/nhận dữ liệu. CanSM, CanNM quản lý trạng thái và kết nối mạng CAN, giao tiếp với EcuM, ComM. DCM quản lý giao tiếp chẩn đoán, WdgM giám sát và E2E bảo vệ dữ liệu. Tất cả các thành phần này liên kết chặt chẽ để tạo nên một hệ thống giao tiếp CAN đầy đủ trong AUTOSAR. Được rồi, tôi sẽ giải thích rõ hơn về các API được sử dụng và cách các thành phần giao tiếp với nhau: 1. **CAN Driver**: - Cung cấp các API cơ bản để giao tiếp với phần cứng bộ điều khiển CAN như: - API đọc/ghi thanh ghi bộ điều khiển CAN - API cấu hình bộ điều khiển CAN (tốc độ, chế độ hoạt động, ...) - API gửi/nhận khung dữ liệu CAN 2. **CAN Transceiver Driver**: - Cung cấp các API để giao tiếp với phần cứng bộ chuyển đổi CAN như: - API kích hoạt/vô hiệu hóa bộ chuyển đổi - API điều khiển chế độ ngủ/thức của bộ chuyển đổi 3. **CAN Interface (CanIf)**: - Sử dụng các API của CAN Driver và CAN Transceiver Driver để giao tiếp với phần cứng - Cung cấp các API cao cấp hơn cho các lớp trên sử dụng, ví dụ: - API gửi yêu cầu truyền dữ liệu (CanIf_Transmit) - API nhận thông báo dữ liệu đã được gửi thành công (CanIf_TxConfirmation) - API nhận chỉ báo dữ liệu đã được nhận (CanIf_RxIndication) - API nhận thông báo lỗi (CanIf_ControllerBusOffNotification, CanIf_ErrorNotification) - API khởi động/dừng bộ điều khiển CAN (CanIf_SetControllerMode) 4. **CAN COM Stack** (gồm CAN Transport Protocol, PDU Router): - Gọi các API của CanIf để gửi/nhận dữ liệu qua mạng CAN - Ví dụ: Gọi CanIf_Transmit để gửi yêu cầu truyền dữ liệu 5. **CAN State Manager (CanSM)**: - Gọi các API của CanIf để khởi động/dừng bộ điều khiển CAN - Ví dụ: Gọi CanIf_SetControllerMode(START) để khởi động bộ điều khiển - Gọi các API của CAN Network Management (CanNM) để quản lý trạng thái mạng - Ví dụ: Gọi CanNm_NetworkRequest để tham gia mạng CAN 6. **CAN Network Management (CanNM)**: - Sử dụng các API của CanIf để gửi/nhận thông tin quản lý mạng - Ví dụ: Gọi CanIf_Transmit để gửi khung quản lý mạng - Gọi các API của CanSM để thông báo thay đổi trạng thái mạng - Ví dụ: Gọi CanSM_ControllerBusOff để báo bộ điều khiển bị ngắt kết nối 7. **Diagnostic Communication Manager (DCM)**: - Sử dụng các API của CanIf để gửi/nhận dữ liệu chẩn đoán - Ví dụ: Gọi CanIf_Transmit để gửi yêu cầu chẩn đoán 8. **Watchdog Manager (WdgM)**: - Giám sát hoạt động của CAN Driver, CanIf thông qua watchdog timer - Nếu phát hiện lỗi, có thể gọi các API để khởi động lại các thành phần 9. **End-to-End Communication Protection (E2E)**: - Sử dụng các API của CanIf để gửi/nhận dữ liệu được bảo vệ - Thực hiện các cơ chế bảo vệ dữ liệu như kiểm tra tính toàn vẹn, phát hiện mất dữ liệu Như vậy, các thành phần giao tiếp với nhau thông qua việc gọi các API cụ thể. Các lớp thấp hơn cung cấp API để lớp trên sử dụng. Các lớp trên gọi các API này để thực hiện chức năng của mình, ví dụ gửi/nhận dữ liệu, khởi động/dừng bộ điều khiển, quản lý mạng, chẩn đoán, ... Qua đó, các thành phần liên kết chặt chẽ để tạo nên hệ thống giao tiếp CAN hoàn chỉnh.

Figure 1.1: AUTOSAR CAN Layer Model (see [7])

Mô-đun CAN Interface bao gồm tất cả các tác vụ không phụ thuộc vào phần cứng CAN, thuộc về các trình điều khiển thiết bị giao tiếp CAN của ECU tương ứng. Chức năng đó được triển khai một lần trong mô-đun CAN Interface, để các trình điều khiển thiết bị CAN bên dưới chỉ tập trung vào việc truy cập và điều khiển thiết bị phần cứng CAN cụ thể.

CanIf đáp ứng các yêu cầu chính về luồng điều khiển và luồng dữ liệu của PDU Router và các mô-đun giao tiếp lớp trên của AUTOSAR COM stack: transmit request processing, transmit confirmation / receive indication / error notification and start / stop of a CAN Controller and thus waking up / participating on a network. API xử lý dữ liệu và thông báo của nó dựa trên CAN L-SDU, trong khi các API dành cho điều khiển và xử lý chế độ cung cấp một quan điểm liên quan đến Bộ điều khiển CAN.

Đây là bản dịch sang tiếng Việt:

Trong trường hợp có Yêu cầu Truyền, CanIf hoàn tất việc truyền L-PDU với các tham số tương ứng và chuyển tiếp L-PDU CAN qua CanDrv thích hợp đến Bộ điều khiển CAN. Khi nhận, CanIf phân phối các L-PDU đã Nhận dưới dạng L-SDU cho lớp trên. Sự gán giữa L-SDU Nhận và lớp trên được cấu hình tĩnh. Tại xác nhận truyền, CanIf chịu trách nhiệm thông báo cho các lớp trên về việc truyền thành công.

Mô-đun CAN Interface cung cấp quyền truy cập trừu tượng giao tiếp CAN cho các dịch vụ của CAN Driver và CAN Transceiver Driver để điều khiển và giám sát mạng CAN. CAN Interface chuyển tiếp xuống các yêu cầu thay đổi trạng thái từ CAN State Manager đến các trình điều khiển thiết bị CAN ở lớp thấp hơn, và chuyển tiếp lên các sự kiện từ CAN Driver / CAN Transceiver Driver bởi mô-đun CAN Interface đến ví dụ mô-đun NM tương ứng.

### Acronyms and Abbreviations

Danh mục thuật ngữ dưới đây bao gồm các từ viết tắt và viết tắt liên quan đến mô-đun CAN Interface mà không được đưa vào trong [8, Bảng chú giải AUTOSAR].

| Abbreviation | Description: |
| --- | --- |
| CAN L-PDU | CAN Protocol Data Unit. Consists of an identifier, Data Length and data (SDU) Visible to the CAN driver. |
| CAN L-SDU | CAN Service Data Unit. Data that are transported inside the CAN L-PDU. Visible to the upper layers of the CAN interface (e.g. PDU Router). |
| CanDrv | CAN Driver module |
| CAN FD | CAN with Flexible Data-Rate |
| CanId | CAN Identifier |
| CanIf | CAN Interface module |
| CanNm | CAN Network Management module |
| CanSm | CAN State Manager module |
| CanTp | CAN Transport Layer module |
| CanTrcv | CAN Transceiver Driver module |
| CanTSyn | Global Time Synchronization over CAN |
| ComM | Communication Manager module |
| DCM | Diagnostic Communication Manager module |
| EcuM | ECU State Manager module |
| HOH | CAN hardware object handle |
| HRH | CAN hardware receive handle |
| HTH | CAN hardware transmit handle |
| J1939Nm | J1939 Network Management module |
| J1939Tp | J1939 Transport Layer module |
| PduR | PDU Router module |
| PN | Partial Networking |
| SchM | Scheduler Module |
| Buffer | Fixed sized memory area for a single data unit (e.g. CAN ID, Data Length, SDU, etc.) is stored at a dedicated memory address in RAM. |
| CAN communication matrix | Describes the complete CAN network: - Participating nodes - Definition of all CAN PDUs (Identifier, Data Length) Source and Sinks for PDUs |
| CAN Controller | A CAN Controller is a CPU on-chip or external standalone hard- ware device. One CAN Controller is connected to one physical channel. |
| CAN Device Driver | Generic term of CAN Driver and CAN Transceiver Driver. |
| CAN Hardware Unit | A CAN Hardware Unit may consist of one or multiple CAN Con- trollers of the same type and one, two or multiple CAN RAM areas. The CAN Hardware Unit is located on-chip or as exter- nal device. The CAN hardware unit is represented by one CAN Driver. |
| CanIf Controller mode state ma- chine | This is not really a state machine, which may be influenced by transmission requests. This is an image of the current abstracted state of an appropriate CAN Controller. The state transitions can only be realized by upper layer modules like the CanSm or by external events like e.g. if a BusOff occurred. |
| CanIf Receive L-PDU / CanIf Rx L-PDU | L-PDU of which the direction is set to "lower to upper layer". |
| CanIf Receive L-PDU buffer / CanIfRxBuffer | Single element RAM buffer located in the CAN Interface module to store whole receive L-PDUs. |
| CanIf Transmit L-PDU / CanIf Tx L-PDU | L-PDU of which the direction is set to "upper to lower layer". |
| CanIf Transmit L-PDU buffer / CanIfTxBuffer | Single CanIfTxBuffer element located in the CanIf to store one or multiple CanIf Tx L-PDUs. If the buffersize of a single CanI- fTxBuffer element is set to 0, a CanIfTxBuffer element is only used to refer a HTH. |
| Hardware object / HW object | A CAN hardware object is defined as a PDU buffer inside the CAN RAM of the CAN Hardware Unit / CAN Controller. |
| Hardware Receive Handle (HRH) | The Hardware Receive Handle (HRH) is defined and provided by the CAN Driver. Each HRH typically represents just one hard- ware object. The HRH is used as a parameter by the CAN Inter- face Layer for i.e. software filtering. |
| Hardware Transmit Handle (HTH) | The Hardware Transmit Handle (HTH) is defined and provided by the CAN Driver. Each HTH typically represents just one or multi- ple CAN hardware objects that are configured as CAN hardware transmit buffer pool. |
| Inner priority inversion | Transmission of a high-priority L-PDU is prevented by the pres- ence of a pending low-priority L-PDU in the same transmit hard- ware object. |
| Integration Code | Code that the Integrator needs to add to an AUTOSAR System, to adapt non-standardized functionalities. Examples are Call- outs of the ECU State Manager and Callbacks of various other BSW modules. The I/O Hardware Abstraction is called Integra- tion Code, too. |
| Lowest In - First Out / LOFO | This is a data storage procedure, whereas always the elements with the lowest values will be extracted. |
| L-PDU channel group | Group of CAN L-PDUs, which belong to just one underlying net- work. Usually they are handled by one upper layer module. |
| Outer priority inversion | A time gap occurs between two consecutive transmit L-PDUs. In this case a lower priority L-PDU from another node can prevent sending the own higher priority L-PDU. Here the higher priority L- PDU cannot participate in arbitration during network access be- cause the lower priority L-PDU already won the arbitration. |
| Physical channel | A physical channel represents an interface from a CAN Controller to the CAN Network. Different physical channels of the CAN Hardware Unit may access different networks. |
| Tx request | Transmit request to the CAN Interface module from a upper layer module of the CanIf |

### Related documentation

#### Input documents & related standards and norms

- Specification of CAN Driver AUTOSAR_SWS_CANDriver

- Specification of CAN Transceiver Driver AUTOSAR_SWS_CANTransceiverDriver

- Specification of CAN State Manager AUTOSAR_SWS_CANStateManager

- Specification of CAN Network Management AUTOSAR_SWS_CANNetworkManagement

- Specification of CAN Transport Layer AUTOSAR_SWS_CANTransportLayer

- Specification of PDU Router AUTOSAR_SWS_PDURouter

- Layered Software Architecture AUTOSAR_EXP_LayeredSoftwareArchitecture

- Glossary AUTOSAR_TR_Glossary

- General Specification of Basic Software Modules AUTOSAR_SWS_BSWGeneral

- General Requirements on Basic Software Modules AUTOSAR_SRS_BSWGeneral

- Requirements on CAN AUTOSAR_SRS_CAN

- ISO 11898-1:2015 – Road vehicles – Controller area network (CAN)

- Specification of ECU State Manager AUTOSAR_SWS_ECUStateManager

- Specification of ECU Configuration AUTOSAR_TPS_ECUConfiguration

#### Related specification

AUTOSAR provides a General Specification on Basic Software modules [9, SWS BSW General], which is also valid for CAN Interface.

Thus, the specification SWS BSW General shall be considered as additional and re- quired specification for CAN Interface.

### Constraints and assumptions

#### Limitations

CAN Interface chỉ có thể được sử dụng cho giao tiếp CAN và được thiết kế đặc biệt để hoạt động với một hoặc nhiều CAN Driver và CAN Transceiver Driver bên dưới. Nhiều mô-đun CAN Driver khác nhau tương ứng với các Đơn vị Phần cứng CAN khác nhau được đại diện bởi một giao diện chung duy nhất như được quy định trong CAN Driver specification [1]. Tương tự, nhiều mô-đun CAN Transceiver Driver khác nhau tương ứng với các thiết bị CAN Transceiver khác nhau cũng được đại diện bởi một giao diện chung duy nhất như được quy định trong CAN Transceiver Driver specification [2, Specification of CAN Transceiver Driver]. Các giao thức khác ngoài CAN (ví dụ: LIN hoặc FlexRay) không được hỗ trợ.

Lưu ý rằng một bộ lọc PnTxFilter đang hoạt động đảm bảo rằng thông điệp đầu tiên trên đường truyền là CanIfTxPduPnFilterPdu. Trong trường hợp CanIfTxPduPnFilterPdu là PDU NM, bộ xếp tầng COM-Stack sẽ đảm bảo rằng các Nhóm PDU bị vô hiệu hóa cho đến khi PDU đó được gửi đi thành công. Tuy nhiên, các yêu cầu gửi PDU khác (ví dụ: PDU khởi động ban đầu, TP-PDU, XCP-PDU) sẽ bị từ chối cho đến khi PDU được cấu hình đã được gửi đi. Chỉ PDU đầu tiên khởi tạo quá trình Thức Mạng mới là CanIfTxPduPnFilterPdu. Trong trường hợp giao tiếp đang diễn ra và có sự nhận thành công khung tin với PnTxFilter được bật, PnTxFilter sẽ bị vô hiệu hóa. Trong trường hợp này, PnTxFilter không cần thiết vì sẽ nhận được phản hồi từ một Nút đã hoạt động.

#### Applicability to car domains

The CAN Interface can be used for all domain applications when the CAN protocol is used.

### Dependencies to other modules

Phần này mô tả các mối quan hệ với các mô-đun khác trong kiến trúc phần mềm cơ bản AUTOSAR. Nó chứa các mô tả ngắn gọn về thông tin cấu hình và các dịch vụ mà Lớp CAN Interface yêu cầu từ các mô-

đun khác (see Figure 5.1).

Figure 5.1: CANIF dependencies in AUTOSAR BSW

Hình 5.1 này minh họa các mối phụ thuộc và mối quan hệ của mô-đun CAN Interface (CanIf) với các mô-đun khác trong kiến trúc phần mềm cơ bản AUTOSAR (AUTOSAR BSW). 1. CAN Communication Stack (CAN COM Stack): - Phần này bao gồm các mô-đun ComServices, Com, CanSM, CanTp, PduR và PduR_CanTp. - Các mô-đun này sử dụng (realize) các dịch vụ của CanIf để gửi/nhận dữ liệu qua giao thức CAN. - CanSM (CAN State Manager) là mô-đun bắt buộc sử dụng dịch vụ của CanIf để quản lý trạng thái của mạng CAN. 2. CanTxA: - Đây là một lớp trừu tượng cung cấp giao diện chung để các mô-đun khác sử dụng dịch vụ của CanIf. 3. CanIf: - Mô-đun CAN Interface cung cấp giao diện để giao tiếp với các trình điều khiển thiết bị CAN như Can và CanTrcv. - CanIf sử dụng (use) dịch vụ của Can và CanTrcv. 4. Can: - Đây là mô-đun bắt buộc, đại diện cho CAN Driver, giao tiếp trực tiếp với phần cứng Bộ điều khiển CAN. 5. CanTrcv: - Mô-đun này đại diện cho CAN Transceiver Driver, giao tiếp với phần cứng Bộ chuyển đổi CAN. 6. Can Driver: - Đây là lớp thấp nhất, gồm các trình điều khiển phần cứng cấp thấp giao tiếp trực tiếp với phần cứng Bộ điều khiển CAN. Tóm lại, các mô-đun trong CAN COM Stack sử dụng dịch vụ của CanIf để giao tiếp qua mạng CAN. CanIf đóng vai trò trung gian, sử dụng dịch vụ của Can và CanTrcv để giao tiếp với phần cứng Bộ điều khiển CAN và Bộ chuyển đổi CAN. Các mối quan hệ này thể hiện cách các thành phần phần mềm và phần cứng CAN liên kết với nhau trong kiến trúc AUTOSAR.

#### Upper Protocol Layers

Trong kiến trúc phần mềm cơ bản AUTOSAR, các lớp trên của mô-đun CAN Interface (Viết tắt: CanIf) được đại diện bởi mô-đun PDU Router (Viết tắt: PduR), mô-đun CAN Network Management (Viết tắt: CanNm), mô-đun CAN Transport Layer (Viết tắt: CanTp), mô-đun CAN State Manager (Viết tắt: CanSm), mô-đun ECU State Manager (Viết tắt: EcuM), mô-đun Complex Driver (Viết tắt: _CDD_), mô-đun Universal Calibration Protocol (Viết tắt: _XCP_), Global Time Synchronization over CAN (Viết tắt: CanTSyn), mô-đun J1939 Transport Layer (Viết tắt: J1939Tp) và mô-đun J1939 Network Management (Viết tắt: J1939Nm).

Kiến trúc phần mềm cơ bản AUTOSAR cho thấy các bộ đệm dữ liệu của ứng dụng được đặt trong lớp trên mà chúng thuộc về. Việc truy cập trực tiếp vào các bộ đệm này bị cấm. Vị trí của bộ đệm được CanIf chuyển từ hoặc đến mô-đun CAN Driver (Viết tắt: CanDrv) trong quá trình truyền và nhận. Trong quá trình thực thi các dịch vụ chỉ báo truyền/nhận này, vị trí của bộ đệm được chuyển. Tính toàn vẹn dữ liệu được đảm bảo bằng cách sử dụng cơ chế khóa mỗi lần truy cập bộ đệm. Xem phần 7.17 "Tính toàn vẹn dữ liệu".

API được CanIf sử dụng bao gồm các dịch vụ thông báo như các tác nhân cơ bản để chuyển dữ liệu liên quan đến CAN (ví dụ: Độ dài Dữ liệu) đến lớp trên đích. Các tham số gọi của các dịch vụ này trỏ đến thông tin được đệm trong CanDrv hoặc chúng tham chiếu trực tiếp đến Phần cứng CAN.

Ngoài ra, CanIf hỗ trợ một callout đến mô-đun Bus Mirroring, để báo cáo nội dung của các khung dữ liệu đã nhận và đã gửi.

#### Initialization: Ecu State Manager

The EcuM initializes the CanIf (refer to [3, Specification of ECU State Manager]).

#### Mode Control: CAN State Manager

Module CanSm chịu trách nhiệm quản lý điều khiển chế độ của tất cả các bộ điều khiển CAN và bộ truyền CAN được hỗ trợ.

#### Lower layers: CAN Driver

Trình điều khiển thiết bị lớp dưới chính của CAN được đại diện bởi CanDrv (see [1, Specifi- cation of CAN Driver]). CanIf có mối quan hệ chặt chẽ với CanDrv do vị trí của nó trong AUTOSAR Basic Software Architecture.

CanDrv cung cấp truy cập trừu tượng phần cứng chỉ đến Bộ điều khiển CAN, nhưng việc kiểm soát các chế độ hoạt động chỉ được thực hiện trong CanSm. CanDrv phát hiện và xử lý các sự kiện của các Bộ điều khiển CAN và thông báo những sự kiện này đến CanIf. CanIf chuyển các yêu cầu chế độ hoạt động của CanSm đến các Bộ điều khiển CAN tương ứng bên dưới.

CanDrv cung cấp một L-PDU chuẩn hóa để đảm bảo CanIf không phụ thuộc vào phần cứng. Con trỏ đến L-PDU chuẩn hóa này trỏ hoặc đến bộ đệm tạm thời (ví dụ: chuẩn hóa dữ liệu) hoặc đến CanDrv phụ thuộc vào phần cứng CAN. Đối với CanIf, loại bộ đệm L-PDU là vô hình.

CanIf cung cấp các dịch vụ thông báo được sử dụng bởi CanDrv trong tất cả các kịch bản thông báo, ví dụ: xác nhận truyền (mục 8.4.2 “CanIf_TxConfirmation”, xem [SWS_CANIF_00007]), chỉ báo nhận (mục 8.4.3 “CanIf_RxIndication”, xem [SWS_CANIF_00006]) và thông báo thay đổi chế độ bộ điều khiển (mục 8.4.8, xem [SWS_CANIF_00699]).

Trong trường hợp sử dụng nhiều CanDrv phục vụ các vectơ ngắt khác nhau, các dịch vụ gọi lại nêu trên phải có khả năng tái nhập, tham khảo mục 7.24 “Hỗ trợ Trình điều khiển CAN Nhiều”. Khả năng tái nhập của các hàm gọi lại được quy định trong mục 8.4.

Các dịch vụ gọi lại do CanDrv gọi được khai báo và triển khai bên trong CanIf. Các dịch vụ gọi lại do CanIf gọi được khai báo và đặt bên trong lớp dịch vụ truyền thông trên, ví dụ như PduR, CanNm, CanTp. Cấu trúc CanIf được quy định trong mục 5.7 “Cấu trúc tệp”.

Số lượng các Bộ điều khiển CAN được cấu hình không nhất thiết tương ứng với số lượng Bộ truyền CAN được sử dụng. Trong trường hợp nhiều Bộ điều khiển CAN của các loại khác nhau hoạt động trên cùng một mạng CAN, một Bộ truyền CAN và CanTrcv là đủ, trong khi đó phụ thuộc vào loại thiết bị Bộ điều khiển CAN, cần một hoặc hai CanDrv khác nhau (see section 7.5 “Physical channel view”).

#### Lower layers: CAN Transceiver Driver

The second available lower layer CAN device driver is represented by the CanTrcv (see [2, Specification of CAN Transceiver Driver]).

Each CanTrcv itself does operation mode control of the CAN Transceiver device. The CanIf just maps all APIs of several underlying CanTrcvs to a unique one, thus CanSm is able to trigger a transition of the corresponding CAN Transceiver modes. No control or handling functionality belonging to CanTrcv is done inside the CanIf.

The CanIf maps the following services of all underlying CanTrcvs to one unique inter- face. These are further described in the CAN Transceiver Driver SWS (see [2, Specifi- cation of CAN Transceiver Driver]):

- Unique CanTrcv mode request and read services to manage the operation modes of each underlying CAN Transceiver device.

- Read service for CAN Transceiver wake up reason support.

- Mode request service to enable/disable/clear wake up event state of each used CAN transceiver (CanIf_SetTrcvMode(), see [SWS_CANIF_00287]).

#### Configuration

Thiết kế CanIf được tối ưu hóa để quản lý các khả năng đặc thù của giao thức CAN và xử lý Bộ điều khiển CAN bên dưới được sử dụng. CanIf có khả năng thay đổi cấu hình CAN mà không cần phải xây dựng lại. Do đó, hàm CanIf_Init() (xem [SWS_CANIF_00001]) lấy thông tin cấu hình CAN cần thiết từ các bộ chứa và tham số cấu hình, được chỉ định (liên kết làm tham chiếu hoặc các tham số bổ sung) trong chương 10, xem Hình 10.1.

Phần này cung cấp tóm tắt về thông tin đã lấy, ví dụ:

- Số lượng Bộ điều khiển CAN. Số lượng Bộ điều khiển CAN là cần thiết để phân phát các L-PDU truyền và nhận và để kiểm soát trạng thái của các Trình điều khiển CAN hiện có (xem CanIfCtrlDrvCfg).

- Số lượng Tay cầm Đối tượng Phần cứng (HTH). Để giám sát các yêu cầu truyền, Giao diện CAN cần biết số lượng HTH và sự gán giữa mỗi HTH và Bộ điều khiển CAN tương ứng (xem CanIfHthCanCtrlIdRef; CanIfHthIdSymRef).

- Phạm vi các ID CAN nhận được đi qua bộ lọc chấp nhận phần cứng cho mỗi đối tượng phần cứng. Giao diện CAN sử dụng các sự gán cố định giữa HRH và L-PDU được nhận trong đối tượng phần cứng tương ứng để thực hiện thuật toán tìm kiếm (xem mục 7.20 “Bộ lọc nhận phần mềm”, xem CanIfHrhSoftwareFilter, CanIfHrhCanCtrlIdRef, CanIfHrhIdSymRef).

CanIf cần thông tin về tất cả các lớp dịch vụ truyền thông phía trên được sử dụng và L-SDU để phân phát. Thông tin sau phải được thiết lập tại thời điểm cấu hình để tích hợp CanIf vào ngăn xếp COM của AUTOSAR:

- Mô-đun lớp trên truyền và I-PDU truyền cho mỗi L-SDU truyền.

=> Được sử dụng để phân phát các dịch vụ xác nhận truyền (xem CanIfTxPduId).

- Mô-đun lớp trên nhận và I-PDU nhận cho mỗi L-SDU nhận.

=> Được sử dụng để phân phát L-SDU trong quá trình chỉ báo nhận (xem CanIfRxPduId).

CanIf cần mô tả về bộ điều khiển và ECU của chính nó, cái được kết nối với một hoặc nhiều mạng CAN. Thông tin sau được lấy từ ma trận truyền thông CAN, một phần của cấu hình hệ thống AUTOSAR (xem CanIfTxPduCfg, CanIfRxPduCfg):

- Tất cả L-PDU nhận được trên mỗi kênh vật lý của ECU này.

=> Được sử dụng để lọc phần mềm và phân phát L-SDU nhận.

- Tất cả các L-SDU sẽ được truyền bởi mỗi kênh vật lý trên ECU này.

=> Được sử dụng cho yêu cầu truyền và phân phát L-PDU truyền.

- Thuộc tính của các L-PDU này (ID, Độ dài Dữ liệu).

=> Được sử dụng cho lọc phần mềm, dịch vụ chỉ báo nhận, kiểm tra Độ dài Dữ liệu.

- Bộ truyền cho mỗi L-SDU truyền (tức là PduR, CanNm, CanTp).

=> Được sử dụng cho các dịch vụ xác nhận truyền.

- Bộ nhận cho mỗi L-SDU nhận (tức là PduR, CanNm, CanTp).

=> Được sử dụng cho phân phát L-PDU.

- Tên tượng trưng của L-PDU/L-SDU.

=> Được sử dụng để biểu diễn địa chỉ bộ đệm dữ liệu Rx/Tx.

#### File structure

##### Code file structure

[SWS_CANIF_00378] [CanIf shall access the location of the API of all used underly- ing CanDrvs for link time configuration by a set of function pointers for each CanDrv.] ()

The values for the function pointers for each CanDrv are given at link time.

##### Header file structure

[SWS_CANIF_00672] [Tệp tiêu đề CanIf.h chỉ chứa các khai báo extern của các hằng số, dữ liệu toàn cục và các dịch vụ được chỉ định trong CanIf.] ()

Các hằng số, kiểu dữ liệu toàn cục và các hàm chỉ được sử dụng nội bộ bởi CanIf, được khai báo trong CanIf.c.

[SWS_CANIF_00903] [CanIf phải bao gồm tệp tiêu đề Mirror.h nếu hỗ trợ Phản chiếu Bus được kích hoạt (xem CanIfBusMirroringSupport).](SRS_Can_01172)

### Requirements Tracing

Các bảng sau đây tham chiếu các yêu cầu được chỉ định trong [10] cũng như [11] và liên kết đến việc thực hiện những yêu cầu này. Xin lưu ý rằng nếu cột 'Được đáp ứng bởi' trống đối với một yêu cầu cụ thể, điều này có nghĩa là yêu cầu đó không được tài liệu này đáp ứng.

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [RS_Ids_00810] | Basic SW security events | [SWS_CANIF_00913] [SWS_CANIF_00915] [SWS_CANIF_00916] [SWS_CANIF_00917] [SWS_CANIF_00918] [SWS_CANIF_00919] [SWS_CANIF_00920] [SWS_CANIF_00921] [SWS_CANIF_91008] [SWS_CANIF_91009] [SWS_CANIF_91010] |
| [SRS_BSW_00007] | All Basic SW Modules written in C language shall conform to the MISRA C 2012 Standard. | [SWS_CANIF_00999] |
| [SRS_BSW_00010] | The memory consumption of all Basic SW Modules shall be documented for a defined configuration for all supported platforms. | [SWS_CANIF_00999] |
| [SRS_BSW_00101] | The Basic Software Module shall be able to initialize variables and hardware in a separate initialization function | [SWS_CANIF_00001] |
| [SRS_BSW_00159] | All modules of the AUTOSAR Basic Software shall support a tool based configuration | [SWS_CANIF_00999] |
| [SRS_BSW_00164] | The Implementation of interrupt service routines shall be done by the Operating System, complex drivers or modules | [SWS_CANIF_00999] |
| [SRS_BSW_00167] | All AUTOSAR Basic Software Modules shall provide configuration rules and constraints to enable plausibility checks | [SWS_CANIF_00999] |
| [SRS_BSW_00168] | SW components shall be tested by a function defined in a common API in the Basis-SW | [SWS_CANIF_00999] |
| [SRS_BSW_00170] | The AUTOSAR SW Components shall provide information about their dependency from faults, signal qualities, driver demands | [SWS_CANIF_00999] |
| [SRS_BSW_00172] | The scheduling strategy that is built inside the Basic Software Modules shall be compatible with the strategy used in the system | [SWS_CANIF_00999] |
| [SRS_BSW_00306] | AUTOSAR Basic Software Modules shall be compiler and platform independent | [SWS_CANIF_00999] |
| [SRS_BSW_00307] | Global variables naming convention | [SWS_CANIF_00999] |
| [SRS_BSW_00308] | AUTOSAR Basic Software Modules shall not define global data in their header files, but in the C file | [SWS_CANIF_00999] |
| [SRS_BSW_00309] | All AUTOSAR Basic Software Modules shall indicate all global data with read-only purposes by explicitly assigning the const keyword | [SWS_CANIF_00999] |
| [SRS_BSW_00312] | Shared code shall be reentrant | [SWS_CANIF_00064] |
| [SRS_BSW_00348] | All AUTOSAR standard types and constants shall be placed and organized in a standard type header file | [SWS_CANIF_00142] |
| [SRS_BSW_00353] | All integer type definitions of target and compiler specific scope shall be placed and organized in a single type header | [SWS_CANIF_00142] |
| [SRS_BSW_00358] | The return type of init() functions implemented by AUTOSAR Basic Software Modules shall be void | [SWS_CANIF_00001] |
| [SRS_BSW_00361] | All mappings of not standardized keywords of compiler specific scope shall be placed and organized in a compiler specific type and keyword header | [SWS_CANIF_00142] |
| [SRS_BSW_00373] | The main processing function of each AUTOSAR Basic Software Module shall be named according the defined convention | [SWS_CANIF_00999] |
| [SRS_BSW_00378] | AUTOSAR shall provide a boolean type | [SWS_CANIF_00999] |
| [SRS_BSW_00405] | BSW Modules shall support multiple configuration sets | [SWS_CANIF_00001] |
| [SRS_BSW_00407] | Each BSW module shall provide a function to read out the version information of a dedicated module implementation | [SWS_CANIF_00158] |
| [SRS_BSW_00411] | All AUTOSAR Basic Software Modules shall apply a naming rule for enabling/disabling the existence of the API | [SWS_CANIF_00158] |
| [SRS_BSW_00414] | Init functions shall have a pointer to a configuration structure as single parameter | [SWS_CANIF_00001] |
| [SRS_BSW_00416] | The sequence of modules to be initialized shall be configurable | [SWS_CANIF_00999] |
| [SRS_BSW_00417] | Software which is not part of the SW-C shall report error events only after the DEM is fully operational. | [SWS_CANIF_00999] |
| [SRS_BSW_00423] | BSW modules with AUTOSAR interfaces shall be describable with the means of the SW-C Template | [SWS_CANIF_00999] |
| [SRS_BSW_00424] | BSW module main processing functions shall not be allowed to enter a wait state | [SWS_CANIF_00999] |
| [SRS_BSW_00425] | The BSW module description template shall provide means to model the defined trigger conditions of schedulable objects | [SWS_CANIF_00999] |
| [SRS_BSW_00426] | BSW Modules shall ensure data consistency of data which is shared between BSW modules | [SWS_CANIF_00999] |
| [SRS_BSW_00427] | ISR functions shall be defined and documented in the BSW module description template | [SWS_CANIF_00999] |
| [SRS_BSW_00428] | A BSW module shall state if its main processing function(s) has to be executed in a specific order or sequence | [SWS_CANIF_00999] |
| [SRS_BSW_00429] | Access to OS is restricted | [SWS_CANIF_00999] |
| [SRS_BSW_00432] | Modules should have separate main processing functions for read/receive and write/transmit data path | [SWS_CANIF_00999] |
| [SRS_BSW_00433] | Main processing functions are only allowed to be called from task bodies provided by the BSW Scheduler | [SWS_CANIF_00999] |
| [SRS_Can_01001] | The CAN Interface implementation and interface shall be independent from underlying CAN Controller and CAN Transceiver | [SWS_CANIF_00023] |
| [SRS_Can_01003] | The appropriate higher communication stack shall be notified by the CAN Interface about an occurred reception | [SWS_CANIF_00012] |
| [SRS_Can_01005] | The CAN Interface shall perform a check for correct DLC of received PDUs | [SWS_CANIF_00026] |
| [SRS_Can_01008] | The CAN Interface shall provide a transmission request service | [SWS_CANIF_00005] |
| [SRS_Can_01009] | The CAN Interface shall provide a transmission confirmation dispatcher | [SWS_CANIF_00007] |
| [SRS_Can_01011] | The CAN Interface shall provide a transmit buffer | [SWS_CANIF_00068] |
| [SRS_Can_01014] | The CAN State Manager shall offer a network configuration independent interface for upper layers | [SWS_CANIF_00999] |
| [SRS_Can_01018] | The CAN Interface shall allow the configuration of its software reception filter Pre-Compile-Time as well as Link-Time and Post-Build-Time | [SWS_CANIF_00030] |
| [SRS_Can_01020] | The TX-Buffer shall be statically configurable | [SWS_CANIF_00063] |
| [SRS_Can_01021] | CAN The CAN Interface shall implement an interface for initialization | [SWS_CANIF_00001] |
| [SRS_Can_01022] | The CAN Interface shall support the selection of configuration sets | [SWS_CANIF_00001] |
| [SRS_Can_01027] | The CAN Interface shall provide a service to change the CAN Controller mode. | [SWS_CANIF_00003] |
| [SRS_Can_01028] | The CAN Interface shall provide a service to query the CAN controller state | [SWS_CANIF_00229] |
| [SRS_Can_01029] | The CAN Interface shall report bus-off state of a device to an upper layer | [SWS_CANIF_00014] |
| [SRS_Can_01114] | Data Consistency of L-PDUs to transmit shall be guaranteed | [SWS_CANIF_00033] |
| [SRS_Can_01125] | The CAN stack shall ensure not to lose messages in receive direction | [SWS_CANIF_00194] |
| [SRS_Can_01126] | The CAN stack shall be able to produce 100% bus load | [SWS_CANIF_00381] [SWS_CANIF_00382] [SWS_CANIF_00881] |
| [SRS_Can_01129] | The CAN Interface module shall provide a procedural interface to read out data of single CAN messages by upper layers (Polling mechanism) | [SWS_CANIF_00194] |
| [SRS_Can_01130] | Receive Status Interface of CAN Interface | [SWS_CANIF_00202] [SWS_CANIF_00230] |
| [SRS_Can_01131] | The CAN Interface module shall provide the possibility to have polling and callback notification mechanism in parallel | [SWS_CANIF_00230] |
| [SRS_Can_01136] | The CAN Interface module shall provide a service to check for validation of a CAN wake-up event | [SWS_CANIF_00179] |
| [SRS_Can_01139] | The CAN Interface and Driver shall offer a CAN Controller specific interface for initialization | [SWS_CANIF_00999] |
| [SRS_Can_01140] | The CAN Interface shall support both Standard (11bit) and Extended (29bit) Identifiers | [SWS_CANIF_00281] [SWS_CANIF_00877] |
| [SRS_Can_01141] | The CAN Interface shall support both Standard (11bit) and Extended (29bit) Identifiers at same time on one network | [SWS_CANIF_00243] [SWS_CANIF_00877] |
| [SRS_Can_01151] | The CAN Interface shall provide a service to check for a CAN Wake-up event. | [SWS_CANIF_00286] |
| [SRS_Can_01162] | The CAN Interface shall support classic CAN and CAN FD frames | [SWS_CANIF_00877] |
| [SRS_Can_01168] | The CAN Interface shall implement an interface for de-initialization | [SWS_CANIF_91002] |
| [SRS_Can_01169] | The CAN interface shall provide a function to return the current CAN controller error state | [SWS_CANIF_91001] |
| [SRS_Can_01172] | The CAN Interface shall provide a function to provide received and transmitted frames to the Bus Mirroring | [SWS_CANIF_00903] [SWS_CANIF_00904] [SWS_CANIF_00905] [SWS_CANIF_00906] [SWS_CANIF_00911] |
| [SRS_Can_01181] | The CAN Driver shall support hardware-based timestamping | [SWS_CANIF_91011] [SWS_CANIF_91012] [SWS_CANIF_91013] [SWS_CANIF_91014] |

### Functional specification

#### General Functionality

Các dịch vụ của CanIf có thể được chia thành các nhóm chính sau:

- Khởi tạo

- Dịch vụ yêu cầu truyền

- Dịch vụ xác nhận truyền

- Dịch vụ chỉ báo nhận

- Dịch vụ kiểm soát chế độ bộ điều khiển

- Dịch vụ kiểm soát chế độ PDU

Các ứng dụng có thể của CanIf:

i. Chế độ Ngắt

CanDrv xử lý các ngắt do Bộ điều khiển CAN kích hoạt. CanIf, dựa trên sự kiện, được thông báo khi có sự kiện xảy ra. Trong trường hợp này, các dịch vụ liên quan của CanIf được gọi trong các ISR tương ứng trong CanDrv.

ii. Chế độ Thăm dò

CanDrv được kích hoạt bởi SchM và thực hiện các quá trình tiếp theo (Chế độ Thăm dò). Trong trường hợp này, Can_MainFunction_<Write/Read/BusOff/Wakeup/Transceiver>() phải được gọi định kỳ trong khoảng thời gian xác định. CanIf được thông báo bởi CanDrv về các sự kiện (Nhận, Truyền, BusOff, Hết thời gian) đã xảy ra trong một trong các Bộ điều khiển CAN, tương tự như hoạt động theo ngắt. CanDrv chịu trách nhiệm cập nhật thông tin tương ứng liên quan đến sự kiện đã xảy ra trong Bộ điều khiển CAN, ví dụ như nhận một L-PDU.

iii. Chế độ Hỗn hợp: CanDrv dựa trên ngắt và thăm dò

Chức năng có thể được chia giữa chế độ hoạt động dựa trên ngắt và chế độ hoạt động dựa trên thăm dò tùy thuộc vào các Bộ điều khiển CAN được sử dụng. Ví dụ: Nhận FullCAN dựa trên thăm dò và nhận BasicCAN dựa trên ngắt, truyền dựa trên thăm dò và nhận dựa trên ngắt, v.v.

Đặc tả này mô tả một giao diện duy nhất, có giá trị cho cả ba loại chế độ hoạt động. Tóm lại, CanIf hoạt động theo cùng một cách, cho dù bất kỳ sự kiện nào được xử lý theo ngắt, mức tác vụ hoặc hỗn hợp. Sự khác biệt duy nhất là ngữ cảnh cuộc gọi và có thể là cách ngắt thông báo: tiền xử lý hoặc hợp tác. Tất cả các dịch vụ được thực hiện theo cấu hình.

Các đoạn sau đây mô tả chức năng của CanIf.

#### Hardware object handles

Tay cầm Đối tượng Phần cứng (Hardware Object Handles - HOH) cho truyền (HTH) cũng như cho nhận (HRH) đại diện cho một tham chiếu trừu tượng đến cấu trúc hộp thư CAN, chứa các tham số liên quan đến CAN như CanId, DLC và dữ liệu. Dựa trên sự trừu tượng hóa bộ đệm phần cứng CAN, mỗi Đối tượng Phần cứng được tham chiếu trong CanIf độc lập với bố trí bộ đệm phần cứng CAN. HOH được sử dụng làm tham số trong các cuộc gọi dịch vụ giao diện của CanDrv và được cung cấp bởi cấu hình của CanDrv và được CanDrv sử dụng làm định danh cho các bộ đệm giao tiếp của hộp thư CAN.

CanIf chỉ đóng vai trò là người dùng của Tay cầm Đối tượng Phần cứng, nhưng không diễn giải nó dựa trên thông tin cụ thể của phần cứng. Do đó, CanIf vẫn độc lập với phần cứng.

[SWS_CANIF_00023] [CanIf phải tránh truy cập trực tiếp vào các bộ đệm giao tiếp cụ thể của phần cứng và phải truy cập chúng thông qua các dịch vụ giao diện của CanDrv.](SRS_Can_01001)

Lý do cho [SWS_CANIF_00023]: CanIf vẫn độc lập với phần cứng vì các giao diện của CanDrv được gọi bằng các tham số HOH, giúp trừu tượng hóa các thuộc tính bộ đệm phần cứng CAN cụ thể.

Mỗi Bộ điều khiển CAN có thể cung cấp nhiều Đối tượng Phần cứng Truyền CAN trong hộp thư CAN. Chúng có thể được liên kết logic với một toàn bộ nhóm Đối tượng Phần cứng (Đối tượng Phần cứng đa kênh) và do đó được định địa chỉ bởi một HTH.

[SWS_CANIF_00662] [CanIf phải sử dụng hai loại HOH để cho phép truy cập vào CanDrv:

- Tay cầm Truyền Phần cứng (HTH) và

- Tay cầm Nhận Phần cứng (HRH).]

[SWS_CANIF_00291] [Định nghĩa của HRH: HRH phải là một tay cầm tham chiếu một Đối tượng Nhận Phần cứng logic của hộp thư Bộ điều khiển CAN.]()

[SWS_CANIF_00665] [HRH phải cho phép CanIf sử dụng phương pháp nhận BasicCAN hoặc FullCAN của đơn vị nhận tham chiếu và chỉ báo một L-SDU Đã Nhận đến mô-đun lớp trên đích.]()

[SWS_CANIF_00663] [Nếu HRH tham chiếu một đơn vị nhận được cấu hình cho nhận BasicCAN, lọc phần mềm phải được kích hoạt trong CanIf.]()

[SWS_CANIF_00664] [Nếu sử dụng nhiều HRH, mỗi HRH phải thuộc ít nhất một nhóm đơn hoặc cố định của Rx L-SDU (CanRxPduIds).]()

HRH có thể được cấu hình để nhận:

- một CanId đơn (FullCAN)

- một nhóm các CanId đơn (BasicCAN)

- một phạm vi/khu vực của CanId (BasicCAN) hoặc

- tất cả các CanId.

Figure 7.1: Mapping between PDU Ids and HW object handles

[SWS_CANIF_00292] [Định nghĩa của HTH: HTH phải là một tay cầm tham chiếu một Đối tượng Phần cứng Truyền logic của hộp thư Bộ điều khiển CAN.]()

[SWS_CANIF_00666] [HTH phải cho phép CanIf sử dụng phương pháp truyền BasicCAN hoặc FullCAN của đơn vị truyền tham chiếu và xác nhận một L-SDU đã truyền đến một mô-đun lớp trên đích.]()

[SWS_CANIF_00466] [Mỗi CanIf Tx L-PDU phải được gán tĩnh cho một bộ chứa cấu hình CanIfBufferCfg vào thời điểm cấu hình (xem CanIfTxPduBufferRef).]()

Lý do cho [SWS_CANIF_00466]: Các CanIf Tx L-PDU không tham chiếu HTHs, nhưng CanIfBufferCfg, mà ngược lại tham chiếu HTHs.

[SWS_CANIF_00667] [Nếu sử dụng nhiều HTHs, mỗi HTH phải thuộc ít nhất một nhóm đơn hoặc cố định của Tx L-PDU (CanTxPduIds).]()

[SWS_CANIF_00115] [CanIf phải có khả năng sử dụng tất cả HRHs và HTHs của một CanDrv như một khu vực đánh số chung, duy nhất bắt đầu từ không.]()

Các HRHs và HTHs dành riêng được suy ra từ bộ cấu hình của CanDrv. Định nghĩa của HTH/HRH trong khu vực đánh số và Đối tượng Phần cứng là trách nhiệm của CanDrv.

#### Static L-PDUs

CanIf cung cấp truy cập chung đến dữ liệu liên quan đến L-SDU CAN cho các lớp trên. Các thuộc tính của bảng sau được biểu diễn dưới dạng các tham số cấu hình và được chỉ định trong chương 10:

| CAN Interface specific attributes | CAN Protocol Control Information (PCI) |
| --- | --- |
| Method of SW filtering CanIfPrivateSoftwareFilterType | CAN Identifier (CanId) CanIfTxPduCanId, range of CanIds per PDU (see CanIfRxPduCanIdRange), CanIfRxPduCanId, CanIfRxPduCanIdMask |
| Direction of L-PDU (Tx, Rx) CanIfTxPduId, CanIfRxPduId) | Type of CAN Identifier (StandardCAN, ExtendedCAN) referenced from CanDrv via CanIfHthIdSymRef, CanIfHrhIdSymRef |
| HTH/HRH of the CAN Controller | Data Length and Data Length Code (DLC) CanIfRxPduDataLength |
| Target ID for the corresponding upper layer CanIfTxPduUserTxConfirmationUL, CanIfRxPduUserRxIndicationUL | Reference to the PDU data (see [1, Specification of CAN Driver]) |
| Type of Transmit L-PDU (STATIC, DYNAMIC) CanIfTxPduType |  |
| Type of Tx/Rx L-PDU (FullCAN, BasicCAN) CanIfHthIdSymRef, CanIfHrhIdSymRef |  |

CanIf hỗ trợ kích hoạt và vô hiệu hóa tất cả các L-PDU thuộc về một Bộ điều khiển CAN cho truyền cũng như cho nhận (xem 7.19.2, xem CanIf_SetPduMode(), [SWS_CANIF_00008]). Đối với việc kiểm soát chế độ L-PDU, xin tham khảo mục 7.19.

Mỗi L-PDU được liên kết với một mô-đun lớp trên để đảm bảo việc phân phối chính xác trong quá trình nhận, xác nhận truyền và truy cập dữ liệu. Mỗi mô-đun lớp trên có thể sử dụng các L-PDU để phục vụ các Bộ điều khiển CAN khác nhau đồng thời.

Theo kiến trúc PDU được định nghĩa cho toàn bộ ngăn xếp giao tiếp AUTOSAR (xem [7, Kiến trúc Phần mềm Lớp]), việc sử dụng L-PDU được chia thành hai cách khác nhau:

- Đối với yêu cầu truyền và API thăm dò truyền/nhận, mô-đun lớp trên sử dụng ID L-SDU (CanTxPduId/CanRxPduId) được định nghĩa bởi CanIf như là tham số.

- Đối với tất cả các API gọi lại, mà được gọi bởi CanIf tại các mô-đun lớp trên, CanIf truyền PduId mục tiêu được định nghĩa bởi mỗi mô-đun lớp trên làm tham số.

Nguyên tắc là người gọi phải sử dụng Id L-PDU/L-SDU mục tiêu được định nghĩa của người được gọi.

Nếu quá trình khởi động không được thực hiện và lớp trên thực hiện yêu cầu truyền tới CanIf, không có L-SDUs được truyền xuống lớp dưới và DET sẽ được kích hoạt. Do đó, không có dữ liệu chưa được khởi tạo nào có thể được truyền trên mạng. Hành vi của hàm truyền L-PDU/L-SDU được chỉ định chi tiết trong mục 8.3.6.

#### Dynamic L-PDUs

CanIf phải hỗ trợ khả năng lọc các tin nhắn đến bằng cách sử dụng CanIfRxPduCanIdMask. Việc lọc sẽ được thực hiện bằng cách so sánh CanId đến với CanIfRxPduCanId được lưu trữ sau khi áp dụng CanIfRxPduCanIdMask cho cả hai ID. Điều này nên được thực hiện sau khi lọc các CanId thông thường mà không có mặt nạ, để cho phép xử lý riêng biệt của một số CanId nằm trong phạm vi được xác định bởi mặt nạ hoặc một phạm vi dựa trên CanId.

Ngoài ra, DYNAMIC Tx và Rx L-SDUs phải được hỗ trợ, trong đó CanId nằm trong MetaData của L-SDU.

Trong quá trình truyền L-SDUs động, khi được xác định một CanIfTxPduCanIdMask, các phần biến của CanId được cung cấp qua MetaData phải được hợp nhất với CanId bằng cách sử dụng mặt nạ này. Khi không có CanIfTxPduCanIdMask và không có CanIfTxPduCanId được cấu hình, MetaData phải được sử dụng trực tiếp làm CanId.

Trong quá trình nhận L-SDUs động, CanId nhận được phải được đặt trong MetaData của L-SDU. Nội dung của MetaData là độc lập với tham số CanIfRxPduCanIdMask.

[SWS_CANIF_00844] [CanIf phải hỗ trợ các L-PDU động, trong đó CanId hoặc các phần liên quan của CanId được đặt trong MetaData của một L-SDU.]()

##### Dynamic Transmit L-PDUs

Định nghĩa của các Transmit L-PDU động: Các L-PDU cho phép việc cấu hình lại CanId trong quá trình chạy (CanIfTxPduType) hoặc nơi mà ID hoặc các phần của nó được cung cấp dưới dạng MetaData của L-SDU.

Việc sử dụng tất cả các phần tử L-PDU khác đều giống như các Transmit L-PDU tĩnh bình thường:

- Thông báo xác nhận truyền CanIfTxPduUserTxConfirmationUL không thể được cấu hình lại vì nó thuộc về L-PDU.

- Chiều dài dữ liệu và con trỏ đến bộ đệm dữ liệu đều được xác định bởi mô-đun lớp trên khi gọi hàm CanIf_Transmit().

Hàm CanIf_SetDynamicTxId() (xem [SWS_CANIF_00189]) cấu hình lại CanId của một L-PDU động với CanIfTxPduType.

[SWS_CANIF_00188] [CanIf phải xử lý hai bit cao nhất của CanId (xem [1, Đặc tả của Driver CAN], định nghĩa của Can_IdType [SWS_Can_00416]) để xác định loại CanId được sử dụng và do đó cách thức truyền L-PDU động sẽ được thực hiện.]()

[SWS_CANIF_00673] [CanIf phải đảm bảo tính nhất quán dữ liệu của CanId trong trường hợp chạy hàm CanIf_SetDynamicTxId(). Dịch vụ này có thể bị gián đoạn bởi cuộc gọi tiền định của CanIf_Transmit() ảnh hưởng đến cùng một L-PDU, xem [SWS_CANIF_00064].]()

[SWS_CANIF_00855] [Nếu CanIfTxPduCanIdMask và CanIfTxPduCanId được bỏ qua, CanId sẽ được lấy trực tiếp từ MetaData.]()

[SWS_CANIF_00856] [CanIfTxPduCanIdMask sẽ được bỏ qua khi cấu hình meta dữ liệu không chứa CAN_ID_32 cho L-SDU này.]()

[SWS_CANIF_00854] [Nếu Mục MetaData CAN_ID_32, CanIfTxPduCanIdMask và CanIfTxPduCanId có sẵn, CanIfTxPduCanIdMask xác định các bit trong CanIfTxPduCanId và các bit của Can_IdType được tạo ra từ CanIfTxPduCanIdType mà sẽ xuất hiện trong CanId thực tế, các bit khác được lấy từ MetaData.]()

Lưu ý: CanId kết quả có thể được tính như sau: (CanIfTxPduCanId & CanIfTxPduCanIdMask) | (<các phần ID động> & ∼CanIfTxPduCanIdMask)

[SWS_CANIF_00857] [CanIf_Init() (xem [SWS_CANIF_00085]) khởi tạo các CanId của các Transmit L-PDU động với CanIfTxPduType đến giá trị được cấu hình qua CanIfTxPduCanId.]()

##### Dynamic receive L-PDUs

Định nghĩa của các Receive L-PDU động: Các L-PDU tương ứng với một tập hợp các CanId, trong đó CanId thực tế được nhận được cung cấp cho các lớp trên như một phần của dữ liệu PDU.

[SWS_CANIF_00847] [Cấu hình phải đảm bảo rằng các Receive L-PDU động sử dụng một phạm vi ID hoặc một mặt nạ và rằng MetaDataItem CAN_ID_32 được cấu hình cho L-SDU. Bên cạnh đó, việc lọc phần mềm phải được kích hoạt cho các L-SDU này.]()

[SWS_CANIF_00848] [Khi nhận được một L-SDU động, CanIf phải đặt CanId vào MetaDataItem của loại CAN_ID_32.]()

#### Physical channel view

Một kênh vật lý được liên kết với một Bộ điều khiển CAN và một Bộ truyền CAN, trong khi một hoặc nhiều kênh vật lý có thể được kết nối với một mạng duy nhất.

CanIf cung cấp các dịch vụ để điều khiển tất cả các thiết bị CAN như Bộ điều khiển CAN và Bộ truyền CAN của tất cả các ECU được hỗ trợ trên kênh CAN. Các API này được sử dụng bởi CanSm để cung cấp một cái nhìn mạng đến ComM (xem [3]) được sử dụng để thực hiện yêu cầu thức dậy và ngủ cho tất cả các kênh vật lý kết nối với một mạng duy nhất.

CanIf chuyển thông tin trạng thái được cung cấp bởi CanDrv và CanTrcv riêng biệt cho mỗi kênh vật lý như thông tin trạng thái cho CanSm (<User_Controller-BusOff>(), xem [SWS_CANIF_00014]).

[SWS_CANIF_00653] [CanIf phải cung cấp một ControllerId, mà trừu tượng hóa từ các Bộ điều khiển khác nhau của các phiên bản CanDrv khác nhau. Phạm vi của ControllerIds trong CanIf phải bắt đầu với '0'. Nó phải được cấu hình thông qua CanIfCtrlId.]()

Example:

[SWS_CANIF_00655] [CanIf phải cung cấp một TransceiverId, mà trừu tượng hóa từ các Transceiver khác nhau của các phiên bản CanTrcv khác nhau. Phạm vi của TransceiverIds trong CanIf phải bắt đầu với '0'. Nó phải được cấu hình thông qua CanIfTrcvId.]()

Example:

Trong quá trình thông báo, CanIf ánh xạ tham số gốc của Bộ điều khiển CAN hoặc Bộ truyền CAN từ mô-đun Driver sang CanSm. Quá trình ánh xạ này được thực hiện khi các tham số Bộ điều khiển CAN hoặc Bộ truyền CAN được tham chiếu được cấu hình với các tham số CanIf trừu tượng ControllerId hoặc TransceiverId.

Figure 7.2: Physical channel view definition example A

CanIf hỗ trợ nhiều kênh CAN vật lý. Các kênh này phải được phân biệt bởi CanSm để điều khiển mạng. API của CanIf cung cấp yêu cầu và kiểm soát đọc cho nhiều kênh CAN vật lý cơ bản.

Hơn nữa, CanIf không phân biệt giữa các loại đặc biệt của các lớp vật lý CAN (tức là CAN tốc độ thấp hoặc CAN tốc độ cao), mà một hoặc nhiều Bộ điều khiển CAN được kết nối đến.

Figure 7.3: Physical channel view definition example B

#### CAN Hardware Unit

Đơn vị Phần cứng CAN kết hợp một hoặc nhiều mô-đun Bộ điều khiển CAN cùng loại, có thể được đặt trên chip hoặc là các thiết bị độc lập nằm ngoài. Mỗi Đơn vị Phần cứng CAN được phục vụ bởi CanDrv tương ứng.

Nếu sử dụng các loại Bộ điều khiển CAN khác nhau, cũng cần áp dụng các loại CanDrv khác nhau với một API thống nhất cho CanIf. CanIf thu thập thông tin về số lượng và loại Bộ điều khiển CAN cũng như các Đối tượng Phần cứng của chúng vào thời điểm cấu hình. Điều này cho phép truy cập đến Bộ điều khiển CAN một cách minh bạch và độc lập với phần cứng từ các mô-đun lớp trên sử dụng HOHs (xem phần 7.2 “Các khóa đối tượng phần cứng” và phần 7.24 “Hỗ trợ nhiều Bộ điều khiển CAN”).

Hình 7.4 cho thấy một Đơn vị Phần cứng CAN bao gồm hai Bộ điều khiển CAN cùng loại được kết nối với hai kênh vật lý:

Figure 7.4: Typical CAN Hardware Unit

#### BasicCAN and FullCAN reception

CanIf phân biệt giữa việc xử lý BasicCAN và FullCAN để kích hoạt việc lọc chấp nhận phần mềm.

Một hộp thư CAN (Đối tượng Phần cứng) cho hoạt động FullCAN chỉ kích hoạt việc truyền hoặc nhận một số CanIds duy nhất. Tương tự, hoạt động BasicCAN của một Đối tượng Phần cứng cho phép truyền hoặc nhận một dãy CanIds.

Một Đối tượng Nhận Phần cứng cho việc nhận BasicCAN được cấu hình có thể nhận một dãy CanIds, đi qua bộ lọc chấp nhận phần cứng của nó. Dãy này có thể vượt quá danh sách được xác định trước của các Rx L-PDU để được nhận bởi HRH này. Do đó, CanIf sau đó phải thực hiện việc lọc phần mềm để chỉ truyền qua danh sách được xác định trước của các Rx L-PDU tới các mô-đun lớp trên tương ứng. Để biết thêm chi tiết, vui lòng tham khảo phần 7.20 “Bộ lọc nhận phần mềm”.

[SWS_CANIF_00467] [CanIf phải cấu hình và lưu trữ một thứ tự trên HTHs và HRHs cho tất cả các HOH tạo ra từ các bộ chứa cấu hình CanIfHthCfg và CanIfHrhCfg.]()

[SWS_CANIF_00468] [CanIf phải tham chiếu đến một bộ lọc chấp nhận phần cứng cho mỗi HOH tạo ra từ các tham số cấu hình CanIfHthIdSymRef và CanIfHrhIdSymRef.]()

Sự khác biệt chính giữa hoạt động BasicCAN và FullCAN nằm ở nhu cầu của cơ chế lọc chấp nhận phần mềm (xem phần 7.20 “Bộ lọc nhận phần mềm”).

[SWS_CANIF_00469] [CanIf phải cung cấp khả năng cấu hình và lưu trữ một bộ lọc chấp nhận phần mềm cho mỗi HRH của loại BasicCAN được cấu hình bởi tham số CanIfHrhSoftwareFilter.]()

[SWS_CANIF_00211] [CanIf phải thực thi việc lọc chấp nhận phần mềm từ [SWS_CANIF_00469] cho HRH được truyền qua hàm gọi lại CanIf_RxIndication().]()

Các đối tượng BasicCAN và FullCAN có thể tồn tại cùng một cấu hình. Nhiều Đối tượng Nhận BasicCAN và FullCAN có thể được sử dụng, nếu được cung cấp bởi các Bộ điều khiển CAN cơ bản.

[SWS_CANIF_00877] [Nếu CanIf nhận một L-PDU (xem CanIf_RxIndication()), nó phải thực hiện các so sánh sau để chọn L-SDU nhận chính xác được cấu hình trong CanIfRxPduCfg:

•	So sánh CanIfRxPduCanId với Mailbox->CanId được truyền (Can_IdType) trừ hai bit có ý nghĩa nhất

•	So sánh CanIfRxPduCanIdType với hai bit có ý nghĩa nhất của Mailbox->CanId (Can_IdType)

](SRS_Can_01140, SRS_Can_01141, SRS_Can_01162)

Cơ bản, CanIf hỗ trợ việc nhận các ID CAN tiêu chuẩn hoặc mở rộng trên một Kênh CAN Vật lý bằng các tham số CanIfTxPduCanIdType và CanIfRxPduCanIdType.

[SWS_CANIF_00281] [CanIf phải chấp nhận và xử lý các ID CAN tiêu chuẩn và mở rộng trên cùng một Kênh Vật l

CanIf  distinguishes between BasicCAN and FullCAN handling for activation of soft- ware acceptance filtering.

A CAN mailbox (Hardware Object) for FullCAN operation only enables transmission or reception of single CanIds. Accordingly, BasicCAN operation of one Hardware Object enables to transmit or receive a range of CanIds.

A Hardware Receive Object for configured BasicCAN reception is able to receive a range of CanIds, which pass its hardware acceptance filter. This range may exceed the list of predefined Rx L-PDUs to be received by this HRH. Therefore, CanIf subse- quently shall execute software filtering to pass only the predefined list of Rx L-PDUs to the corresponding upper layer modules. For more details please refer to section 7.20 “Software receive filter”.

[SWS_CANIF_00467] [CanIf shall configure and store an order on HTHs and HRHs for all HOHs derived from the configuration containers CanIfHthCfg and Can- IfHrhCfg.]()

[SWS_CANIF_00468] [CanIf shall reference a hardware acceptance filter for each HOH derived from the configuration parameters CanIfHthIdSymRef and Can- IfHrhIdSymRef.]()

The main difference between BasicCAN and FullCAN operation is in the need of a software acceptance filtering mechanism (see section 7.20 “Software receive filter”).

[SWS_CANIF_00469] [CanIf shall give the possibility to configure and store a soft- ware acceptance filter for each HRH of type BasicCAN configured by parameter Can- IfHrhSoftwareFilter.]()

[SWS_CANIF_00211] [CanIf shall execute the software acceptance filter from [SWS_CANIF_00469] for the HRH passed by callback function CanIf_RxIndica- tion().]()

BasicCAN and FullCAN objects may coexist in a single configuration setup. Multiple BasicCAN and FullCAN receive objects can be used, if provided by the underlying CAN Controllers.

[SWS_CANIF_00877] [If CanIf receives a L-PDU (see CanIf_RxIndication()), it shall perform the following comparisons to select the correct reception L-SDU configured in CanIfRxPduCfg:

- compare CanIfRxPduCanId with the passed Mailbox->CanId (Can_Id- Type) excluding the two most significant bits

- compare CanIfRxPduCanIdType with the two most significant bits of the passed Mailbox->CanId (Can_IdType)

](SRS_Can_01140, SRS_Can_01141, SRS_Can_01162)

Cơ bản, CanIf hỗ trợ việc nhận các ID CAN tiêu chuẩn hoặc mở rộng trên một Kênh CAN Vật lý thông qua các tham số CanIfTxPduCanIdType và CanIfRxPduCanIdType.

[SWS_CANIF_00281] [CanIf phải chấp nhận và xử lý các ID CAN tiêu chuẩn và mở rộng trên cùng một Kênh Vật lý (= hoạt động chế độ hỗn hợp).](SRS_Can_01140)

Trong một hoạt động chế độ hỗn hợp, các ID CAN tiêu chuẩn và mở rộng có thể được sử dụng kết hợp cùng một lúc trên cùng một mạng CAN. Hoạt động chế độ hỗn hợp có thể được thực hiện, nếu các Đối tượng CAN cơ bản/đầy đủ đã được cấu hình riêng biệt cho hoạt động StandardCAN hoặc ExtendedCAN sử dụng các tham số cấu hình CanIfTxPduCanIdType và CanIfRxPduCanIdType. Trong trường hợp hoạt động chế độ hỗn hợp, thuật toán lọc chấp nhận phần mềm (xem phần 7.20 “Bộ lọc nhận phần mềm”) phải có khả năng xử lý cả hai loại CanIds.

[SWS_CANIF_00281] là một tính năng tùy chọn. Tính năng này có thể được thực hiện bằng các biến thể khác nhau của triển khai, không có tùy chọn cấu hình nào có sẵn.

#### Initialization

EcuM gọi hàm CanIf_Init() của CanIf để khởi tạo toàn bộ CanIf (xem [SWS_CANIF_00001]). Tất cả các biến toàn cục và cấu trúc dữ liệu được khởi tạo bao gồm các cờ và bộ đệm trong quá trình khởi tạo. EcuM thực hiện việc khởi tạo CanDrvs và CanTrcvs một cách riêng biệt thông qua việc gọi các dịch vụ khởi tạo tương ứng của chúng (tham khảo [1] và [2, Specification of CAN Transceiver Driver]).

CanIf mong đợi rằng CAN Controller sẽ giữ ở trạng thái Dừng như sau khi thiết lập lại nguồn sau khi quá trình khởi tạo đã hoàn thành. Ở chế độ này, CanIf và CanDrv không thể truyền hoặc nhận các L-PDU CAN (xem [SWS_CANIF_00001]).

Nếu cần thiết phải thiết lập lại toàn bộ các mô-đun CAN trong quá trình chạy, EcuM phải gọi CanSm (xem [3]) để khởi động các chuyển đổi trạng thái cần thiết của CAN Controller thông qua việc gọi dịch vụ API CanIf_SetControllerMode() của mô-đun Giao diện CAN. CanIf ánh xạ các cuộc gọi từ CanSm thành các cuộc gọi của các CanDrvs tương ứng (xem phần con 8.6.3).

#### Transmit request

Hàm yêu cầu truyền của CanIf, CanIf_Transmit() ([SWS_CANIF_00005]), là một giao diện chung cho các tầng trên để truyền các L-PDU trên mạng CAN. Các tầng giao tiếp trên chỉ khởi động việc truyền thông qua dịch vụ của CanIf mà không trực tiếp truy cập vào CanDrv. Yêu cầu Truyền được khởi động thành công nếu CanDrv có thể viết dữ liệu L-PDU vào đối tượng truyền phần cứng CAN.

Các tầng trên sử dụng dịch vụ API CanIf_Transmit() để khởi động một yêu cầu truyền (tham khảo phần con 8.3.6 “CanIf_Transmit”).

CanIf thực hiện các hành động sau cho việc truyền L-PDU khi gọi dịch vụ CanIf_Transmit():

- Kiểm tra trạng thái khởi tạo của CanIf

- Xác định CanDrv (chỉ nếu có nhiều CanDrvs được sử dụng)

- Xác định HTH để truy cập vào đối tượng truyền phần cứng CAN

- Gọi Can_Write() của CanDrv

Việc truyền được hoàn thành thành công nếu dịch vụ yêu cầu truyền CanIf_Transmit() trả về E_OK.

[SWS_CANIF_00382] [Nếu một L-PDU được yêu cầu được truyền qua một chế độ kênh PDU, mà bằng CANIF_OFFLINE, CanIf phải báo mã lỗi chạy thời gian CANIF_E_STOPPED cho dịch vụ Det_ReportRuntimeError() của DET và CanIf_Transmit() phải trả về E_NOT_OK.](SRS_Can_01126)

Lưu ý cho [SWS_CANIF_00382]: Xem phần con 7.19.2 “Các chế độ kênh PDU”.

Nếu cuộc gọi của Can_Write() trả về CAN_BUSY, vui lòng tham khảo phần 7.12 “Xác nhận truyền” để biết thêm chi tiết.

#### Transmit data flow

Hàm yêu cầu Truyền CanIf_Transmit() dựa trên L-PDUs. Truy cập vào dữ liệu cụ thể của L-SDU được tổ chức bằng các tham số sau:

- L-PDU truyền => ID L-SDU

- Tham chiếu đến một cấu trúc dữ liệu, chứa dữ liệu liên quan đến L-SDU: Con trỏ đến L-SDU, con trỏ đến MetaData và độ dài của L-SDU.

Tham chiếu đến cấu trúc dữ liệu L-SDU được sử dụng như một tham số trong một số dịch vụ API của CanIf, ví dụ như CanIf_Transmit() hoặc dịch vụ gọi lại <User_RxIndica- tion()>. Trong trường hợp L-PDU được cấu hình cho việc truyền kích hoạt, con trỏ L-SDU là một con trỏ null.

Figure 7.5: Transmit data flow

CanIf lưu thông tin về các đối tượng phần cứng có sẵn được cấu hình cho mục đích truyền. Hàm CanIf_Transmit() ánh xạ CanTxPduId tới HTH tương ứng và gọi hàm Can_Write() (xem [SWS_CANIF_00318]).

[SWS_CANIF_00904] [Nếu Tính năng Trình chiếu Bus được kích hoạt toàn cầu (xem CanIfBusMirroringSupport) và đã được kích hoạt bằng cuộc gọi CanIf_EnableBusMirroring() cho một CAN Controller, CanIf sẽ lưu nội dung của mỗi khung trước khi nó được truyền trên bộ điều khiển đó bằng cách sử dụng Can_Write().]

Chú ý: Nội dung khung chỉ nên được cung cấp cho mô-đun Trình chiếu Bus khi nó thực sự được gửi đi. Do đó, nội dung phải được lưu trữ để có thể được cung cấp cho mô-đun Trình chiếu Bus từ bên trong hàm CanIf_TxConfirmation()

.

#### Transmit buffering

##### General behavior

At the CanIf level, the transmission process begins with the call of CanIf_Transmit() and concludes with the invocation of the upper layer module's callback service <User_TxConfirmation()>. Throughout the transmission process, CanIf, CanDrv, and the CAN Mailbox collectively store the L-PDU to be transmitted only once at a single location. Depending on the transmission method, this location can be:

- The CAN hardware transmit object, or

- The Transmit L-PDU Buffer inside CanIf, if transmit buffering is enabled.

For triggered transmission, CanIf only needs to store the transmit request for the given L-PDU but not its data. The data is fetched just in time by means of the trigger transmit function when the HTH is free again. It's crucial to note that a single Tx L-PDU requested for transmission should never be stored twice. This behavior aligns with the usual way of periodic communication on the CAN network.

If transmit buffering is enabled, CanIf will store a Tx L-PDU in a CanIf Transmit L-PDU Buffer (CanIfBufferCfg) if it is rejected by CanDrv at Transmit Request.

Essentially, the overall buffer in CanIf for buffering Tx L-PDUs consists of one or multiple CanIfBufferCfg configurations. Each CanIfBufferCfg is assigned to one or multiple dedicated CanIfBufferHthRef references and can be configured to buffer one or multiple Tx L-PDUs. However, as mentioned earlier, only one instance per Tx L-PDU can be buffered in the overall amount of CanIfBufferCfg.

Hành vi của CanIf trong quá trình truyền L-PDU thay đổi tùy thuộc vào việc có kích hoạt bộ đệm truyền hay không trong cài đặt cấu hình cho Tx L-PDU tương ứng.

Nếu bộ đệm truyền bị vô hiệu hóa:

- Nếu yêu cầu truyền đến CanDrv thất bại (ví dụ, vì hộp thư CAN Controller đang được sử dụng, BasicCAN), L-PDU không được sao chép vào hộp thư của CAN Controller và CanIf_Transmit() trả về giá trị E_NOT_OK.

Nếu bộ đệm truyền được kích hoạt:

- Nếu yêu cầu truyền đến CanDrv thất bại, L-PDU có thể được lưu trữ trong một CanIfTxBuffer, tùy thuộc vào cấu hình CanIfTxBuffer. Trong trường hợp này, API CanIf_Transmit() trả về giá trị E_OK, mặc dù việc truyền không thể được thực hiện ngay lập tức. CanIf chịu trách nhiệm xử lý việc truyền L-PDU còn tồn đọng thông qua callback CanIf_TxConfirmation(), và lớp trên không cần phải thử lại yêu cầu truyền.

Số lượng bộ đệm truyền CanIf Tx L-PDU có sẵn có thể được cấu hình hoàn toàn độc lập với số lượng Tx L-PDU được sử dụng được xác định trong tệp mô tả mạng CAN cho ECU này.

Theo [SWS_CANIF_00835], một Tx L-PDU chỉ định HTHs thông qua container cấu hình CanIfBufferCfg (xem CanIfBufferCfg). Điều này vẫn đúng ngay cả khi bộ đệm truyền không cần thiết. Trong trường hợp này, kích thước bộ đệm (xem CanIfBufferSize) của container CanIfBufferCfg phải được đặt thành 0. Sau đó container cấu hình CanIfBufferCfg chỉ được sử dụng để tham chiếu đến một HTH.

##### Buffer characteristics

CanIfTxPduBufferRef, CanIfBufferCfg, CanIfBufferHthRef and CanIf- BufferSize describe the possible CanIfBufferCfg configurations.

###### Storage of L-PDUs in the transmit L-PDU buffer

CanIf cố gắng lưu trữ một Transmit L-PDU mới hoặc yêu cầu truyền của nó trong Bộ đệm Transmit L-PDU chỉ khi CanDrv trả về CAN_BUSY trong quá trình gọi Can_Write() (xem [SWS_CANIF_00381]).

[SWS_CANIF_00063] [CanIf phải hỗ trợ việc lưu trữ của một CAN L-PDU cho việc truyền BasicCAN trong CanIf, nếu tham số CanIfPublicTxBuffering (xem CanIfPublicTxBuffering) được kích hoạt.] (SRS_Can_01020)

[SWS_CANIF_00849] [Đối với các Transmit L-PDU động, cũng cần lưu trữ CanId trong CanIfTxBuffer.]()

[SWS_CANIF_00381] [Nếu việc lưu trữ bộ đệm truyền được kích hoạt (xem [SWS_CANIF_00063]) và nếu cuộc gọi của Can_Write() cho một PDU được cấu hình để truyền trực tiếp trả về với CAN_BUSY, CanIf sẽ kiểm tra xem có thể lưu trữ CanIf Tx L-PDU, mà được yêu cầu truyền thông qua Can_Write(), trong một CanIfTxBuffer không.] (SRS_Can_01126)

Khi cuộc gọi của Can_Write() trả về CAN_BUSY, CanDrv đã từ chối việc truyền được yêu cầu của L-PDU (xem [1]) vì không có đối tượng phần cứng trống khả dụng vào thời điểm của yêu cầu truyền (yêu cầu Tx).

[SWS_CANIF_00895] [Nếu độ dài dữ liệu bị từ chối vượt quá kích thước được cấu hình, CanIf sẽ:

•	lưu trữ lượng dữ liệu được cấu hình và loại bỏ phần còn lại

•	và báo cáo mã lỗi thời gian chạy CANIF_E_DATA_LENGTH_MISMATCH đến dịch vụ Det_ReportRuntimeError() của DET.]

[SWS_CANIF_00881] [Nếu việc lưu trữ bộ đệm truyền được kích hoạt (xem [SWS_CANIF_00063]) và nếu cuộc gọi của Can_Write() cho một PDU được cấu hình để truyền theo cơ chế kích hoạt trả về với CAN_BUSY, CanIf sẽ kiểm tra xem có thể lưu trữ Yêu cầu Truyền, đã được yêu cầu truyền thông qua Can_Write(), trong một CanIfTxBuffer.] (SRS_Can_01126)

[SWS_CANIF_00835] [Khi CanIf kiểm tra xem có thể lưu trữ một CanIf Tx L-PDU hoặc một Yêu cầu Truyền (xem [SWS_CANIF_00381], [SWS_CANIF_00881]), điều này chỉ có thể thực hiện được nếu CanIf Tx L-PDU được gán (xem CanIfTxPduBufferRef) cho một CanIfBufferCfg (xem CanIfBuffer- Cfg), được cấu hình với kích thước bộ đệm (xem CanIfBufferSize) lớn hơn không.]()

Kích thước bộ đệm của bất kỳ CanIfTxBuffer nào chỉ có thể được cấu hình lớn hơn không, nếu lưu trữ truyền được kích hoạt. Ngoài ra, kích thước bộ đệm của một CanIfTxBuffer duy nhất chỉ có thể được cấu hình lớn hơn không nếu CanIfTxBuffer không được gán cho một FullCAN HTH (xem CanIfBufferSize).

[SWS_CANIF_00836] [Nếu có thể lưu trữ một CanIf Tx L-PDU hoặc một Yêu cầu Truyền, vì kích thước bộ đệm của CanIfTxBuffer được gán lớn hơn không (xem [SWS_CANIF_00835]), CanIf sẽ lưu trữ một CanIf Tx L-PDU hoặc Yêu cầu Truyền trong một phần tử bộ đệm trống của CanIfTxBuffer được gán, nếu CanIf Tx L-PDU hoặc Yêu cầu Truyền chưa được lưu trữ trong CanIfTxBuffer.]()

[SWS_CANIF_00068] [Nếu có thể lưu trữ một CanIf Tx L-PDU hoặc Yêu cầu Truyền, vì kích thước bộ đệm của CanIfTxBuffer được gán lớn hơn không (xem [SWS_CANIF_00835]), CanIf sẽ ghi đè lên CanIf Tx L-PDU được truyền trực tiếp trong CanIfTxBuffer được gán, nếu CanIf Tx L-PDU đã được lưu trữ trong CanIfTxBuffer khi Can_Write() trả về CAN_BUSY.] (SRS_Can_01011)

Chú ý: Không có gì để thực hiện cho các Yêu cầu Truyền đã được lưu trữ trước đó (xem [SWS_CANIF_00068]) do dữ liệu sẽ được CanDrv bắt trực tiếp (sử dụng CanIf_TriggerTransmit()). Do đó, dữ liệu mới nhất sẽ được gửi tự động.

Nếu thứ tự của các yêu cầu truyền của các L-PDU khác nhau cần được giữ nguyên, các yêu cầu truyền của các module lớp trên phải được kết nối với các thông báo xác nhận truyền trước đó. Điều này có nghĩa là một L-PDU sau đó được yêu cầu truyền bởi các module lớp trên chỉ khi xác nhận truyền của L-PDU trước đó được thông báo bởi CanIf.

Chú ý: Ngoài ra, thứ tự của các yêu cầu truyền có thể khác nhau tùy thuộc vào số lượng các đối tượng truyền phần cứng được cấu hình.

[SWS_CANIF_00837] [Nếu kích thước bộ đệm lớn hơn không, tất cả các phần tử bộ đệm đều bận và CanIf_Transmit() được gọi với một L-PDU mới (không có bản sao khác của cùng một L-PDU đã được lưu trữ trong bộ đệm), thì L-PDU mới hoặc Yêu cầu Truyền của nó sẽ không được lưu trữ và CanIf_Transmit() sẽ trả về E_NOT_OK.]()

###### Clearance of transmit L-PDU buffers

[SWS_CANIF_00386] [CanIf sẽ đánh giá trong quá trình xác nhận việc truyền (xem [SWS_CANIF_00007]), liệu có L-PDU CanIf Tx hoặc Yêu cầu Truyền đang chờ lưu trữ trong các bộ đệm CanIfTxBuffers, được gán cho Đối tượng Truyền Phần cứng mới không (xem [SWS_CANIF_00466]).]()

[SWS_CANIF_00668] [Nếu L-PDU CanIf Tx hoặc Yêu cầu Truyền đang chờ lưu trữ trong các CanIfTxBuffers theo [SWS_CANIF_00386], thì CanIf sẽ gọi Can_Write() cho L-PDU CanIf Tx hoặc Yêu cầu Truyền đó (của cái được gán cho Đối tượng Truyền Phần cứng mới) với ưu tiên cao nhất (xem [SWS_CANIF_00070]).]()

[SWS_CANIF_00070] [CanIf sẽ truyền các L-PDU hoặc Yêu cầu Truyền được lưu trữ trong các bộ đệm L-PDU Truyền theo thứ tự ưu tiên (xem [12, ISO 11898-1:2015]) cho mỗi HTH. CanIf sẽ không phân biệt giữa các L-PDU và Yêu cầu Truyền.] ()

[SWS_CANIF_00183] [Khi CanIf gọi hàm Can_Write() cho các L-PDU ưu tiên và Yêu cầu Truyền được lưu trữ trong CanIfTxBuffer và giá trị trả về của Can_Write() là E_OK, sau đó CanIf sẽ loại bỏ L-PDU hoặc Yêu cầu Truyền này khỏi Bộ đệm L-PDU Truyền ngay lập tức, trước khi xác nhận việc truyền trả về.]()

Hành vi được chỉ định trong [SWS_CANIF_00183] đơn giản hóa việc lựa chọn L-PDU truyền mới được lưu trữ trong Bộ đệm L-PDU Truyền.

###### Initialization of transmit L-PDU buffers

[SWS_CANIF_00387] [Khi hàm CanIf_Init() được gọi, CanIf sẽ khởi tạo mọi Bộ đệm L-PDU Truyền được gán cho CanIf.]()

Yêu cầu [SWS_CANIF_00387] là cần thiết để ngăn chặn việc truyền dữ liệu cũ sau khi khởi động lại Bộ điều khiển CAN.

##### Data integrity of transmit L-PDU buffers

[SWS_CANIF_00033] [CanIf phải bảo vệ khỏi việc truy cập song song vào Bộ đệm L-PDU Truyền để truyền L-PDU và Yêu cầu Truyền.](SRS_Can_- 01114)

Điều này có thể được thực hiện bằng cách sử dụng các khu vực độc quyền được xác định trong BSW Scheduler. Các khu vực độc quyền này có thể được cấu hình sao cho tất cả các ngắt sẽ bị vô hiệu hóa khi khu vực độc quyền được nhập vào. Các dịch vụ tương ứng từ mô-đun BSW Scheduler là SchM_Enter_CanIf() và SchM_Exit_CanIf().

Lý do: đối với [SWS_CANIF_00033]: Truy cập đa nhiệm vào Bộ đệm L-PDU Truyền không thể luôn luôn được tránh. Các truy cập Bộ đệm L-PDU Truyền như lưu trữ một L-PDU mới hoặc loại bỏ L-PDU đã được truyền có thể xảy ra theo cách chia lãnh đạo.

#### Transmit confirmation

Nếu một yêu cầu truyền trước đó được hoàn thành thành công, CanDrv thông báo nó cho CanIf

bằng cách gọi CanIf_TxConfirmation() ([SWS_CANIF_00007]).

[SWS_CANIF_00905] [Nếu Bus Mirroring được bật toàn cầu (xem CanIfBusMirror- ingSupport) và đã được kích hoạt bằng cách gọi CanIf_EnableBusMirroring () cho một CAN Controller, CanIf phải gọi Mirror_ReportCanFrame() cho mỗi lần truyền khung trên bộ điều khiển đó được xác nhận bằng CanIf_TxConfir- mation(), cung cấp nội dung đã lưu trữ và ID CAN thực tế.] (SRS_Can_01172)

[SWS_CANIF_00383] [Khi thông báo gọi lại CanIf_TxConfirma- tion() được gọi, CanIf phải xác định lớp giao tiếp lớp trên cùng (xem [SWS_CANIF_00414]), mà được liên kết với L-PDU đã được truyền thành công, và phải thông báo cho nó về quá trình truyền bằng cách gọi dịch vụ xác nhận truyền của CanIf <User_TxConfirmation>(E_OK).]()

Lưu ý cho [SWS_CANIF_00383]: Xem phần 7.12 “Xác nhận truyền”.

Dịch vụ gọi lại <User_TxConfirmation>() được thực hiện bởi mô-đun lớp trên cùng được thông báo.

Một mô-đun lớp giao tiếp trên cùng có thể được thiết kế hoặc cấu hình sao cho các xác nhận truyền có thể được xử lý với các dịch vụ gọi lại đơn lẻ hoặc nhiều dịch vụ gọi lại cho các L-PDU khác nhau hoặc các nhóm L-PDU. Tất cả các dịch vụ đó được gọi bởi CanIf khi xác nhận truyền của yêu cầu truyền L-PDU tương ứng. L-PDU Truyền cho phép phân phát các dịch vụ xác nhận khác nhau được liên kết với mô-đun lớp trên cùng mục tiêu. Sự phân công này được thực hiện tĩnh trong quá trình cấu hình.

Một L-PDU truyền chỉ có thể được phân cho một dịch vụ gọi lại xác nhận truyền đơn lẻ. Vui lòng xem phần con-điều hướng 8.6.3.2 “<User_TxConfirmation>”.

[SWS_CANIF_00740] [Nếu CanIfPublicTxConfirmPollingSupport được bật, CanIf phải lưu trữ thông tin về một xác nhận Tx nhận được cho mỗi CAN Con- troller, nếu trạng thái chế độ điều khiển của bộ điều khiển đó ở trạng thái CAN_CS_STARTED.]()

#### Receive data flow

According to the AUTOSAR Basic Software Architecture the received data will be eval- uated and processed in the upper layer communication stacks (i.e. AUTOSAR COM, CanNm, CanTp, DCM). This means, upper layer modules may neither work with (i.e. change) buffers of CanDrv (Rx) nor do they have access to buffers of CanIf (Tx).

CanIf provides internal buffering in the receive path only if CanIfPublicReadRxP- duDataApi is set to TRUE (refer to section 7.15). Tx buffering is addressed in section

7.11 and dynamic L-PDUs are concerned in section 7.4.

In case of a new reception of an L-PDU CanDrv calls CanIf_RxIndication() (refer to [SWS_CANIF_00006]) of CanIf. The access to the L-PDU specific data is orga- nized by these parameters:

- Hardware Receive Handle (HRH)

- Received CAN Identifier (CanId)

- Received Data Length

- Reference to Received L-PDU

The Received L-PDU is hardware dependent (nibble and byte ordering, access type) and allocated to the lowest layer in the communication system - to CanDrv. HRH serves as a link between CanDrv and the upper layer module using the L-PDU. The HRH identifies one CAN hardware receive object, where a new CAN L-PDU was received.

After the indication of a received L-PDU by CanDrv (CanIf_RxIndication() is called) the CanIf shall proceed as described in 7.14 Receive indication. CanIf is not able to recognize, whether CanDrv uses temporary buffering or a direct hardware access. It expects normalized L-PDU data in calls of the CanIf_RxIndication().

The CAN hardware receive object is locked until the end of the copy process to the tem- porary or upper layer module buffer. The hardware object will be immediately released after CanIf_RxIndication() of CanIf returns to avoid loss of data.

CanDrv, CanIf and the upper layer module, which belongs to the received L-PDU, access the same temporary intermediate buffer, which can be located either in the CAN hardware receive object of the CAN Controller or as temporary buffer in CanDrv.

| [Yes] | «datastore» Temporary buffer in CAN Driver |  |
| --- | --- | --- |

Receive Interrupt

Data

normalization necessary?

[No]

Call CanIf_RxIndication()

Rx L-PDU	[Yes]

received in BasicCAN ?

Software filtering

[No]

[Yes]

Data Length Check enabled?

[L-PDU passed]

[No]

Data Length Check failed ?

[No]

[Yes]

Call Det_ReportRuntimeError() with error code

CANIF_E_INVALID_DATA_LENGTH

[L-PDU not passed]

Call <User_RxIndication>() to upper layers

«datastore»

Copy data to L-PDU buffer

<User_RxIndication>() returns CanIf_RxIndication() returns

Figure 7.6: Receive data flow

#### Receive indication

A call of CanIf_RxIndication() (see [SWS_CANIF_00006]) references in its pa- rameters a newly received CAN L-PDU. If the function CanIf_RxIndication() is

called, the CanIf evaluates the CAN L-PDU for acceptance and prepares the L-SDU for later access by the upper communication layers. The CanIf notifies upper layer modules about this asynchronous event using <User_RxIndication>() (see sub- subsection 8.6.3.3 “<User_RxIndication>”, [SWS_CANIF_00012]), if configured and if this CAN L-PDU is successfully detected and accepted for further processing. The detailed requirements for this behavior follow here.

[SWS_CANIF_00906] [If Bus Mirroring is enabled globally (see CanIfBusMirror- ingSupport) and has been activated with a call to CanIf_EnableBusMirroring () for a CAN Controller, the CanIf shall call Mirror_ReportCanFrame() for each frame reception on that controller that is indicated with CanIf_RxIndication ().](SRS_Can_01172)

[SWS_CANIF_00389] [If the function CanIf_RxIndication() is called, the CanIf shall process the Software Filtering on the received L-PDU, if configured (see multi- plicity of CanIfHrhRangeCfg equals 0..∗). If Software Filtering rejects the received L-PDU, the CanIf shall end the receive indication for that call of CanIf_RxIndica- tion().]()

Note for [SWS_CANIF_00389]: See 7.20.

[SWS_CANIF_00390] [If CanIf accepts an L-PDU received via CanIf_RxIndica- tion() during Software Filtering (see [SWS_CANIF_00389]), CanIf shall process the Data Length check afterwards, if configured (see CanIfPrivateDataLength- Check and CanIfRxPduDataLengthCheck).]()

For further details, please refer to section 7.21 “Data Length Check”.

[SWS_CANIF_00297] [If CanIf has accepted a L-PDU received via CanIf_- RxIndication() during Data Length Check (see [SWS_CANIF_00390]), CanIf shall copy the number of bytes according to the configured Data Length (see ECUC_CanIf_00599) to the static receive buffer, if configured for that L-PDU (see [SWS_CANIF_00198], ECUC_CanIf_00600).]()

[SWS_CANIF_00851] [If MetaData is configured for a received L-SDU, CanIf shall copy the PDU payload to the static receive buffer and the CAN ID to the Meta- DataItem of type CAN_ID_32.]()

[SWS_CANIF_00056] [If CanIf accepts a L-PDU received via CanIf_- RxIndication() during Data Length Check (see [SWS_CANIF_00390], [SWS_CANIF_00026]), CanIf shall identify if a target upper layer module was configured (see configuration descrption of [SWS_CANIF_00012], CanIfRxPdu- UserRxIndicationUL, CanIfRxPduUserRxIndicationName) to be called with its providing receive indication service for the received L-SDU.]()

[SWS_CANIF_00135] [If a target upper layer module was configured to be called with its providing receive indication service (see [SWS_CANIF_00056]), the CanIf shall call this configured receive indication callback service (see CanIfRxPduUserRxIndi- cationName) and shall provide the parameters required for upper layer notification callback functions (see [SWS_CANIF_00012]) based on the parameters of CanIf_- RxIndication().](SRS_BSW_00325)

Note: A single receive L-PDU can only be assigned to a single receive indication call- back service (refer to multiplicity of CanIfRxPduUserRxIndicationName).

Overview: CanIf performs the following steps at a call of CanIf_RxIndication():

- Software Filtering (only BasicCAN), if configured

- Data Length Check, if configured

- buffer received L-SDU if configured

- call upper layer receive indication callback service, if configured.

#### Read received data

The read received data API CanIf_ReadRxPduData() (see [SWS_CANIF_00194]) is a common interface for upper layer modules to read CAN L-SDUs recently received from the CAN network. The upper layer modules initiate the receive request only via CanIf services without direct access to CanDrv. The initiated receive request is suc- cessfully completed, if CanIf wrote the received L-SDU into the upper layer module I-PDU buffer.

The function CanIf_ReadRxPduData() makes reading out data without dependence of reception event (RxIndication) possible. When it is enabled at configuration time (see CanIfPublicReadRxPduDataApi), not necessarily a receive indication service for the same L-SDU has to be configured (see CanIfRxPduUserRxIndicationUL). If needed, the receive indication can be enabled, too.

By this way the type of mechanism to receive L-SDUs (in the upper layer modules of CanIf) can be chosen at configuration time by the parameter CanIfRxPduUser- RxIndicationUL and parameter CanIfRxPduReadData according to the needs of the upper layer module, to which the corresponding receive L-SDU belongs to. For details please refer to section 9.9 “Read received data”.

[SWS_CANIF_00198] [If the configuration parameter CanIfPublicReadRxPdu- DataApi is set to TRUE, CanIf shall store each received L-SDU, at which CanI- fRxPduReadData is enabled, into a receive L-SDU buffer. This means that if the con- figuration parameter CanIfRxPduReadData is set to TRUE, CanIf has to allocate a receive L-SDU buffer for this receive L-SDU.]()

[SWS_CANIF_00199] [After call of CanIf_RxIndication() and passing of soft- ware filtering and Data Length Check, CanIf shall store the received L-SDU in this receive L-SDU buffer. During the call of CanIf_ReadRxPduData() the assigned receive L-SDU buffer containing a recently received L-SDU, CanIf shall avoid pre- emptive receive L-SDU buffer access events (refer to [SWS_CANIF_00064]) to that receive L-SDU buffer.]()

#### Read Tx/Rx notification status

In addition to the notification callback functions CanIf provides the API service CanIf_ReadTxNotifStatus() (see [SWS_CANIF_00202]) to read the transmit confirmation status of any transmit L-SDU and the API service CanIf_ReadRxNo- tifStatus() is provided to read the receive indication status of any receive L-SDU.

CanIf’s API services CanIf_ReadTxNotifStatus() (see [SWS_CANIF_00202]) and CanIf_ReadRxNotifStatus() (see [SWS_CANIF_00230]) can be en- abled/disabled globally or per L-SDU at pre-compile time configuration using the con- figuration parameters CanIfPublicReadTxPduNotifyStatusApi, CanIfPubli- cReadRxPduNotifyStatusApi, CanIfTxPduReadNotifyStatus, and CanI- fRxPduReadNotifyStatus.

[SWS_CANIF_00472] [If configuration parameter CanIfPublicReadTxPduNoti- fyStatusApi is set to TRUE, CanIf shall store the current notification status for each transmit L-SDU.]()

[SWS_CANIF_00473] [If configuration parameter CanIfPublicReadRxPduNoti- fyStatusApi is set to TRUE, CanIf shall store the current notification status for each receive L-SDU.]()

Rationale for [SWS_CANIF_00391] and [SWS_CANIF_00393] respectively [SWS_CANIF_00392] and [SWS_CANIF_00394]: This ’read-and-consume’ be- havior ensures, that at least one successful transmit or receive event occurred after last call of this service.

#### Data integrity

[SWS_CANIF_00064] Shared code shall be reentrant [CanIf shall protect preemp- tive events, which access shared resources, that could be changed during CanIf’s event handling, against each other.](SRS_BSW_00312)

Rationale: An attempt to update the data in the upper layer module buffers as well as in CanIf’s internal buffers has to be done with respect to possible changes done in the context of an interrupt service routine or other preemptive events. Preemptive events probably occur either from preemptive tasks, multiple CAN interrupts, if multiple physical channels i.e. for gateways are used, or in case of other peripherals or net- work systems interrupts, which have the needs to transmit and receive L-PDUs on the network.

Handling of shared transmit and receive L-PDU/L-SDU buffers are critical issues for the implementation of CanIf. Therefore CanIf shall ensure data integrity and thus use appropriate mechanisms for access to shared resources like transmission/reception L-PDU/L-SDU buffers. Preemptive events, i.e. transmission and reception event from other CAN Controllers could compromise data integrity by writing into the same L-PDU/L-SDU buffer.

CanIf can e.g. use CanDrv services to enable (Can_EnableControllerInter- rupts()) and disable (Can_Disable-ControllerInterrupts()) CAN interrupts and its notifications at entry and exit of the critical sections separately for each CAN Controller. If there are common resources for multiple CAN Controllers, the entire CAN Interrupts must be locked. These sections must not take a long time in order to prevent serious performance degradation. Thus copying of data, change of static variables, counters and semaphores should be carried out inside these critical sections. It is up to the implementation to use appropriate mechanisms to guarantee data integrity, interrupt ability and reentrancy.

The transmit request API CanIf_Transmit() must be able to operate re-entrant to allow multiple transmit request calls caused by different preemptive events of different L-PDUs/L-SDUs. CanDrv’s transmit request API Can_Write() operates re-entrant as well.

#### CAN Controller Mode

##### General Functionality

CanIf provides services for controlling the communication mode of all supported CAN Controllers represented by the underlying CanDrv. This means that all CAN Controllers are controlled by the corresponding provided API services to request and read the current controller mode.

The CAN Controller status may be changed at request of the upper layer by the calling of CanIf_SetControllerMode() service. The request is passed by CanIf via the CanDrv API to the addressed CAN Controller.

The consistent management of all CAN Controllers connected at one CAN network is the task of CanSm. By this way CanSm is responsible to set all CAN Controllers of one CAN network sequentially to sleep mode or to wake them up.

CanIf accepts every state transition request by calling the function CanIf_SetCon- trollerMode() or CanIf_ControllerBusOff(). CanIf does not decide if a re- quested mode transition of the CAN Controller is valid or not. CanIf only interacts with CanDrv by fetching the current mode and execution of requested mode transi- tions.

This network related state machine is implemented in CanSm. Refer to [3]. CanIf only stores the requested mode and executes the requested transition.

Hint: As optimisation to avoid frequent requests to CanDrv for internal use the last state indicated by CanIf_ControllerModeIndication() and Can_GetCon- trollerMode() could be stored per controller.

Hint: It has to be regarded that not only CanSm is able to request CAN Controller Mode changes.

##### CAN Controller Operation Modes

According to the requested operation mode by CanSm, CanIf forwards request Can- Drvs.

[SWS_CANIF_00677] [If a controller mode referenced by ControllerId is in state CAN_CS_STOPPED and if the PduIdType parameter in a call of CanIf_Transmit() is assigned to that CAN Controller, then the call of CanIf_Transmit() does not result in a call of Can_Write() (see [SWS_CANIF_00317]) and returns E_NOT_OK.] ()

[SWS_CANIF_00485] [If a controller mode referenced by ControllerId enters state CAN_CS_STOPPED, then CanIf shall clear the CanIf transmit buffers assigned to the CAN Controller corresponding.]()

[SWS_CANIF_00739] [If a controller mode referenced by ControllerId enters state CAN_CS_STOPPED, then CanIf shall inform corresponding upper layer modules about failed transmission by calling <User_TxConfirmation>(id, E_NOT_OK) for every outstanding TxConfirmation assigned to that CAN Controller. If CanIfPublicTx- ConfirmPollingSupport is enabled, CanIf shall also clear the information about a TxConfirmation (see [SWS_CANIF_00740]).]()

Note: This ensures, that for each PDU, which shall be transmitted via CanIf_Trans- mit(), either a positive or negative <User_TxConfirmation>() is called.

[SWS_CANIF_00724] [When callback CanIf_ControllerBusOff(Control- lerId) is called, the CanIf shall call CanSM_ControllerBusOff(Control- lerId) of the CanSm or a CDD (see [SWS_CANIF_00559], [SWS_CANIF_00560]).] ()

Note for [SWS_CANIF_00724]: See subsubsection 8.6.3.9 “<User_ControllerMod- eIndication>”.

[SWS_CANIF_00711] [When callback CanIf_ControllerModeIndication (ControllerId, ControllerMode) is called, CanIf shall call CanSm_Con- trollerModeIndication(ControllerId, ControllerMode) of the CanSm or a CDD (see [SWS_CANIF_00691], [SWS_CANIF_00692]).]()

Note for [SWS_CANIF_00711]: See subsubsection 8.6.3.9 “<User_ControllerMod- eIndication>”.

[SWS_CANIF_00712]	[When	callback	CanIf_TrcvModeIndication (Transceiver, TransceiverMode) is	called,		CanIf shall	call	CanSM_- TransceiverModeIndication(TransceiverId, TransceiverMode) of the CanSm or a CDD (see [SWS_CANIF_00697], [SWS_CANIF_00698]).]()

Note for [SWS_CANIF_00712]: See subsubsection 8.6.3.9 “<User_ControllerMod- eIndication>”.

##### Controller Mode Transitions

The API for state change requests to the CAN Controller behaves in an asyn- chronous manner with asynchronous notification via callback services.

The real transition to the requested mode occurs asynchronously based on setting of transition requests in the CAN controller hardware, e.g. request for sleep transition CAN_CS_SLEEP. After successful change to e.g. CAN_CS_SLEEP mode CanDrv calls function CanIf_ControllerModeIndication() and CanIf in turn calls function

<User_ControllerModeIndication>(). If CAN transitions very fast, CanIf_- ControllerModeIndication() can be called during CanIf_SetController- Mode(). This is implementation specific.

Unsuccessful or no mode transitions of the CAN Controllers have to be tracked by upper layer modules. Mode transitions CAN_CS_STARTED and CAN_CS_STOPPED are treated similar.

Upper layer modules of CanIf can poll the current Controller Mode by CanIf_Get- ControllerMode().

Not all types of CAN Controllers support Sleep and Wake-Up Mode. These modes are then encapsulated by CanDrv by providing hardware independent oper- ation modes via its interface, which has to be managed by CanIf.

Note: It is possible that during transition from CAN_CS_STOPPED to CAN_CS_SLEEP CAN Controller may indicate a wake-up interrupt to the ECU Integration Code.

CanIf distinguishes between internal initiated CAN controller wake-up request (inter- nal request) and network wake-up request (external request). The internal request is initiated by call of CanIf’s function CanIf_SetControllerMode(ControllerId, CAN_CS_STARTED) and it is an internal asynchronous request. The external request is a CAN controller event, which is notified by CanDrv or CanTrcv to the ECU Inte- gration Code. For details see respective UML diagram in the chapter "CAN Wakeup Sequences" of document [13].

##### Wake-up

The ECU supports wake-up over CAN network, regardless of the used wake-up method (directly about CAN Controller or CAN Transceiver), only if the CAN Controller and CAN Transceiver are set to some kind of "listen for wake-up" mode. This is usually a Sleep Mode, where the usual communication is disabled. Only this mode ensures that the CAN Controller is stopped. Thus, the wake-up interrupt can be enabled.

###### Wake-up detection

If wake-up support is enabled (see ECUC_CanIf_00843) CanIf is notified by the Inte- gration Code about a detected CAN wake-up by the service CanIf_CheckWakeup() (see CAN Wakeup Sequences of [13]).

In case of a CAN bus "wake-up" event the function CanIf_CheckWakeup(Wakeup- Source) may be called during execution of EcuM_CheckWakeup(WakeupSource) (see wake-up sequence diagrams of EcuM). CanIf in turn checks by configured input reference to EcuMWakeupSource in CanDrvs, which CanDrvs have to be checked. CanIf gets this information via reference CanIfCtrlCanCtrlRef.

The Communication Service, which is called, belongs to the service defined during configuration (see CanIfDispatchCfg). In this way EcuM as well as CanSm are able to change CAN Controller States and to control the system behavior concerning the BusOff recovery or wake-up procedure.

[SWS_CANIF_00395] [When CanIf_CheckWakeup(EcuM_WakeupSourceType WakeupSource) is invoked, CanIf shall query CanDrvs / CanTrcvs via CanTrcv_- CheckWakeup() or Can_CheckWakeup(), which exact CAN hardware device caused the bus wake-up.]()

Note: It is implementation specific, which controllers and transceivers are queried.

CanIf just has to find out the exact CAN hardware device.

[SWS_CANIF_00720] [If at least one function call of Can_CheckWakeup() or CanTrcv_CheckWakeup() returns E_OK to CanIf, then CanIf_CheckWakeup() shall return E_OK.]()

[SWS_CANIF_00678] [If all calls of Can_CheckWakeup() or CanTrcv_Check- Wakeup() return E_NOT_OK to CanIf, then CanIf_CheckWakeup() shall return E_NOT_OK.]()

###### Wake-up Validation

Note: When a CAN Controller / CAN Transceiver detects a bus wake-up event, then this will be notified to the ECU State Manager directly. If such a wake-up event needs to be validated, the EcuM (or a CDD) switches on the corresponding CAN Con- troller (CanIf_SetControllerMode()) and CAN Transceiver (CanIf_Set- TrcvMode()) (For more details see chapter 9 of [13]).

Attention: CanIf notifies the upper layer modules about received messages after the PDU Channel Mode has been set to CANIF_ONLINE or CANIF_TX_OFFLINE. Thus, it is necessary that the PDU Channel Mode is not set to CANIF_ONLINE or CANIF_- TX_OFFLINE if wake-up validation is required.

Note: As per [SWS_CAN_00411] and CAN Controller State Diagram (see [1]) a direct transition from mode CAN_CS_SLEEP to CAN_CS_STARTED is not allowed.

SWS_CANIF_00286] [If CanIfPublicWakeupCheckValidSupport equals TRUE, CanIf enables the detection for CAN wake-up validation. Therefore, CanIf stores the event of the first valid call of CanIf_RxIndication() of a CAN Controller which has been set to CAN_CS_STARTED. The first call of CanIf_RxIndication() is valid:

        - only for received NM messages if CanIfPublicWakeupCheckValidByNM is

TRUE

        - for all received messages corresponding to a configured Rx PDU if CanIfPub- licWakeupCheckValidByNM is FALSE.

](SRS_Can_01151)

[SWS_CANIF_00179] [<User_ValidateWakeupEvent>(sources) shall be called during CanIf_CheckValidation(WakeupSource), whereas sources is set to WakeupSource, if the event of the first called CanIf_RxIndication() is stored in CanIf at the corresponding CAN Controller.](SRS_Can_01136)

Note: The parameter of the function <User_ValidateWakeupEvent>() is of type:

        - sources: EcuM_WakeupSourceType (see [13])

[SWS_CANIF_00756] [When controller mode is set to CAN_CS_SLEEP the stored event from previous wake-up (first call of CanIf_RxIndication) shall be cleared (see [SWS_CANIF_00179]).]()

#### PDU channel mode control

##### PDU channel groups

Each L-PDU is assigned to one dedicated physical CAN channel connected to one CAN Controller and one CAN network. By this way all L-PDUs belonging to one Physical Channel can be controlled on the view of handling logically single L-PDU channel groups. Those logical groups represent all L-PDUs of one ECU connected to one underlying CAN network.

Figure 7.7 below shows one possible usage of L-PDU channel group and its relation to the upper layers and/or networks.

An L-PDU can only be assigned to one channel group.

Typical users like PduR or the Network Management are responsible for controlling the PDU operation modes.

Figure 7.7: Channel PDU groups

##### PDU channel modes

CanIf provides the services CanIf_SetPduMode() and CanIf_GetPduMode() to prevent the processing of

- all Transmit L-PDUs belonging to one logical channel,

- all Transmit L-PDUs and Receive L-PDUs belonging to one logical channel.

Changing the PDU channel mode is only allowed in case corresponding controller mode equals CAN_CS_STARTED (refer to [SWS_CANIF_00874]).

While CANIF_ONLINE and CANIF_OFFLINE affecting the whole communicatoin the PDU channel modes CANIF_TX_OFFLINE and CANIF_TX_OFFLINE_ACTIVE en- able/disable transmission path seperately.

CanIf provides information about the current PDU channel mode via the service

CanIf_GetPduMode().

Figure 7.8: PDU channel mode control

Figure 7.8 shows a diagram with possible PDU channel modes. Each L-PDU chan- nel can be in CANIF_OFFLINE (no communication), CANIF_TX_OFFLINE (passive mode => listen without sending), CANIF_TX_OFFLINE_ACTIVE (simulated transmis- sion with listening), and CANIF_ONLINE (full communication). The default state is the CANIF_OFFLINE mode.

###### CANIF_OFFLINE

[SWS_CANIF_00864] [During initialization CanIf shall switch every channel to

CANIF_OFFLINE.]()

[SWS_CANIF_00865] [If CanIf_SetControllerMode(ControllerId, CAN_- CS_SLEEP) is called, CanIf shall set the PDU channel mode of the corresponding channel to CANIF_OFFLINE.]()

[SWS_CANIF_00073] [For Physical Channels switching to CANIF_OFFLINE

mode CanIf shall:

        - prevent forwarding of transmit requests CanIf_Transmit() of associated L-

-PDUs to CanDrv (return E_NOT_OK to the calling upper layer modules),

        - clear the corresponding CanIf transmit buffers,

        - prevent invocation of receive indication callback services of the upper layer mod- ules,

        - prevent invocation of transmit confirmation callback services of the upper layer modules.

]()

[SWS_CANIF_00866] [If CanIf_SetControllerMode(ControllerId, CAN_- CS_STOPPED) or CanIf_ControllerBusOff(ControllerId) is called, CanIf shall set the PDU channel mode of the corresponding channel to CANIF_TX_OF- FLINE.]()

[SWS_CANIF_00489] [For Physical Channels switching to CANIF_TX_OFFLINE

mode CanIf shall:

        - prevent forwarding of transmit requests CanIf_Transmit() of associated L-

-PDUs to CanDrv (return E_NOT_OK to the calling upper layer modules),

        - clear the corresponding CanIf transmit buffers,

        - prevent invocation of transmit confirmation callback services of the upper layer modules.

        - enable invocation of receive indication callback services of the upper layer mod- ules.

]()

The BusOff notification is implicitly suppressed in case of CANIF_OFFLINE and CANIF_TX_OFFLINE due to the fact, that no L-PDUs can be transmitted and thus the CAN Controller is not able to go in BusOff mode by newly requested L-PDUs for transmission.

[SWS_CANIF_00118] [If those Transmit L-PDUs, which are already waiting for transmission in the CAN Transmit Hardware Object, will be transmitted imme- diately after change to CANIF_TX_OFFLINE or CANIF_OFFLINE mode and a subse- quent BusOff event occurs, CanIf does not prohibit execution of the BusOff notifica- tion <User_ControllerBusOff>(ControllerId).]()

The wake-up notification is not affected concerning PDU channel mode changes.

###### CANIF_ONLINE

[SWS_CANIF_00075] [For Physical Channels switching to CANIF_ONLINE mode CanIf shall:

        - enable forwarding of transmit requests CanIf_Transmit() of associated L-

-PDUs to CanDrv,

        - enable invocation of receive indication callback services of the upper layer mod- ules,

        - enable invocation of transmit confirmation callback services of the upper layer modules.

]()

###### CANIF_OFFLINE_ACTIVE

If CanIfTxOfflineActiveSupport = TRUE CanIf provides simulation of suc- cessful transmission by CANIF_TX_OFFLINE_ACTIVE mode. This mode is enabled by call of CanIf_SetPduMode(ControllerId, CANIF_TX_OFFLINE_ACTIVE) and only affects the transmission path.

[SWS_CANIF_00072] [For every L-PDU assigned to a channel which is in CANIF_- TX_OFFLINE_ACTIVE mode CanIf shall call the transmit confirmation callback ser- vices of the upper layer modules immediately instead of buffering or forwarding of the L-PDUs to CanDrv during the call of CanIf_Transmit().]()

Note: During CANIF_TX_OFFLINE_ACTIVE mode the upper layer has to handle the execution of the transmit confirmations. The transmit confirmation handling is executed immediately at the end of the transmit request (see [SWS_CANIF_00072]).

Rational: This functionality is useful to realize special operating modes (i.e. diagnosis passive mode) to avoid bus traffic without impact to the notification mechanism. This mode is typically used for diagnostic usage.

#### Software receive filter

Not all L-PDUs, which may pass the hardware acceptance filter and therefore are successful received in BasicCAN Hardware Objects, are defined as Receive L-

-PDUs and thus needed from the corresponding ECU. CanIf optionally filters out these L-PDUs and prohibits further software processing.

Certain software filter algorithms are provided to optimize software filter runtime. The approach of software filter mechanisms is to find out the corresponding L-PDU from the HRH and CanId currently being processed. After the L-PDU is found, CanIf accepts the reception and enables upper layers to access L-SDU information directly.

##### Software filtering concept

The configuration tool handles the information about hardware acceptance filter set- tings. The most important settings are the number of the L-PDU hardware objects and their range. The outlet range defines, which Receive L-PDUs belongs to each Hardware Receive Object. The following definitions are possible:

- a single Receive L-PDU (FullCAN reception),

- a list of Receive L-PDUs or

- one or multiple ranges of Receive L-PDUs can be linked to a Hardware Re- ceive Object (BasicCAN reception).

For definition of range reception it is necessary to define at least one Rx L-PDU where the CanId or the complete ID range is inside the defined range.

[SWS_CANIF_00645] [A range of CanIds which shall pass the software receive filter shall either be defined by its upper limit (see CanIfHrhRangeRxPduUpperCanId) and lower limit (see CanIfHrhRangeRxPduLowerCanId) CanId, or by a base ID (see CanIfHrhRangeBaseId) and a mask that defines the relevant bits of the base ID (see CanIfHrhRangeMask).]()

Note: Software receive filtering is optional (see multiplicity of 0..∗ in Can- IfHrhRangeCfg).

[SWS_CANIF_00646]   [Each   configurable   range   of   CanIds (see [SWS_CANIF_00645]), which shall pass the software receive filter, shall be configurable either for Standard CAN IDs or Extended CAN IDs via Can- IfHrhRangeRxPduRangeCanIdType.]()

Receive L-PDUs are provided as constant structures statically generated from the communication matrix. They are arranged according to the corresponding hardware acceptance filter, so that there is one single list of receive CanIds for every Hardware Receive Object (HRH). The corresponding list can be derived by the HRH, if multiple BasicCAN objects are used. The subsequent filtering is the search through one list of multiple CanIds by comparing them with the new received CanId. In case of a hit the Receive L-PDU is derived from the found CanId.

[SWS_CANIF_00030] [If the CanId of the received L-PDU in the HRH is configured to be received, then CanIf shall accept this L-PDU and the software filtering algorithm shall derive the corresponding Receive L-PDU from the found CanId.](SRS_Can_- 01018)

Figure 7.9: Software filtering example

[SWS_CANIF_00852] [If a range is (partly) contained in another range, or a single CanId is contained in a range, the software filter shall select the L-PDU based on the following assumptions:

- A single CanId is always more relevant than a range.

- A smaller range is more relevant than a larger range.

]()

##### Software filter algorithms

The choice of suitable software search algorithms it is up to the implementation of CanIf. According to the wide range of possible receive BasicCAN operations provided by the CAN Controller it is recommended to offer several search algorithms like linear search, table search and/or hash search variants to provide the most optimal solution for most use cases.

#### Data Length Check

The received Data Length value is compared with the configured Data Length value of the received L-PDU. The configured Data Length value shall be derived from the size of used bytes inside this L-PDU. The configured Data Length value may not be necessarily that Data Length value defined in the CAN communication matrix and used by the sender of this CAN L-PDU.

[SWS_CANIF_00026]  [CanIf  shall  accept  all  received  L-PDUs (see [SWS_CANIF_00390]) with a Data Length value equal or greater then the configured Data Length value (see CanIfRxPduDataLength).](SRS_Can_01005)

[SWS_CANIF_00902] [The Data Length Check shall be processed if it is enabled globally (see CanIfPrivateDataLengthCheck) and not disabled individually per PDU (see CanIfRxPduDataLengthCheck).]()

Hint: If the Data Length Check is disabled globally, it can’t be enabled individually per PDU.

[SWS_CANIF_00168] [If the Data Length Check rejects a received L-PDU (see [SWS_CANIF_00026]), CanIf shall report runtime error code CANIF_E_IN- VALID_DATA_LENGTH to the Det_ReportRuntimeError() service of the DET module.]()

[SWS_CANIF_00829] [CanIf shall pass the received (see [SWS_CANIF_00006]) length value to the target upper layer module (see [SWS_CANIF_00135]), if the Data Length Check is passed.]()

[SWS_CANIF_00830] [CanIf shall pass the received (see [SWS_CANIF_00006]) length value to the target upper layer module (see [SWS_CANIF_00135]), if the Data Length Check is not configured (see CanIfPrivateDataLengthCheck and CanI- fRxPduDataLengthCheck)]()

#### L-SDU dispatcher to upper layers

Rationale: At transmission side the L-SDU dispatcher has to find out the correspond- ing Tx confirmation callback service of the target upper layer module. At reception side each L-SDU belongs to one single upper layer module as destination. This relation is

assigned statically at configuration time. The task of the L-SDU dispatcher inside of CanIf is to find out the customer for a received L-SDU and to dispatch the indica- tions towards the found upper layer. These transmit confirmation as well as receive indication notification services may exist several times with different names defined in the notified upper layer modules. Those notification services are statically configured, depending on the layers that have to be served.

#### Polling mode

The polling mode provides handling of transmit, receive and error events occurred in the CAN hardware without the usage of hardware interrupts. Thus the CanIf and the CanDrv provides notification services for detection and execution corresponding hardware events. In polling mode the behavior of these CanIf notification services does not change. By this way upper layer modules are abstracted from the strategy to detect hardware events. If different CanDrvs are in use, the calling frequency has to be harmonized during configuration setup and system integration.

These notification services are able to detect new events that occurred in the CAN hardware objects since its last execution. The CanIf’s notification services for forward- ing of detected events by the CanDrv are the same like for interrupt operation (see section 8.4 “Callback notifications”).

The user has to consider, that the CanIf has to be able to perform notification ser- vices triggered by interrupt on interrupt level as well as to perform invoked notification services on task level. If any access to the CAN controller’s mailbox is blocked, subse- quent transmit buffering takes place (refer section 7.11 “Transmit buffering”).

The Polling and Interrupt mode can be configured for each underlying CAN controller.

#### Multiple CAN Driver support

CanIf needs a specific mapping to cover multiple CanDrv to provide a common inter- face to upper layers. Thus, CanIf must dispatch all actions up-down to the APIs of the corresponding CanDrv and underlying CAN Controller(s). For the way down-up CanIf has to provide adequate callback notifications to differentiate between multiple CanDrvs.

Each CanDrv supports a certain number of underlying CAN Controllers and a fixed number of HTHs/HRHs. Each CanDrv has an own numbering area, which starts always at zero for CAN Controllers and HTHs. CanIf has to derive the corresponding CanDrv from the L-SDU passed in the APIs. The parameters have to be translated accordingly: i.e. L-SDU => HTH/HRH, CanId, Data Length."

The support for multiple CanDrvs can be enabled and disabled by the configuration parameter CanIfPublicMultipleDrvSupport.

##### Transmit requests by using multiple CAN Drivers

Each Transmit L-PDU enables CanIf to derive the corresponding CAN Con- troller and implicitly CanDrv serving the affected Hardware Unit. Resolving of these dependencies is possible because of the construction of the CAN Controller Handle: it combines CanDrv Handle and the corresponding CAN Controller in the Hardware Unit.

At configuration time a CAN Controller Handle will be mapped to each CAN Con- troller. The sequence diagram Figure 7.10 below demonstrates two transmit re- quests directed to different CanDrvs. CanIf needs only to select the corresponding CanDrv in order to call the correct API service.

Note: Figure 7.10 and the following table serve only as an example. Finally, it is up to the implementation to access the correct APIs of underlying CanDrvs.

CanIf User

«mod...

CanIf

Can_99_Ext1: Can

«Peripheral»

CanController A: CanController

Can_99_Ext2: Can

«Peripheral»

CanController B: CanController

alt CAN Controller A/B [CAN Controller A used]

CanIf_Transmit(Std_ReturnType, PduIdType, const PduInfoType*)

Can_Write(Std_ReturnType, Can_HwHandleType, const Can_PduType*)

Copy L-PDU in CAN Hardware A()

Copy L-PDU in CAN Hardware A()

Can_Write()

CanIf_Transmit()

[CAN Controller B used]

CanIf_Transmit(Std_ReturnType, PduIdType, const PduInfoType*)

Can_Write(Std_ReturnType, Can_HwHandleType, const Can_PduType*)

Copy L-PDU in CAN Hardware B()

Copy L-PDU in CAN Hardware B()

Can_Write()

CanIf_Transmit()

Figure 7.10: Transmission request with multiple CAN Drivers - simplified

| Operations called | Description |
| --- | --- |
| CanIf_Transmit (PduId_1, PduInfoPtr_1) | Upper layer initiates a transmit request. The PduId is used for tracing the requested CAN Controller and then to serving the Hardware Unit. The number of the Hardware Unit is relevant for the dispatch as it is used as index for the array with pointer to functions. At first the number of the PDU channel group will be extracted from the PduId_1. Each PDU channel group refers to a CAN channel and thus as well the Hardware Unit Number and the CAN Controller Number. |

|  | The Hardware Unit Number points on an instance of CanDrv and therefore refers all API services configured for the used Hardware Unit(s). One of these services is the requested transmit service. |
| --- | --- |
| Can_Write (Hth, PduInfoPtr) | Request for transmission to the corresponding CAN_Driver serving i.e. CAN Controller #0 within the "A" Hardware Unit. |
| Hardware request | All L-PDU data will be set in the Hardware of i.e. CAN Controller #0 within Hardware Unit "A" and the transmit request enabled. |
| CanIf_Transmit (PduId_2, PduInfoPtr_2) | Upper layer initiates Transmit Request. The PduId leads to another CAN Controller and then to another Hardware Unit. The number of the Hardware Unit is relevant for the dispatch as it is used as index for the array with pointer to functions. At first the number of the PDU channel group will be extracted from the PduId_2. Each PDU channel group refers to a CAN channel and thus as well to the Hardware Unit Number and to the CAN Controller Number. The Hardware Unit Number points on an instance of CanDrv and therefore refers all API services configured for the used Hardware Unit(s). One of these services is the requested transmit service. |
| Can_Write (Hth, PduInfoPtr_2) | Request for transmission to the corresponding CAN_Driver serving i.e. CAN Controller #1 within the "B" Hardware Unit. |
| Hardware request | All L-PDU data will be set in the Hardware of i.e. CAN Controller #1 within Hardware Unit "B" and the transmit request enabled. |

##### Notification mechanism using multiple CAN Drivers

Even if multiple CanDrvs are used in a single ECU Every notification callback service invoked by CanDrvs at the CanIf exists only once. This means, that CanIf has to identify calling CanDrv using the passed parameters. CanIf identifies the calling CanDrv from the ControllerId within the Mailbox (Can_HwType) structure.

CanIf User

Receive Interrupt()

CanIf_RxIndication(const Can_HwType*, const PduInfoType*)

<User_RxIndication>(PduIdType, const PduInfoType*)

Received L-PDU

validation check (SW Filtering, Data Length Check)

Copy Data()

Copy Data()

<User_RxIndication>()

CanIf_RxIndication()

Receive Interrupt()

Receive Interrupt()

CanIf_RxIndication(const Can_HwType*, const PduInfoType*)

Received L-PDU validation check (SW Filtering, Data Length Check)

<User_RxIndication>(PduIdType, const PduInfoType*)

Copy data()

Copy data()

<User_RxIndication>()

CanIf_RxIndication()

Receive Interrupt()

«Peripheral» CanController

Can_99_Ext2: Can

Can_99_Ext1: Can

«module» CanIf

Figure 7.11: Receive interrupt with multiple CanDrvs - simplified

| Operations called | Description |
| --- | --- |
| Receive Interrupt | CAN Controller 1 signals a successful reception and triggers a receive interrupt. The ISR of CanDrv A is invoked. |
| CanIf_RxIndication (Mailbox_1, PduInfoPtr_1) | The reception is indicated to CanIf by calling of CanIf_RxIndication(). The pointer Mailbox_1 identifies the HRH and its corresponding CAN Controller, which contains the received L-PDU specified by PduInfoPtr_1. |
| Validation check (SW Filter- ing, Data Length Check) | The Software Filtering checks, whether the Received L-PDU will be processed on a local ECU. If not, the Received L-SDU is not indicated to upper layers and further processing is suppressed. If the L-PDU is found, the Data Length of the Received L-PDU is compared with the expected, statically configured one for the received L-PDU. |
| <User_RxIndication> (CanRxPduId_1, CanPduInfoPtr_1) | The corresponding receive indication service of the upper layer is called. This signals a successful reception to the target upper layer. The parameter CanRxPduId_1 specifies the ID of the received L-SDU. The second parameter is the reference on PduInfoType which provides access to the buffer containing the L-SDU. |

| Receive Interrupt | The CAN Controller 2 signals a successful reception and triggers a receive interrupt. The ISR of CanDrv B is invoked. |
| --- | --- |
| CanIf_RxIndication (Mailbox_2, PduInfoPtr_2) | The reception is indicated to CanIf by calling of CanIf_RxIndication(). The pointer Mailbox_2 identifies the HRH and its corresponding CAN Controller, which contains the received L-PDU specified by PduInfoPtr_2. |
| Validation check (SW Filter- ing, Data Length Check) | The Software Filtering checks, whether the Received L-PDU will be processed on a local ECU. If not, the Received L-SDU is not indicated to upper layers and further processing is suppressed. If the L-PDU is found, the Data Length of the Received L-PDU is compared with the expected, statically configured one for the received L-PDU. |
| <User_RxIndication> (CanRxPduId_2, CanPduInfoPtr_2) | The corresponding receive indication service of the upper layer is called. This signals a successful reception to the target upper layer. The parameter CanRxPduId_2 specifies the ID of the received L-SDU. The second parameter is the reference on PduInfoType which provides access to the buffer containing the L-SDU. |

#### Partial Networking

[SWS_CANIF_00747] [If Partial Networking (PN) is enabled (see CanIfPublicPn- Support), CanIf shall support a PnTxFilter per CAN Controller which overlays the PDU channel modes.]()

[SWS_CANIF_00748] [The PnTxFilter of [SWS_CANIF_00747] shall only have an effect and transition its modes (enabled/disabled) if more than zero Tx L-PDUs per CAN Controller are configured as CanIfTxPduPnFilterPdu (see CanIfTx- PduPnFilterPdu).]()

[SWS_CANIF_00863] [PnTxFilter shall be enabled during initialization (ref. to [SWS_CANIF_00747] and [SWS_CANIF_00748]).]()

[SWS_CANIF_00749] [If CanIf_SetControllerMode(ControllerId, CAN_- CS_SLEEP) is called the PnTxFilter of the corresponding CAN Controller shall be enabled (ref. to [SWS_CANIF_00748] and [SWS_CANIF_00747]).]()

[SWS_CANIF_00750] [If the PnTxFilter of a CAN Controller is enabled, CanIf shall block all Tx requests to that CAN Controller (return E_NOT_OK when CanIf_- Transmit() is called), except if the requested Tx L-PDUs is one of the configured CanIfTxPduPnFilterPdus of that CAN Controller. These CanIfTxPduPnFil- terPdus shall always be passed to the corresponding CAN Driver.]()

[SWS_CANIF_00751] [If CanIf_TxConfirmation() is called, the corre- sponding PnTxFilter shall be disabled (ref. to [SWS_CANIF_00747] and [SWS_CANIF_00748]).]()

[SWS_CANIF_00896] [If CanIf_RxIndication() is called and PnTxFilter is en- abled, the corresponding PnTxFilter shall be disabled (ref. to [SWS_CANIF_00747] and [SWS_CANIF_00748]).]()

[SWS_CANIF_00752] [If the PnTxFilter of a CAN Controller is disabled, CanIf shall behave as requested via CanIf_SetPduMode() (see [SWS_CANIF_00008]).] ()

[SWS_CANIF_00878] [If CanIf_SetPduMode(ControllerId, CANIF_TX_OF- FLINE) is called and Partial Networking is enabled (ref. to CanIfPublicPnSup- port) the PnTxFilter of the corresponding CAN Controller shall be enabled (ref. to [SWS_CANIF_00748] and [SWS_CANIF_00747]).]()

#### CAN FD Support

For performance reasons some CAN Controllers allow to use a Flexible Data-Rate feature called CAN FD (see [12, ISO 11898-1:2015]). Besides, the higher baud rate for the payload CAN FD also supports an extended payload which allows the transmission of up to 64 bytes. If these features are available depends on the general CAN FD support by the CAN Controller and if the CAN Controller is in CAN FD mode (valid CanControllerFdBaudrateConfig).

If an L-SDU shall be sent as CAN FD or conventional CAN 2.0 frame depends on the configured CanIfTxPduCanIdType. CanIf indicates this to CanDrv utilizing the second most significant bit of PduInfo->id (Can_IdType) passed while calling Can_Write().

Note: If CanDrv is not in CAN FD mode (no CanControllerFdBaudrateConfig, the L-PDU will be sent as conventional CAN 2.0 frame as long as the SduLength <= 8 bytes.

Note: The arbitration phase of conventional CAN 2.0 frames and CAN FD frames does not differ if the same CanId is used. Therefore, even when using CAN FD frames each CanId must not be used more than once.

Which kind of frame was received by CanDrv is also indicated utilizing the second most significant bit of the Can_IdType passed with CanIf_RxIndication() (Mailbox-

->CanId). Based on this information CanIf decides how to map to the configured

L-SDU (CanIfRxPduCfg) as described in [SWS_CANIF_00877].

Note: If upper layers don’t care if a message was received by conventional CAN 2.0 frame or CAN FD frame, it is possible to use only one CanIfRxPduCfg for both types (see CanIfRxPduCanIdType). This might allow local optimization. However, from a system point of view, the format for each frame has to be configured. Otherwise the sender wouldn’t know which kind of frame shall be transmitted.

#### Security Events

[SWS_CANIF_91010] Security events for CanIf [

| Name | Description | ID |
| --- | --- | --- |
| CANIF_SEV_TX_ERROR_DETECTED | A transmission related error was detected. Depending on the context data this could indicate suspicious CAN activity. | 19 |
| CANIF_SEV_RX_ERROR_DETECTED | A reception related error was detected. Depending on the context data this could indicate suspicious CAN activity. | 20 |
| CANIF_SEV_ERRORSTATE_PASSIVE | The CAN controller transitioned to state passive. | 21 |
| CANIF_SEV_ERRORSTATE_BUSOFF | The CAN controller transitioned to state busoff. | 22 |

](RS_Ids_00810)

[SWS_CANIF_00913] [If security event reporting has been enabled for the CanIf module (CanIfEnableSecurityEventReporting = true) the respective security events shall bereported to the IdsM via the interfaces defined in AUTOSAR_SWS_- BSWGeneral.](RS_Ids_00810)

[SWS_CANIF_00915] [If CanIf_ErrorNotification() is called by CanDrv, the function shall evaluate whether a Tx related error was detected. If this is the case the CanIfshall report the security event CANIF_SEV_TX_ERROR_DETECTED.

The context data is structured as follows:

Context Data (2 Byte)

- ControllerID (1 Byte)

- CanError (1 Byte)

  - CAN_ERROR_BIT_MONITORING1 (0x1)

  - CAN_ERROR_BIT_MONITORING0 (0x2)

  - CAN_ERROR_BIT (0x3)

  - CAN_ERROR_CHECK_ACK_FAILED (0x4)

  - CAN_ERROR_ACK_DELIMITER (0x5)

  - CAN_ERROR_ARBITRATION_LOST (0x6)

  - CAN_ERROR_OVERLOAD (0x7)

](RS_Ids_00810)

[SWS_CANIF_00916] [If CanIf_ErrorNotification() is called by CanDrv, the function shall evaluate whether a Rx related error was detected. If this is the case the CanIf shall report the security event CANIF_SEV_RX_ERROR_DETECTED.

The context data is structured as follows: Context Data (2 Byte)

- ControllerID (1 Byte)

- CanError (1 Byte)

  - CAN_ERROR_CHECK_FORM_FAILED (0x8)

  - CAN_ERROR_CHECK_STUFFING_FAILED (0x9)

  - CAN_ERROR_CHECK_CRC_FAILED (0xA)

  - CAN_ERROR_BUS_LOCK (0xB)

](RS_Ids_00810)

[SWS_CANIF_00917] [If CanIf_ControllerErrorStatePassive() is called by CanDrv, the CanIf shall report the security event CANIF_SEV_ERRORSTATE_PAS- SIVE in following cases:

- TxErrorCounter > 127 and TxErrorCounter <= 255

- RxErrorCounter > 127 and TxErrorCounter <= 255 The context data is structured as follows:

Context Data (2 Byte)

- ControllerID (1 Byte)

- ErrorCounterThreshold (1 Byte)

  - TxErrorCounter > 127 AND RxErrorCounter > 127(0x0)

  - TxErrorCounter > 127 AND RxErrorCounter < 127 (0x1)

  - RxErrorCounter > 127 AND TxErrorCounter < 127 (0x2)

](RS_Ids_00810)

[SWS_CANIF_00918] [If CanIf_ControllerBusOff is called by CanDrv, the CanIf shall report the security event CANIF_SEV_ERRORSTATE_BUSOFF. The context data is structured as follows:

Context Data (1 Byte)

- Controller ID (1 Byte)

](RS_Ids_00810)

#### Error classification

##### Development Errors [SWS_CANIF_91006] [

| Type of error | Related error code | Error value |
| --- | --- | --- |
| API service called with invalid CAN ID | CANIF_E_PARAM_CANID | 10 |
| API service called with invalid hardware object | CANIF_E_PARAM_HOH | 12 |
| API service called with invalid PDU ID | CANIF_E_PARAM_LPDU | 13 |
| API service called with invalid controller ID | CANIF_E_PARAM_CONTROLLERID | 15 |
| API service called with invalid wakeup source | CANIF_E_PARAM_WAKEUPSOURCE | 16 |
| API service called with invalid transceiver ID | CANIF_E_PARAM_TRCV | 17 |
| API service called with invalid transceiver mode | CANIF_E_PARAM_TRCVMODE | 18 |
| API service called with invalid transceiver wakeup mode | CANIF_E_PARAM_TRCVWAKEUPMODE | 19 |
| API service called with invalid pointer | CANIF_E_PARAM_POINTER | 20 |
| API service called with invalid controller mode | CANIF_E_PARAM_CTRLMODE | 21 |
| API service called with invalid PDU mode | CANIF_E_PARAM_PDU_MODE | 22 |
| API services called with invalid parameter | CANIF_E_PARAM_CAN_ERROR | 23 |
| API service used without module initialization | CANIF_E_UNINIT | 30 |
| Transmit PDU ID invalid | CANIF_E_INVALID_TXPDUID | 50 |
| Receive PDU ID invalid | CANIF_E_INVALID_RXPDUID | 60 |
| CAN Interface initialisation failed | CANIF_E_INIT_FAILED | 80 |

##### Runtime Errors [SWS_CANIF_91007] [

| Type of error | Related error code | Error value |
| --- | --- | --- |
| Failed Data Length Check | CANIF_E_INVALID_DATA_LENGTH | 61 |
| Data Length | CANIF_E_DATA_LENGTH_MISMATCH | 62 |
| Transmit requested on offline PDU channel | CANIF_E_STOPPED | 70 |
| Message length was exceeding the maximum length | CANIF_E_TXPDU_LENGTH_EXCEEDED | 90 |

]()

##### Transient Faults

There are no transient faults.

##### Production Errors

There are no production errors.

##### Extended Production Errors

There are no extended production errors.

### API specification

#### Imported types

In this chapter all types included from the following modules are listed.

[SWS_CANIF_00142] [

| Module | Header File | Imported Type |
| --- | --- | --- |
| Can | Can_GeneralTypes.h | Can_ControllerStateType |
|  | Can_GeneralTypes.h | Can_ErrorStateType |
|  | Can_GeneralTypes.h | Can_ErrorType |
|  | Can_GeneralTypes.h | Can_HwHandleType |
|  | Can_GeneralTypes.h | Can_HwType |
|  | Can_GeneralTypes.h | Can_IdType |
|  | Can_GeneralTypes.h | Can_PduType |
|  | Can_GeneralTypes.h | Can_TimeStampType (draft) |
| CanTrcv | Can_GeneralTypes.h | CanTrcv_TrcvModeType |
|  | Can_GeneralTypes.h | CanTrcv_TrcvWakeupModeType |
|  | Can_GeneralTypes.h | CanTrcv_TrcvWakeupReasonType |
| ComStack_Types | ComStack_Types.h | PduIdType |
|  | ComStack_Types.h | PduInfoType |
|  | ComStack_Types.h | PduLengthType |
| EcuM | EcuM.h | EcuM_WakeupSourceType |
| IdsM | IdsM_Types.h | IdsM_SecurityEventIdType |
| Std | Std_Types.h | Std_ReturnType |
|  | Std_Types.h | Std_VersionInfoType |

](SRS_BSW_00348, SRS_BSW_00353, SRS_BSW_00361)

#### Type definitions

##### CanIf_ConfigType

[SWS_CANIF_00144] [

| Name | CanIf_ConfigType |  |
| --- | --- | --- |
| Kind | Structure |  |
| Elements | implementation specific |  |
|  | Type | – |
|  | Comment | The contents of the initialization data structure are CAN interface specific |

| Description | This type defines a data structure for the post build parameters of the CAN interface for all underlying CAN drivers. At initialization the CanIf gets a pointer to a structure of this type to get access to its configuration data, which is necessary for initialization. |
| --- | --- |
| Available via | CanIf.h |

[SWS_CANIF_00523] [The initialization data structure for a specific CanIf_Config- Type shall include the definition of CanIf public parameters and the definition for each L-PDU/L-SDU.]()

Note: The definition of CanIf public parameters and the definition for each L-PDU/ L-SDU are specified in chapter 10.

##### CanIf_PduModeType [SWS_CANIF_00137] [

| Name | CanIf_PduModeType |  |  |
| --- | --- | --- | --- |
| Kind | Enumeration |  |  |
| Range | CANIF_OFFLINE | 0x00 | = 0 Transmit and receive path of the corresponding channel are disabled => no communication mode |
|  | CANIF_TX_OFFLINE | 0x01 | Transmit path of the corresponding channel is disabled. The receive path is enabled. |
|  | CANIF_TX_OFFLINE_ ACTIVE | 0x02 | Transmit path of the corresponding channel is in offline active mode (see SWS_ CANIF_00072). The receive path is enabled. This mode requires CanIfTxOfflineActive Support = TRUE. |
|  | CANIF_ONLINE | 0x03 | Transmit and receive path of the corresponding channel are enabled => full operation mode |
| Description | The PduMode of a channel defines its transmit or receive activity. Communication direction (transmission and/or reception) of the channel can be controlled separately or together by upper layers. |  |  |
| Available via | CanIf.h |  |  |

| Name | CanIf_NotifStatusType |
| --- | --- |
| Kind | Enumeration |

CanIf_NotifStatusType [SWS_CANIF_00201] [

q

Δ

| Range | CANIF_TX_RX_ NOTIFICATION | – | The requested Rx/Tx CAN L-PDU was successfully transmitted or received. |
| --- | --- | --- | --- |
|  | CANIF_NO_NOTIFICATION | 0x00 | No transmit or receive event occurred for the requested L-PDU. |
| Description | Return value of CAN L-PDU notification status. |  |  |
| Available via | CanIf.h |  |  |

]()

#### Function definitions

[SWS_CANIF_00661] [All CanIf API services other than CanIf_Init() and CanIf_GetVersionInfo() shall not execute their normal operation and return E_- NOT_OK unless the CanIf has been initialized with a preceding call of CanIf_Init ().]()

##### CanIf_Init [SWS_CANIF_00001] [

| Service Name | CanIf_Init |  |
| --- | --- | --- |
| Syntax | void CanIf_Init ( const CanIf_ConfigType* ConfigPtr ) |  |
| Service ID [hex] | 0x01 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | ConfigPtr | Pointer to configuration parameter set, used e.g. for post build parameters |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service Initializes internal and external interfaces of the CAN Interface for the further processing. |  |
| Available via | CanIf.h |  |

](SRS_BSW_00405, SRS_BSW_00101, SRS_BSW_00358, SRS_BSW_00414, SRS_Can_01021, SRS_Can_01022)

Note: All underlying CAN controllers and transceivers still remain not operational. Note: The service CanIf_Init() is called only by the EcuM.

[SWS_CANIF_00085] [The service CanIf_Init() shall initialize the global variables and data structures of the CanIf including flags and buffers.]()

##### CanIf_DeInit [SWS_CANIF_91002] [

| Service Name | CanIf_DeInit |
| --- | --- |
| Syntax | void CanIf_DeInit ( void ) |
| Service ID [hex] | 0x02 |
| Sync/Async | Synchronous |
| Reentrancy | Non Reentrant |
| Parameters (in) | None |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | None |
| Description | De-initializes the CanIf module. |
| Available via | CanIf.h |

](SRS_Can_01168, SRS_BSW_00336)

Note: General behavior and constraints on de-initialization functions are specified by

[SWS_BSW_00152], [SWS_BSW_00072], [SWS_BSW_00232], [SWS_BSW_00233].

Caveat: Caller of the CanIf_DeInit() function has to be sure there are no on-going transmissions/receptions, nor any pending transmission confirmations.

##### CanIf_SetControllerMode

[SWS_CANIF_00003] [

| Service Name | CanIf_SetControllerMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_SetControllerMode ( uint8 ControllerId, Can_ControllerStateType ControllerMode ) |  |
| Service ID [hex] | 0x03 |  |
| Sync/Async | Asynchronous |  |
| Reentrancy | Reentrant (Not for the same controller) |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller, which is requested for mode transition. |
|  | ControllerMode | Requested mode transition |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Controller mode request has been accepted E_NOT_OK: Controller mode request has not been accepted |
| Description | This service calls the corresponding CAN Driver service for changing of the CAN controller mode. |  |
| Available via | CanIf.h |  |

](SRS_Can_01027)

Note: The service CanIf_SetControllerMode() initiates a transition to the re- quested CAN controller mode ControllerMode of the CAN controller which is as- signed by parameter ControllerId.

[SWS_CANIF_00308] [The service CanIf_SetControllerMode() shall call Can_SetControllerMode(Controller, Transition) for the requested CAN controller.]()

[SWS_CANIF_00311] [If parameter ControllerId of CanIf_SetController- Mode() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_CONTROLLERID to the Det_ReportError service of the DET module, when CanIf_SetControllerMode() is called.](SRS_BSW_00323)

[SWS_CANIF_00774] [If parameter ControllerMode of CanIf_SetController- Mode() has an invalid value (not CAN_CS_STARTED, CAN_CS_SLEEP or CAN_CS_- STOPPED), the CanIfshall report development error code CANIF_E_PARAM_CTRLMODE to the Det_ReportError service of the DET module, when CanIf_SetCon- trollerMode() is called.](SRS_BSW_00323)

Note: The ID of the CAN controller is published inside the configuration description of the CanIf.

##### CanIf_GetControllerMode [SWS_CANIF_00229] [

| Service Name | CanIf_GetControllerMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetControllerMode ( uint8 ControllerId, Can_ControllerStateType* ControllerModePtr ) |  |
| Service ID [hex] | 0x04 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller, which is requested for current operation mode. |
| Parameters (inout) | None |  |
| Parameters (out) | ControllerModePtr | Pointer to a memory location, where the current mode of the CAN controller will be stored. |
| Return value | Std_ReturnType | E_OK: Controller mode request has been accepted. E_NOT_OK: Controller mode request has not been accepted. |
| Description | This service calls the corresponding CAN Driver service for obtaining the current status of the CAN controller. |  |

](SRS_Can_01028)

[SWS_CANIF_00313] [If parameter ControllerId of CanIf_GetController- Mode() has an invalid, the CanIf shall report development error code CANIF_- E_PARAM_CONTROLLERID to the Det_ReportError service of the DET, when CanIf_GetControllerMode() is called.](SRS_BSW_00323)

[SWS_CANIF_00656] [If parameter ControllerModePtr of CanIf_GetCon- trollerMode() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_ReportError service of the DET, when CanIf_GetControllerMode() is called.](SRS_BSW_00323)

Note: The ID of the CAN controller module is published inside the configuration de- scription of the CanIf.

##### CanIf_GetControllerErrorState [SWS_CANIF_91001] [

| Service Name | CanIf_GetControllerErrorState |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetControllerErrorState ( uint8 ControllerId, Can_ErrorStateType* ErrorStatePtr ) |  |
| Service ID [hex] | 0x4b |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant for the same ControllerId |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller, which is requested for ErrorState. |
| Parameters (inout) | None |  |
| Parameters (out) | ErrorStatePtr | Pointer to a memory location, where the error state of the CAN controller will be stored. |
| Return value | Std_ReturnType | E_OK: Error state request has been accepted. E_NOT_OK: Error state request has not been accepted. |
| Description | This service calls the corresponding CAN Driver service for obtaining the error state of the CAN controller. |  |
| Available via | CanIf.h |  |

](SRS_Can_01169)

[SWS_CANIF_00898] [If parameter ControllerId of CanIf_GetCon- trollerErrorState() has an invalid value, the CanIf shall report develop- ment error code CANIF_E_PARAM_CONTROLLERID to the Det_ReportError service of the DET, when CanIf_GetControllerErrorState() is called.] (SRS_BSW_00323)

[SWS_CANIF_00899] [If parameter ErrorStatePtr of CanIf_GetCon- trollerErrorState() is a null pointer, the CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_ReportError service of the DET, when CanIf_GetControllerErrorState() is called.](SRS_BSW_00323)

##### CanIf_Transmit [SWS_CANIF_00005] [

| Service Name | CanIf_Transmit |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_Transmit ( PduIdType TxPduId, const PduInfoType* PduInfoPtr ) |  |
| Service ID [hex] | 0x49 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant for different PduIds. Non reentrant for the same PduId. |  |
| Parameters (in) | TxPduId | Identifier of the PDU to be transmitted |
|  | PduInfoPtr | Length of and pointer to the PDU data and pointer to MetaData. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Transmit request has been accepted. E_NOT_OK: Transmit request has not been accepted. |
| Description | Requests transmission of a PDU. |  |
| Available via | CanIf.h |  |

](SRS_Can_01008)

Note: The corresponding CAN Controller and HTH have to be resolved by the Tx- PduId.

[SWS_CANIF_00317] [The service CanIf_Transmit() shall not accept a transmit request, if the controller mode referenced by ControllerId is different to CAN_CS_- STARTED and the channel mode at least for the transmit path is not online or offline active.]()

[SWS_CANIF_00318] [CanIf_Transmit() shall call Can_Write() with the hard- ware transmit handle corresponding to the provided TxPduId and a Can_PduType structure where:

- swPduHandle is set to the CanTxPduId used in the corresponding CanIf_- TxConfirmation() call

- length is set to the value provided as PduInfoPtr->SduLength, possibly reduced according to [SWS_CANIF_00894]

- id is set to the CAN ID associated with the TxPduId

- sdu is set to the pointer provided as PduInfoPtr->SduDataPtr

Note: PduInfoPtr is a pointer to a L-SDU user memory, CAN Identifier, L-SDU han- dle and Data Length (see [1, Specification of CAN Driver]).

[SWS_CANIF_00243] [CanIf shall set the two most significant bits (’IDentifier Ex- tension flag’ (see [12, ISO 11898-1:2015]) and ’CAN FD flag’) of the CanId ( PduInfoPtr->id) before CanIf passes the predefined CanId to CanDrv at call of Can_Write() (see [1, Specification of CAN Driver], definition of Can_IdType [SWS_Can_00416]). The CanId format type of each CAN L-PDU can be configured by CanIfTxPduCanIdType, refer to CanIfTxPduCanIdType.](SRS_Can_01141)

[SWS_CANIF_00882] [CanIf_Transmit() shall accept a NULL pointer as PduIn- foPtr->SduDataPtr, if the PDU is configured for triggered transmission: CanIfTx- PduTriggerTransmit = TRUE.]()

[SWS_CANIF_00162] [If the call of Can_Write() returns E_OK the transmit request service CanIf_Transmit() shall return E_OK.]()

Note: If the call of Can_Write() returns E_NOT_OK, then the transmit request service CanIf_Transmit() shall return E_NOT_OK. If the transmit request service CanIf_- Transmit() returns E_NOT_OK, then the upper layer module is responsible to repeat the transmit request.

[SWS_CANIF_00319] [If parameter TxPduId of CanIf_Transmit() has an invalid value, CanIf shall report development error code CANIF_E_INVALID_TXPDUID to the Det_ReportError service of the DET, when CanIf_Transmit() is called.] (SRS_BSW_00323)

[SWS_CANIF_00320] [If parameter PduInfoPtr of CanIf_Transmit() has an in- valid value, CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_ReportError service of the DET module, when CanIf_Transmit() is called.](SRS_BSW_00323)

[SWS_CANIF_00893] [When CanIf_Transmit() is called with PduInfoPtr->Sd- uLength exceeding the maximum length of the PDU referenced by TxPduId:

- SduLength > 8 if the Can_IdType indicates a classic CAN frame

- SduLength > 64 if the Can_IdType indicates a CAN FD frame

CanIf shall report runtime error code CANIF_E_DATA_LENGTH_MISMATCH to the

Det_ReportRuntimeError() service of the DET.]()

Note: Besides static configured transmissions there are dynamic transmissions, too. Therefore, the valid data length is always passed by PduInfoPtr->SduLength. Furthermore, even the frame type might change via CanIf_SetDynamicTxId(). [SWS_CANIF_00893] ensures that not matching transmit requests can be detected via DET.

[SWS_CANIF_00894] [When CanIf_Transmit() is called with PduInfoPtr->Sd- uLength exceeding the length of the global PDU (see [ECUC_EcuC_00078]) refer- enced by TxPduId and CanIfTxPduTruncation is enabled, CanIf shall transmit data up to the length of the global PDU (see [ECUC_EcuC_00078]) and discard the rest.]()

[SWS_CANIF_00900] [When CanIf_Transmit() is called with PduInfoPtr->Sd- uLength exceeding the length of the global PDU (see [ECUC_EcuC_00078]) refer- enced by TxPduId and CanIfTxPduTruncation is disabled, CanIf shall report the runtime error CANIF_E_TXPDU_LENGTH_EXCEEDED and return E_NOT_OK with- out further actions.]()

Note: During the call of CanIf_Transmit() the buffer of PduInfoPtr is controlled by CanIf and this buffer should not be accessed for read/write from another call con- text. After return of this call the ownership changes to the upper layer.

##### CanIf_ReadRxPduData [SWS_CANIF_00194] [

| Service Name | CanIf_ReadRxPduData |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_ReadRxPduData ( PduIdType CanIfRxSduId, PduInfoType* CanIfRxInfoPtr ) |  |
| Service ID [hex] | 0x06 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | CanIfRxSduId | Receive L-SDU handle specifying the corresponding CAN L-SDU ID and implicitly the CAN Driver instance as well as the corresponding CAN controller device. |
| Parameters (inout) | None |  |
| Parameters (out) | CanIfRxInfoPtr | Contains the length (SduLength) of the received PDU, a pointer to a buffer (SduDataPtr) containing the PDU, and the MetaData related to this PDU. |
| Return value | Std_ReturnType | E_OK: Request for L-SDU data has been accepted E_NOT_OK: No valid data has been received |
| Description | This service provides the Data Length and the received data of the requested CanIfRxSduId to the calling upper layer. |  |
| Available via | CanIf.h |  |

](SRS_Can_01125, SRS_Can_01129)

[SWS_CANIF_00324] [The function CanIf_ReadRxPduData() shall not accept a request and return E_NOT_OK, if the corresponding controller mode refrenced by Con- trollerId is different to CAN_CS_STARTED and the channel mode is in the receive path online.]()

[SWS_CANIF_00325] [If parameter CanIfRxSduId of CanIf_ReadRxPduData() has an invalid value, e.g. not configured to be stored within CanIf via CanIfRxP- duReadData, CanIf shall report development error code CANIF_E_INVALID_RX- PDUID to the Det_ReportError service of the DET, when CanIf_ReadRxPdu- Data() is called.](SRS_BSW_00323)

[SWS_CANIF_00326] [If parameter CanIfRxInfoPtr of CanIf_ReadRxPduData () has an invalid value, CanIf shall report development error code CANIF_E_- PARAM_POINTER to the Det_ReportError service of the DET module, when CanIf_ReadRxPduData() is called.](SRS_BSW_00323)

[SWS_CANIF_00329] [CanIf_ReadRxPduData() shall not be used for CanIfRxS- duId, which are defined to receive multiple CAN-Ids (range reception).]()

Note: During the call of CanIf_ReadRxPduData() the buffer of CanIfRxInfoPtr is controlled by CanIf and this buffer should not be accessed for read/write from another call context. After return of this call the ownership changes to the upper layer.

[SWS_CANIF_00330] [Configuration of CanIf_ReadRxPduData(): This API can be enabled or disabled at pre-compile time configuration by the configuration parameter CanIfPublicReadRxPduDataApi.]()

##### CanIf_ReadTxNotifStatus [SWS_CANIF_00202] [

| Service Name | CanIf_ReadTxNotifStatus |  |
| --- | --- | --- |
| Syntax | CanIf_NotifStatusType CanIf_ReadTxNotifStatus ( PduIdType CanIfTxSduId ) |  |
| Service ID [hex] | 0x07 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | CanIfTxSduId | L-SDU handle to be transmitted. This handle specifies the corresponding CAN L-SDU ID and implicitly the CAN Driver instance as well as the corresponding CAN controller device. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | CanIf_NotifStatusType | Current confirmation status of the corresponding CAN Tx L-PDU. |
| Description | This service returns the confirmation status (confirmation occurred or not) of a specific static or dynamic CAN Tx L-PDU, requested by the CanIfTxSduId. |  |
| Available via | CanIf.h |  |

](SRS_Can_01130)

Note: This function notifies the upper layer about any transmit confirmation event to the corresponding requested L-SDU.

[SWS_CANIF_00393] [If configuration parameters CanIfPublicReadTxPduNoti- fyStatusApi and CanIfTxPduReadNotifyStatus for the transmitted L-SDU are set to TRUE, and if CanIf_ReadTxNotifStatus() is called, the CanIf shall reset the notification status for the transmitted L-SDU.]()

[SWS_CANIF_00331] [If parameter CanIfTxSduId of CanIf_ReadTxNotifSta- tus() is out of range or if no status information was configured for this CAN Tx L-SDU, CanIf shall report development error code CANIF_E_INVALID_TXPDUID to the Det_ReportError service of the DET when CanIf_ReadTxNotifStatus () is called.](SRS_BSW_00323)

[SWS_CANIF_00335] [Configuration of CanIf_ReadTxNotifyStatus(): This API can be enabled or disabled at pre-compile time configuration globally by the parameter CanIfPublicReadTxPduNotifyStatusApi.]()

##### CanIf_ReadRxNotifStatus [SWS_CANIF_00230] [

| Service Name | CanIf_ReadRxNotifStatus |  |
| --- | --- | --- |
| Syntax | CanIf_NotifStatusType CanIf_ReadRxNotifStatus ( PduIdType CanIfRxSduId ) |  |
| Service ID [hex] | 0x08 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | CanIfRxSduId | Receive L-SDU handle specifying the corresponding CAN L-SDU ID and implicitly the CAN Driver instance as well as the corresponding CAN controller device. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | CanIf_NotifStatusType | Current indication status of the corresponding CAN Rx L-PDU. |
| Description | This service returns the indication status (indication occurred or not) of a specific CAN Rx L-PDU, requested by the CanIfRxSduId. |  |
| Available via | CanIf.h |  |

](SRS_Can_01130, SRS_Can_01131)

Note: This function notifies the upper layer about any receive indication event to the corresponding requested L-SDU.

[SWS_CANIF_00394] [If configuration parameters CanIfPublicReadRxPduNoti- fyStatusApi and CanIfRxPduReadNotifyStatus are set to TRUE, and if CanIf_ReadRxNotifStatus() is called, then CanIf shall reset the notification sta- tus for the received L-SDU.]()

[SWS_CANIF_00336] [If parameter CanIfRxSduId of CanIf_ReadRxNotifSta- tus() is out of range or if status for CanRxPduId was requested whereas CanIfRx- PduReadData is disabled or if no status information was configured for this CAN Rx

L-SDU, CanIf shall report development error code CANIF_E_INVALID_RXPDUID to the Det_ReportError service of the DET, when CanIf_ReadRxNotifStatus () is called.](SRS_BSW_00323)

Note: The function CanIf_ReadRxNotifStatus() must not be used for CanI- fRxSduIds, which are defined to receive multiple CAN-Ids (range reception).

[SWS_CANIF_00340] [Configuration of CanIf_ReadRxNotifStatus(): This API can be enabled or disabled at pre-compile time configuration globally by the parameter CanIfPublicReadRxPduNotifyStatusApi.]()

##### CanIf_SetPduMode [SWS_CANIF_00008] [

| Service Name | CanIf_SetPduMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_SetPduMode ( uint8 ControllerId, CanIf_PduModeType PduModeRequest ) |  |
| Service ID [hex] | 0x09 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | ControllerId | All PDUs of the own ECU connected to the corresponding CanIf ControllerId, which is assigned to a physical CAN controller are addressed. |
|  | PduModeRequest | Requested PDU mode change |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Request for mode transition has been accepted. E_NOT_OK: Request for mode transition has not been accepted. |
| Description | This service sets the requested mode at the L-PDUs of a predefined logical PDU channel. |  |
| Available via | CanIf.h |  |

]()

Note: The channel parameter denoting the predefined logical PDU channel can be derived from parameter ControllerId of function CanIf_SetPduMode().

[SWS_CANIF_00341] [If CanIf_SetPduMode() is called with invalid Control- lerId, CanIf shall report development error code CANIF_E_PARAM_CONTROL- LERID to the Det_ReportError service of the DET module.](SRS_BSW_00323)

[SWS_CANIF_00860] [If CanIf_SetPduMode() is called with invalid PduMod- eRequest, CanIf shall report development error code CANIF_E_PARAM_PDU_MODE to the Det_ReportError service of the DET module.](SRS_BSW_00323)

[SWS_CANIF_00874] [The service CanIf_SetPduMode() shall not accept any re- quest and shall return E_NOT_OK, if the controller mode referenced by ControllerId is not in state CAN_CS_STARTED.]()

##### CanIf_GetPduMode [SWS_CANIF_00009] [

| Service Name | CanIf_GetPduMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetPduMode ( uint8 ControllerId, CanIf_PduModeType* PduModePtr ) |  |
| Service ID [hex] | 0x0a |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant (Not for the same channel) |  |
| Parameters (in) | ControllerId | All PDUs of the own ECU connected to the corresponding CanIf ControllerId, which is assigned to a physical CAN controller are addressed. |
| Parameters (inout) | None |  |
| Parameters (out) | PduModePtr | Pointer to a memory location, where the current mode of the logical PDU channel will be stored. |
| Return value | Std_ReturnType | E_OK: PDU mode request has been accepted E_NOT_OK: PDU mode request has not been accepted |
| Description | This service reports the current mode of a requested PDU channel. |  |
| Available via | CanIf.h |  |

[SWS_CANIF_00346] [If CanIf_GetPduMode() is called with invalid Control- lerId, CanIf shall report development error code CANIF_E_PARAM_CONTROL- LERID to the Det_ReportError service of the DET module.](SRS_BSW_00323)

[SWS_CANIF_00657] [If CanIf_GetPduMode() is called with invalid PduModePtr, CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_- ReportError service of the DET module.](SRS_BSW_00323)

| Service Name | CanIf_GetVersionInfo |
| --- | --- |
| Syntax | void CanIf_GetVersionInfo ( Std_VersionInfoType* VersionInfo ) |

CanIf_GetVersionInfo [SWS_CANIF_00158] [

q

Δ

| Service ID [hex] | 0x0b |  |
| --- | --- | --- |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | None |  |
| Parameters (inout) | None |  |
| Parameters (out) | VersionInfo | Pointer to where to store the version information of this module. |
| Return value | None |  |
| Description | This service returns the version information of the called CAN Interface module. |  |
| Available via | CanIf.h |  |

](SRS_BSW_00407, SRS_BSW_00411)

##### CanIf_SetDynamicTxId [SWS_CANIF_00189] [

| Service Name | CanIf_SetDynamicTxId |  |
| --- | --- | --- |
| Syntax | void CanIf_SetDynamicTxId ( PduIdType CanIfTxSduId, Can_IdType CanId ) |  |
| Service ID [hex] | 0x0c |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | CanIfTxSduId | L-SDU handle to be transmitted. This handle specifies the corresponding CAN L-SDU ID and implicitly the CAN Driver instance as well as the corresponding CAN controller device. |
|  | CanId | Standard/Extended CAN ID of CAN L-SDU that shall be transmitted as FD or conventional CAN frame. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service reconfigures the corresponding CAN identifier of the requested CAN L-PDU. |  |
| Available via | CanIf.h |  |

]()

Note: CanIf_SetDynamicTxId() may be interrupted by CanIf_Transmit() called by several modules in the communication stack. Therefore precautions for preventing inconsistency need to be considered.

[SWS_CANIF_00352] [If parameter CanIfTxSduId of CanIf_SetDynamicTxId () has an invalid value, CanIf shall report development error code CANIF_E_IN- VALID_TXPDUID to the Det_ReportError service of the DET module, when CanIf_SetDynamicTxId() is called.](SRS_BSW_00323)

[SWS_CANIF_00353] [If parameter CanId of CanIf_SetDynamicTxId() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_CANID to the Det_ReportError service of the DET module, when CanIf_SetDynamic- TxId() is called.](SRS_BSW_00323)

[SWS_CANIF_00355] [If CanIf was not initialized before calling CanIf_SetDynam- icTxId(), then the function CanIf_SetDynamicTxId() shall not execute a recon- figuration of Tx CanId.]()

[SWS_CANIF_00357] [Configuration of CanIf_SetDynamicTxId(): This function shall be pre compile time configurable On/Off by the configuration parameter CanIf- PublicSetDynamicTxIdApi.]()

##### CanIf_SetTrcvMode [SWS_CANIF_00287] [

| Service Name | CanIf_SetTrcvMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_SetTrcvMode ( uint8 TransceiverId, CanTrcv_TrcvModeType TransceiverMode ) |  |
| Service ID [hex] | 0x0d |  |
| Sync/Async | Asynchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId, which is assigned to a CAN transceiver, which is requested for mode transition |
|  | TransceiverMode | Requested mode transition |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Transceiver mode request has been accepted. E_NOT_OK: Transceiver mode request has not been accepted. |
| Description | This service changes the operation mode of the tansceiver TransceiverId, via calling the corresponding CAN Transceiver Driver service. |  |
| Available via | CanIf.h |  |

]()

Note: For more details, please refer to the [2, Specification of CAN Transceiver Driver].

[SWS_CANIF_00358] [The function CanIf_SetTrcvMode() shall call the function CanTrcv_SetOpMode(Transceiver, OpMode) on the corresponding requested CAN Transceiver Driver module.]()

Note: The parameters of the service CanTrcv_SetOpMode() are of type:

- OpMode: CanTrcv_TrcvModeType(desired operation mode)

- Transceiver: uint8 (Transceiver to which function call has to be applied)

(see [2, Specification of CAN Transceiver Driver])

[SWS_CANIF_00538] [If parameter TransceiverId of CanIf_SetTrcvMode() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_- TRCV to the Det_ReportError service of the DET, when CanIf_SetTrcvMode () is called.](SRS_BSW_00323)

Note: The mode of a transceiver can only be changed to CANTRCV_TRCVMODE_- STANDBY, when the former mode of the transceiver has been CANTRCV_TRCVMODE_- NORMAL (see [2]). But this is not checked by the CanIf.

Note: The mode of a transceiver can only be changed to CANTRCV_TRCVMODE_- SLEEP, when the former mode of the transceiver has been CANTRCV_TRCVMODE_- STANDBY (see [2]). But this is not checked by the CanIf.

[SWS_CANIF_00648] [If parameter TransceiverMode of CanIf_SetTrcvMode() has an invalid value (not CANTRCV_TRCVMODE_STANDBY, CANTRCV_TRCVMODE_- SLEEP or CANTRCV_TRCVMODE_NORMAL), the CanIf shall report development error code CANIF_E_PARAM_TRCVMODE to the Det_ReportError service of the DET module, when CanIf_SetTrcvMode() is called.](SRS_BSW_00323)

Note: The function CanIf_SetTrcvMode() should be applicable to all CAN transceivers with all values of TransceiverMode independent, if the transceiver hard- ware supports these modes or not. This is to ease up the view of the CanIf to the assigned physical CAN channel.

[SWS_CANIF_00362] [Configuration of CanIf_SetTrcvMode(): The number of supported transceiver types for each network is set up in the configuration phase (see CanIfTrcvCfg and CanIfTrcvDrvCfg). If no transceiver is used, this function may be omitted. Therefore, if no transceiver is configured in LT or PB class the API shall return with E_NOT_OK.]()

| Service Name | CanIf_GetTrcvMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetTrcvMode ( uint8 TransceiverId, CanTrcv_TrcvModeType* TransceiverModePtr ) |  |
| Service ID [hex] | 0x0e |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId, which is assigned to a CAN transceiver, which is requested for current operation mode. |
| Parameters (inout) | None |  |

CanIf_GetTrcvMode [SWS_CANIF_00288] [

q

Δ

| Parameters (out) | TransceiverModePtr | Requested mode of requested network the Transceiver is connected to. |
| --- | --- | --- |
| Return value | Std_ReturnType | E_OK: Transceiver mode request has been accepted. E_NOT_OK: Transceiver mode request has not been accepted. |
| Description | This function invokes CanTrcv_GetOpMode and updates the parameter TransceiverModePtr with the value OpMode provided by CanTrcv. |  |
| Available via | CanIf.h |  |

]()

Note: For more details, please refer to the [2, Specification of CAN Transceiver Driver].

[SWS_CANIF_00363] [The function CanIf_GetTrcvMode() shall call the function CanTrcv_GetOpMode(Transceiver, OpMode) on the corresponding requested CAN Transceiver Driver module.]()

Note: The parameters of the function CanTrcv_GetOpMode are of type:

- OpMode: CanTrcv_TrcvModeType (desired operation mode)

- Transceiver: uint8 (Transceiver to which API call has to be applied) (see [2, Specification of CAN Transceiver Driver])

[SWS_CANIF_00364] [If parameter TransceiverId of CanIf_GetTrcvMode() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_- TRCV to the Det_ReportError service of the DET module, when CanIf_GetTr- cvMode() is called.](SRS_BSW_00323)

[SWS_CANIF_00650] [If parameter TransceiverModePtr of CanIf_GetTrcv- Mode() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_POINTER to the Det_ReportError service of the DET module, when CanIf_GetTrcvMode() was called.](SRS_BSW_00323)

[SWS_CANIF_00367] [Configuration of CanIf_GetTrcvMode(): The number of supported transceiver types for each network is set up in the configuration phase (see CanIfTrcvCfg and CanIfTrcvDrvCfg). If no transceiver is used, this function may be omitted. Therefore, if no transceiver is configured in LT or PB class the API shall return with E_NOT_OK.]()

| Service Name | CanIf_GetTrcvWakeupReason |
| --- | --- |

CanIf_GetTrcvWakeupReason [SWS_CANIF_00289] [

q

Δ

| Syntax | Std_ReturnType CanIf_GetTrcvWakeupReason ( uint8 TransceiverId, CanTrcv_TrcvWakeupReasonType* TrcvWuReasonPtr ) |  |
| --- | --- | --- |
| Service ID [hex] | 0x0f |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId, which is assigned to a CAN transceiver, which is requested for wake up reason. |
| Parameters (inout) | None |  |
| Parameters (out) | TrcvWuReasonPtr | provided pointer to where the requested transceiver wake up reason shall be returned |
| Return value | Std_ReturnType | E_OK: Transceiver wake up reason request has been accepted. E_NOT_OK: Transceiver wake up reason request has not been accepted. |
| Description | This service returns the reason for the wake up of the transceiver TransceiverId, via calling the corresponding CAN Transceiver Driver service. |  |
| Available via | CanIf.h |  |

]()

Note: The ability to detect and differentiate the possible wake up reasons depends strongly on the CAN transceiver hardware. For more details, please refer to the [2, Specification of CAN Transceiver Driver].

[SWS_CANIF_00368] [The function CanIf_GetTrcvWakeupReason() shall call CanTrcv_GetBusWuReason(Transceiver, Reason) on the corresponding re- quested CanTrcv.]()

Note: The parameters of the function CanTrcv_GetBusWuReason() are of type:

- Reason: CanTrcv_TrcvWakeupReasonType

- Transceiver: uint8 (Transceiver to which API call has to be applied) (see [2, Specification of CAN Transceiver Driver])

[SWS_CANIF_00537] [If parameter TransceiverId of CanIf_GetTrcvWake- upReason() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_GetTrcvWakeupReason() is called.](SRS_BSW_00323)

[SWS_CANIF_00649] [If parameter TrcvWuReasonPtr of CanIf_GetTrcvWake- upReason() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_ReportError service of the DET mod- ule, when CanIf_GetTrcvWakeupReason() is called.](SRS_BSW_00323)

Note: Please be aware, that if more than one network is available, each network may report a different wake-up reason. E.g. if an ECU uses CAN, a wake-up by CAN may occur and the incoming data may cause an internal wake-up for another CAN network.

The service CanIf_GetTrcvWakeupReason() has a "per network" view and does not vote the more important reason or sequence internally. The same may be true if

e.g. one transceiver controls the power supply and the other is just powered or un- powered. Then one may be able to return CANIF_TRCV_WU_POWER_ON, whereas the other may state e.g. CANIF_TRCV_WU_RESET. It is up to the calling module to decide, how to handle the wake-up information.

[SWS_CANIF_00371] [Configuration of CanIf_GetTrcvWakeupReason(): The number of supported transceiver types for each network is set up in the configura- tion phase (see CanIfTrcvCfg and CanIfTrcvDrvCfg). If no transceiver is used, this function may be omitted. Therefore, if no transceiver is configured in LT or PB class the API shall return with E_NOT_OK.]()

##### CanIf_SetTrcvWakeupMode [SWS_CANIF_00290] [

| Service Name | CanIf_SetTrcvWakeupMode |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_SetTrcvWakeupMode ( uint8 TransceiverId, CanTrcv_TrcvWakeupModeType TrcvWakeupMode ) |  |
| Service ID [hex] | 0x10 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId, which is assigned to a CAN transceiver, which is requested for wake up notification mode transition. |
|  | TrcvWakeupMode | Requested transceiver wake up notification mode |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Will be returned, if the wake up notifications state has been changed to the requested mode. E_NOT_OK: Will be returned, if the wake up notifications state change has failed or the parameter is out of the allowed range. The previous state has not been changed. |
| Description | This function shall call CanTrcv_SetTrcvWakeupMode. |  |
| Available via | CanIf.h |  |

Note: For more details, please refer to [2, Specification of CAN Transceiver Driver].

[SWS_CANIF_00372] [The function CanIf_SetTrcvWakeupMode() shall call CanTrcv_SetWakeupMode(Transceiver, TrcvWakeupMode) on the corre- sponding requested CanTrcv.]()

Info: The parameters of the function CanTrcv_SetWakeupMode() are of type:

- TrcvWakeupMode: CanTrcv_TrcvWakeupModeType (see [2, Specification of CAN Transceiver Driver])

- Transceiver: uint8 (Transceiver to which API call has to be applied) (see [2, Specification of CAN Transceiver Driver])

Note: The following three paragraphs are already described in the Specification of CanTrcv (see [2]). They describe the behavior of a CanTrcv in the respective transceiver wake-up mode, which is requested in parameter TrcvWakeupMode.

CANIF_TRCV_WU_ENABLE:

If the CanTrcv has a stored wake-up event pending for the addressed CanNetwork, the notification is executed within or immediately after the function CanTrcv_SetTr- cvWakeupMode() (depending on the implementation).

CANIF_TRCV_WU_DISABLE:

No notifications for wake-up events for the addressed CanNetwork are passed through the CanTrcv. The transceiver device and the underlying communication driver has to buffer detected wake-up events and raise the event(s), when the wake-up noti- fication is enabled again.

CANIF_TRCV_WU_CLEAR:

If notification of wake-up events is disabled (see description of mode CANIF_- TRCV_WU_DISABLE), detected wake-up events are buffered. Calling CanIf_Set- TrcvWakeupMode() with parameter CANIF_TRCV_WU_CLEAR clears these bufferd events. Clearing of wake-up events has to be used, when the wake-up notification is disabled to clear all stored wake-up events under control of the higher layers of the CanTrcv.

[SWS_CANIF_00535] [If parameter TransceiverId of CanIf_SetTrcvWakeup- Mode() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_SetTrcvWakeupMode() is called.](SRS_BSW_00323)

[SWS_CANIF_00536] [If parameter TrcvWakeupMode of CanIf_SetTrcvWakeup- Mode() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_TRCVWAKEUPMODE to the Det_ReportError service of the DET mod- ule, when CanIf_SetTrcvWakeupMode() is called.](SRS_BSW_00323)

[SWS_CANIF_00373] [Configuration of CanIf_SetTrcvWakeupMode(): The num- ber of supported transceiver types for each network is set up in the configuration phase (see CanIfTrcvCfg and CanIfTrcvDrvCfg). If no transceiver is used, this function may be omitted. Therefore, if no transceiver is configured in LT or PB class the API shall return with E_NOT_OK.]()

##### CanIf_CheckWakeup [SWS_CANIF_00219] [

| Service Name | CanIf_CheckWakeup |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_CheckWakeup ( EcuM_WakeupSourceType WakeupSource ) |  |
| Service ID [hex] | 0x11 |  |
| Sync/Async | Asynchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | WakeupSource | Source device, which initiated the wake up event: CAN controller or CAN transceiver |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Will be returned, if the check wake up request has been accepted E_NOT_OK: Will be returned, if the check wake up request has not been accepted |
| Description | This service checks, whether an underlying CAN driver or a CAN transceiver driver already signals a wakeup event. |  |
| Available via | CanIf.h |  |

]()

Note: Integration Code calls this function

[SWS_CANIF_00398] [If parameter WakeupSource of CanIf_CheckWakeup() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_WAKE- UPSOURCE to the Det_ReportError service of the DET, when CanIf_Check- Wakeup() is called.](SRS_BSW_00323)

Note: The call context of CanIf_CheckWakeup() is either on interrupt level (interrupt mode) or on task level (polling mode).

##### CanIf_CheckValidation [SWS_CANIF_00178] [

| Service Name | CanIf_CheckValidation |  |  |
| --- | --- | --- | --- |
| Syntax | Std_ReturnType CanIf_CheckValidation ( EcuM_WakeupSourceType WakeupSource ) |  |  |
| Service ID [hex] | 0x12 |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Reentrant |  |  |
| Parameters (in) | WakeupSource | Source device which initiated the wake-up event and which has to be validated: CAN controller or CAN transceiver |  |
| Parameters (inout) | None |  |  |
| Parameters (out) | None |  |  |
| Return value | Std_ReturnType |  | E_OK: Will be returned, if the check validation request has been accepted. E_NOT_OK: Will be returned, if the check validation request has not been accepted. |
| Description | This service is performed to validate a previous wakeup event. |  |  |
| Available via | CanIf.h |  |  |

Note: Integration Code calls this function

[SWS_CANIF_00404] [If parameter WakeupSource of CanIf_CheckValidation () has an invalid value, the CanIf shall report development error code CANIF_E_- PARAM_WAKEUPSOURCE to the Det_ReportError service of the DET module, when CanIf_CheckValidation() is called.](SRS_BSW_00323)

Note: The call context of CanIf_CheckValidation() is either on interrupt level (interrupt mode) or on task level (polling mode).

Caveat:  The corresponding CAN controller and transceiver must be switched on via CanTrcv_SetOpMode(Transceiver, CANTRCV_TRCVMODE_NORMAL) and Can_SetControllerMode(Controller, CAN_CS_STARTED) and the corre- sponding mode indications must have been called.

[SWS_CANIF_00408] [Configuration of CanIf_CheckValidation(): If no valida- tion is needed, this API can be omitted by disabling of CanIfPublicWakeupCheck- ValidSupport.]()

##### CanIf_GetTxConfirmationState [SWS_CANIF_00734] [

| Service Name | CanIf_GetTxConfirmationState |  |
| --- | --- | --- |
| Syntax | CanIf_NotifStatusType CanIf_GetTxConfirmationState ( uint8 ControllerId ) |  |
| Service ID [hex] | 0x19 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant (Not for the same controller) |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | CanIf_NotifStatusType | Combined TX confirmation status for all TX PDUs of the CAN controller |
| Description | This service reports, if any TX confirmation has been done for the whole CAN controller since the last CAN controller start. |  |
| Available via | CanIf.h |  |

SWS_CANIF_00736] [If parameter ControllerId of CanIf_GetTxConfirma- tionState() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_CONTROLLERID to the Det_ReportError service of the DET module, when CanIf_GetTxConfirmationState() is called.]()

Note: The call context of CanIf_GetTxConfirmationState() is on task level (polling mode).

[SWS_CANIF_00738] [Configuration of CanIf_GetTxConfirmationState(): If BusOff Recovery of CanSm doesn’t need the status of the Tx confirmations (see [SWS_CANIF_00740]), this API can be omitted by disabling of CanIfPublic- TxConfirmPollingSupport.]()

##### CanIf_ClearTrcvWufFlag [SWS_CANIF_00760] [

| Service Name | CanIf_ClearTrcvWufFlag |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_ClearTrcvWufFlag ( uint8 TransceiverId ) |  |
| Service ID [hex] | 0x1e |  |
| Sync/Async | Asynchronous |  |
| Reentrancy | Reentrant for different CAN transceivers |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to the designated CAN transceiver. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Request has been accepted E_NOT_OK: Request has not been accepted |
| Description | Requests the CanIf module to clear the WUF flag of the designated CAN transceiver. |  |
| Available via | CanIf.h |  |

[SWS_CANIF_00766]  [Within  CanIf_ClearTrcvWufFlag()  the  function CanTrcv_ClearTrcvWufFlag() shall be called.]()

[SWS_CANIF_00769] [If parameter TransceiverId of CanIf_ClearTrcvWuf- Flag() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_ClearTrcvWufFlag() is caled.]()

[SWS_CANIF_00771] [Configuration of CanIf_ClearTrcvWufFlag(): Whether the CanIf supports this function shall be pre compile time configurable On/Off by the configuration parameter CanIfPublicPnSupport.]()

##### CanIf_CheckTrcvWakeFlag [SWS_CANIF_00761] [

| Service Name | CanIf_CheckTrcvWakeFlag |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_CheckTrcvWakeFlag ( uint8 TransceiverId ) |  |
| Service ID [hex] | 0x1f |  |
| Sync/Async | Asynchronous |  |
| Reentrancy | Reentrant for different CAN transceivers |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to the designated CAN transceiver. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Request has been accepted E_NOT_OK: Request has not been accepted |
| Description | Requests the CanIf module to check the Wake flag of the designated CAN transceiver. |  |
| Available via | CanIf.h |  |

[SWS_CANIF_00765]	[Within	CanIf_CheckTrcvWakeFlag() the function CanTrcv_CheckWakeFlag() shall be called.]()

[SWS_CANIF_00770] [If parameter TransceiverId of CanIf_CheckTrcvWake- Flag() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_CheckTrcvWakeFlag() is caled.]()

[SWS_CANIF_00813] [Configuration of CanIf_CheckTrcvWakeFlag(): Whether the CanIf supports this function shall be pre compile time configurable On/Off by the configuration parameter CanIfPublicPnSupport.]()

##### CanIf_SetBaudrate [SWS_CANIF_00867] [

| Service Name | CanIf_SetBaudrate |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_SetBaudrate ( uint8 ControllerId, uint16 BaudRateConfigID ) |  |
| Service ID [hex] | 0x27 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant for different ControllerIds. Non reentrant for the same ControllerId. |  |
| Parameters (in) | ControllerId | Abstract CanIf ControllerId which is assigned to a CAN controller, whose baud rate shall be set. |
|  | BaudRateConfigID | references a baud rate configuration by ID (see CanController BaudRateConfigID) |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Service request accepted, setting of (new) baud rate started E_NOT_OK: Service request not accepted |
| Description | This service shall set the baud rate configuration of the CAN controller. Depending on necessary baud rate modifications the controller might have to reset. |  |
| Available via | CanIf.h |  |

]()

[SWS_CANIF_00868] [The service CanIf_SetBaudrate() shall call Can_- SetBaudrate(Controller, BaudRateConfigID) for the requested CAN Con- troller.]()

[SWS_CANIF_00869] [If CanIf_SetBaudrate() is called with invalid Control- lerId, CanIf shall report development error code CANIF_E_PARAM_CONTROL- LERID to the Det_ReportError service of the DET module.](SRS_BSW_00323)

Note: The parameter BaudRateConfigID of CanIf_SetBaudrate() is not checked by CanIf. This has to be done by responsible CanDrv.

Note: The call context of CanIf_SetBaudrate() is on task level (polling mode).

[SWS_CANIF_00871] [If CanIf supports changing baud rate and thus CanIf_Set- Baudrate(), shall be configurable via CanIfSetBaudrateApi.]()

##### CanIf_GetControllerRxErrorCounter [SWS_CANIF_91003] [

| Service Name | CanIf_GetControllerRxErrorCounter |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetControllerRxErrorCounter ( uint8 ControllerId, uint8* RxErrorCounterPtr ) |  |
| Service ID [hex] | 0x4d |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant for the same ControllerId |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller. |
| Parameters (inout) | None |  |
| Parameters (out) | RxErrorCounterPtr | Pointer to a memory location, where the current Rx error counter of the CAN controller will be stored. |
| Return value | Std_ReturnType | E_OK: Rx error counter available. E_NOT_OK: Wrong ControllerId, or Rx error counter not available. |
| Description | This service calls the corresponding CAN Driver service for obtaining the Rx error counter of the CAN controller. |  |
| Available via | CanIf.h |  |

[SWS_CANIF_00907] [If parameter ControllerId of CanIf_GetControllerRx- ErrorCounter() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_CONTROLLERID to the Det_ReportError service of the DET, when CanIf_GetControllerRxErrorCounter() is called.](SRS_BSW_00323)

[SWS_CANIF_00908] [If parameter RxErrorCounterPtr of CanIf_GetCon- trollerRxErrorCounter() is a null pointer, the CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_ReportError service of the DET, when CanIf_GetControllerRxErrorCounter() is called.](SRS_BSW_- 00323)

##### CanIf_GetControllerTxErrorCounter [SWS_CANIF_91004] [

| Service Name | CanIf_GetControllerTxErrorCounter |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetControllerTxErrorCounter ( uint8 ControllerId, uint8* TxErrorCounterPtr ) |  |
| Service ID [hex] | 0x4e |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant for the same ControllerId |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller. |
| Parameters (inout) | None |  |
| Parameters (out) | TxErrorCounterPtr | Pointer to a memory location, where the current Tx error counter of the CAN controller will be stored. |
| Return value | Std_ReturnType | E_OK: Tx error counter available. E_NOT_OK: Wrong ControllerId, or Tx error counter not available. |
| Description | This service calls the corresponding CAN Driver service for obtaining the Tx error counter of the CAN controller. |  |
| Available via | CanIf.h |  |

[SWS_CANIF_00909] [If parameter ControllerId of CanIf_GetControllerTx- ErrorCounter() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_CONTROLLERID to the Det_ReportError service of the DET, when CanIf_GetControllerTxErrorCounter() is called.](SRS_BSW_00323)

[SWS_CANIF_00910] [If parameter TxErrorCounterPtr of CanIf_GetCon- trollerTxErrorCounter() is a null pointer, the CanIf shall report development error code CANIF_E_PARAM_POINTER to the Det_ReportError service of the DET, when CanIf_GetControllerTxErrorCounter() is called.](SRS_BSW_- 00323)

##### CanIf_EnableBusMirroring [SWS_CANIF_91005] [

| Service Name | CanIf_EnableBusMirroring |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_EnableBusMirroring ( uint8 ControllerId, boolean MirroringActive ) |  |
| Service ID [hex] | 0x4c |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller. |
|  | MirroringActive | TRUE: Mirror_ReportCanFrame will be called for each frame received or transmitted on the given controller. FALSE: Mirror_ ReportCanFrame will not be called for the given controller. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: Mirroring mode was changed. E_NOT_OK: Wrong ControllerId, or mirroring globally disabled (see CanIfBusMirroringSupport). |
| Description | Enables or disables mirroring for a CAN controller. |  |
| Available via | CanIf.h |  |

[SWS_CANIF_00911] [If Bus Mirroring is not enabled (see CanIfBusMirroring- Support), the API CanIf_EnableBusMirroring() can be omitted.](SRS_Can_- 01172)

[SWS_CANIF_00912] [If parameter ControllerId of CanIf_EnableBusMirror- ing() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_CONTROLLERID to the Det_ReportError service of the DET, when CanIf_EnableBusMirroring() is called.](SRS_BSW_00323)

##### CanIf_GetCurrentTime [SWS_CANIF_91014]{DRAFT} [

| Service Name | CanIf_GetCurrentTime (draft) |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetCurrentTime ( uint8 Controller, Can_TimeStampType* timeStampPtr ) |  |
| Service ID [hex] | 0x51 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | Controller | Index of the addresses CAN controller. |
| Parameters (inout) | None |  |
| Parameters (out) | timeStampPtr | current time stamp |
| Return value | Std_ReturnType | E_OK: successful E_NOT_OK: failed |
| Description | This service calls the corresponding CAN Driver service to retrieve the current time value out of the HW registers. Tags: atp.Status=draft |  |
| Available via | CanIf.h |  |

](SRS_Can_01181)

[SWS_CANIF_00922]{DRAFT} [If development error detection is enabled: the func- tion shall check that the service CanIf_Init() was previously called. If the check fails, the function shall raise the development error CANIF_E_UNINIT]()

[SWS_CANIF_00923]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter Controller for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_CONTROLLERID.]()

[SWS_CANIF_00924]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter timeStampPtr for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_POINTER.]()

[SWS_CANIF_00925]{DRAFT} [The function shall be pre compile time configurable On/Off by the configuration parameter: CanIfGlobalTimeSupport]()

##### CanIf_EnableEgressTimeStamp [SWS_CANIF_91011]{DRAFT} [

| Service Name | CanIf_EnableEgressTimeStamp (draft) |  |
| --- | --- | --- |
| Syntax | void CanIf_EnableEgressTimeStamp ( PduIdType TxPduId ) |  |
| Service ID [hex] | 0x52 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TxPduId | L-PDU handle of CAN L-PDU for which the time stamping shall be enabled. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service calls the corresponding CAN Driver service to activate egress time stamping on a dedicated message object. Tags: atp.Status=draft |  |
| Available via | CanIf.h |  |

](SRS_Can_01181)

[SWS_CANIF_00926]{DRAFT} [If development error detection is enabled: the func- tion shall check that the service CanIf_Init() was previously called. If the check fails, the function shall raise the development error CANIF_E_UNINIT]()

[SWS_CANIF_00927]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter TxPduId for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_LPDU.]()

[SWS_CANIF_00928]{DRAFT} [The function shall be pre compile time configurable On/Off by the configuration parameter: CanIfGlobalTimeSupport]()

##### CanIf_GetEgressTimeStamp [SWS_CANIF_91012]{DRAFT} [

| Service Name | CanIf_GetEgressTimeStamp (draft) |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetEgressTimeStamp ( PduIdType TxPduId, Can_TimeStampType* timeStampPtr ) |  |
| Service ID [hex] | 0x53 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant for the same TxPduId, Reentrant for different TxPduId |  |
| Parameters (in) | TxPduId | L-PDU handle of CAN L-PDU for which the time stamp shall be returned. |
| Parameters (inout) | None |  |
| Parameters (out) | timeStampPtr | current time stamp |
| Return value | Std_ReturnType | E_OK: successful E_NOT_OK: failed |
| Description | This service calls the corresponding CAN Driver service to read back the egress time stamp on a dedicated message object. It needs to be called within the TxConfirmation() function. Tags: atp.Status=draft |  |
| Available via | CanIf.h |  |

](SRS_Can_01181)

[SWS_CANIF_00929]{DRAFT} [If development error detection is enabled: the func- tion shall check that the service CanIf_Init() was previously called. If the check fails, the function shall raise the development error CANIF_E_UNINIT]()

[SWS_CANIF_00930]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter TxPduId for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_LPDU.]()

[SWS_CANIF_00931]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter timeStampPtr for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_POINTER.]()

[SWS_CANIF_00932]{DRAFT} [The function shall be pre compile time configurable On/Off by the configuration parameter: CanIfGlobalTimeSupport]()

##### CanIf_GetIngressTimeStamp [SWS_CANIF_91013]{DRAFT} [

| Service Name | CanIf_GetIngressTimeStamp (draft) |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_GetIngressTimeStamp ( PduIdType RxPduId, Can_TimeStampType* timeStampPtr ) |  |
| Service ID [hex] | 0x54 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant for the same RxPduId, Reentrant for different RxPduIds |  |
| Parameters (in) | RxPduId | ID of the received I-PDU for which the time stamp shall be returned. |
| Parameters (inout) | None |  |
| Parameters (out) | timeStampPtr | current time stamp |
| Return value | Std_ReturnType | E_OK: successful E_NOT_OK: failed |
| Description | This service calls the corresponding CAN Driver service to reads back the ingress time stamp on a dedicated message object. It needs to be called within the RxIndication() function. Tags: atp.Status=draft |  |
| Available via | CanIf.h |  |

](SRS_Can_01181)

[SWS_CANIF_00933]{DRAFT} [If development error detection is enabled: the func- tion shall check that the service CanIf_Init() was previously called. If the check fails, the function shall raise the development error CANIF_E_UNINIT]()

[SWS_CANIF_00934]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter RxPduId for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_LPDU.]()

[SWS_CANIF_00935]{DRAFT} [If development error detection is enabled: the func- tion shall check the parameter timeStampPtr for being valid. If the check fails, the function shall raise the development error CANIF_E_PARAM_POINTER.]()

[SWS_CANIF_00936]{DRAFT} [The function shall be pre compile time configurable On/Off by the configuration parameter: CanIfGlobalTimeSupport]()

#### Callback notifications

This is a list of functions provided for other modules.

##### CanIf_TriggerTransmit [SWS_CANIF_00883] [

| Service Name | CanIf_TriggerTransmit |  |
| --- | --- | --- |
| Syntax | Std_ReturnType CanIf_TriggerTransmit ( PduIdType TxPduId, PduInfoType* PduInfoPtr ) |  |
| Service ID [hex] | 0x41 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant for different PduIds. Non reentrant for the same PduId. |  |
| Parameters (in) | TxPduId | ID of the SDU that is requested to be transmitted. |
| Parameters (inout) | PduInfoPtr | Contains a pointer to a buffer (SduDataPtr) to where the SDU data shall be copied, and the available buffer size in SduLengh. On return, the service will indicate the length of the copied SDU data in SduLength. |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: SDU has been copied and SduLength indicates the number of copied bytes. E_NOT_OK: No SDU data has been copied. PduInfoPtr must not be used since it may contain a NULL pointer or point to invalid data. |
| Description | Within this API, the upper layer module (called module) shall check whether the available data fits into the buffer size reported by PduInfoPtr->SduLength. If it fits, it shall copy its data into the buffer provided by PduInfoPtr->SduDataPtr and update the length of the actual copied data in PduInfoPtr->SduLength. If not, it returns E_NOT_OK without changing PduInfoPtr. |  |
| Available via | CanIf.h |  |

]()

[SWS_CANIF_00884] [CanIf shall only provide the API function CanIf_Trigger- Transmit() if TriggerTransmit support is enabled (CanIfTriggerTransmitSup- port = TRUE).]()

[SWS_CANIF_00885] [The function CanIf_TriggerTransmit() shall call the cor- responding <User_TriggerTransmit>() function, passing the translated TxPduId and the pointer to the PduInfo structure (PduInfoPtr). Upon return, CanIf_Trig- gerTransmit() shall return the return value of its <User_TriggerTransmit>().] ()

##### CanIf_TxConfirmation [SWS_CANIF_00007] [

| Service Name | CanIf_TxConfirmation |  |
| --- | --- | --- |
| Syntax | void CanIf_TxConfirmation ( PduIdType CanTxPduId ) |  |
| Service ID [hex] | 0x13 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | CanTxPduId | L-PDU handle of CAN L-PDU successfully transmitted. This ID specifies the corresponding CAN L-PDU ID and implicitly the CAN Driver instance as well as the corresponding CAN controller device. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service confirms a previously successfully processed transmission of a CAN TxPDU. |  |
| Available via | CanIf_Can.h |  |

](SRS_Can_01009)

Note: The service CanIf_TxConfirmation() is implemented in CanIf and called by the CanDrv after the CAN L-PDU has been transmitted on the CAN network.

Note: Due to the fact CanDrv does not support the HandleId concept as described in [14, Specification of ECU Configuration]: Within the service CanIf_TxConfirma- tion(), CanDrv uses PduInfo->swPduHandle as CanTxPduId, which was pre- served from Can_Write(Hth, *PduInfo).

[SWS_CANIF_00391] [If configuration parameters CanIfPublicReadTxPduNoti- fyStatusApi and CanIfTxPduReadNotifyStatus for the Transmitted L-PDU are set to TRUE, and if CanIf_TxConfirmation() is called, CanIf shall set the notification status for the Transmitted L-PDU.]()

[SWS_CANIF_00410] [If parameter CanTxPduId of CanIf_TxConfirmation() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_- LPDU to the Det_ReportError service of the DET module, when CanIf_TxCon- firmation() is called.](SRS_BSW_00323)

[SWS_CANIF_00412] [If CanIf was not initialized before calling CanIf_TxConfir- mation(), CanIf shall not call the service <User_TxConfirmation>() and shall not set the Tx confirmation status, when CanIf_TxConfirmation() is called.]()

Note: The call context of CanIf_TxConfirmation() is either on interrupt level (in- terrupt mode) or on task level (polling mode).

[SWS_CANIF_00414] [Configuration of CanIf_TxConfirmation(): Each Tx L-

-PDU (see CanIfTxPduCfg) has to be configured with a corresponding transmit con- firmation service of an upper layer module (see [SWS_CANIF_00011]) which is called in CanIf_TxConfirmation().]()

##### CanIf_RxIndication [SWS_CANIF_00006] [

| Service Name | CanIf_RxIndication |  |
| --- | --- | --- |
| Syntax | void CanIf_RxIndication ( const Can_HwType* Mailbox, const PduInfoType* PduInfoPtr ) |  |
| Service ID [hex] | 0x14 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | Mailbox | Identifies the HRH and its corresponding CAN Controller |
|  | PduInfoPtr | Pointer to the received L-PDU |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates a successful reception of a received CAN Rx L-PDU to the CanIf after passing all filters and validation checks. |  |
| Available via | CanIf_Can.h |  |

]()

Note: The service CanIf_RxIndication() is implemented in CanIf and called by CanDrv after a CAN L-PDU has been received.

[SWS_CANIF_00415] [Within the service CanIf_RxIndication() the CanIf

routes this indication to the configured upper layer target service(s).]()

[SWS_CANIF_00392] [If configuration parameters CanIfPublicReadRxPduNoti- fyStatusApi and CanIfRxPduReadNotifyStatus for the Received L-PDU are

set to TRUE, and if CanIf_RxIndication() is called, the CanIf shall set the notifi- cation status for the Received L-PDU.]()

[SWS_CANIF_00416] [If parameter Mailbox->Hoh of CanIf_RxIndication() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_- HOH to the Det_ReportError service of the DET module, when CanIf_RxIndica- tion() is called.](SRS_BSW_00323)

[SWS_CANIF_00417] [If parameter Mailbox->CanId of CanIf_RxIndication() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_- CANID to the Det_ReportError service of the DET module, when CanIf_RxIndi- cation() is called.](SRS_BSW_00323)

Note: If CanIf_RxIndication() is called with invalid PduInfoPtr-> SduLength,  runtime error CANIF_E_INVALID_DATA_LENGTH is reported (see [SWS_CANIF_00168]).

[SWS_CANIF_00419] [If parameter PduInfoPtr or Mailbox of CanIf_RxIndi- cation() has an invalid value, CanIf shall report development error code CANIF_- E_PARAM_POINTER to the Det_ReportError service of the DET module, when CanIf_RxIndication() is called.](SRS_BSW_00323)

[SWS_CANIF_00421] [If CanIf was not initialized before calling CanIf_RxIndica- tion(), CanIf shall not execute Rx indication handling, when CanIf_RxIndica- tion(), is called.]()

Note: The call context of CanIf_RxIndication() is either on interrupt level (inter- rupt mode) or on task level (polling mode).

[SWS_CANIF_00423] [Configuration of CanIf_RxIndication(): Each Rx L-PDU (see CanIfRxPduCfg) has to be configured with a corresponding receive indica- tion service of an upper layer module (see [SWS_CANIF_00012]) which is called in CanIf_RxIndication().]()

##### CanIf_ControllerBusOff [SWS_CANIF_00218] [

| Service Name | CanIf_ControllerBusOff |  |
| --- | --- | --- |
| Syntax | void CanIf_ControllerBusOff ( uint8 ControllerId ) |  |
| Service ID [hex] | 0x16 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ControllerId | Abstract CanIf ControllerId which is assigned to a CAN controller, where a BusOff occured. |

| Parameters (inout) | None |
| --- | --- |
| Parameters (out) | None |
| Return value | None |
| Description | This service indicates a Controller BusOff event referring to the corresponding CAN Controller with the abstract CanIf ControllerId. |
| Available via | CanIf_Can.h |

]()

Note: The callback service CanIf_ControllerBusOff() is called by CanDrv and implemented in CanIf. It is called in case of a mode change notification of the CanDrv.

[SWS_CANIF_00429] [If parameter ControllerId of CanIf_ControllerBusOff () has an invalid value, CanIf shall report development error code CANIF_E_- PARAM_CONTROLLERID to the Det_ReportError service of the DET module, when CanIf_ControllerBusOff() is called.](SRS_BSW_00323)

[SWS_CANIF_00431] [If CanIf was not initialized before calling CanIf_Con- trollerBusOff(), CanIf shall not execute BusOff notification, when CanIf_Con- trollerBusOff(), is called.]()

Note: The call context of CanIf_ControllerBusOff() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00433] [Configuration of CanIf_ControllerBusOff(): ID of the CAN Controller is published inside the configuration description of the CanIf (see CanIfCtrlCfg).]()

Note: This service always has to be available, so there does not exist an appropriate configuration parameter.

##### CanIf_ConfirmPnAvailability [SWS_CANIF_00815] [

| Service Name | CanIf_ConfirmPnAvailability |  |
| --- | --- | --- |
| Syntax | void CanIf_ConfirmPnAvailability ( uint8 TransceiverId ) |  |
| Service ID [hex] | 0x1a |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to a CAN transceiver, which was checked for PN availability. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |

| Return value | None |
| --- | --- |
| Description | This service indicates that the transceiver is running in PN communication mode referring to the corresponding CAN transceiver with the abstract CanIf TransceiverId. |
| Available via | CanIf_CanTrcv.h |

[SWS_CANIF_00753] [If CanIf_ConfirmPnAvailability() is called, CanIf

calls <User_ConfirmPnAvailability>().]()

Note: CanIf passes the delivered parameter TransceiverId to the upper layer mod- ule.

[SWS_CANIF_00816] [If parameter TransceiverId of CanIf_ConfirmPnAvail- ability() has an invalid value, CanIf shall report development error code CANIF_- E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_ConfirmPnAvailability() is called.]()

[SWS_CANIF_00817] [If CanIf was not initialized before calling CanIf_ConfirmP- nAvailability(), CanIf shall not execute notification, when CanIf_ConfirmP- nAvailability() is called.]()

Note: The call context of CanIf_ConfirmPnAvailability() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00754] [Configuration of CanIf_ConfirmPnAvailability(): This function shall be pre compile time configurable ON/OFF by the configuration parameter CanIfPublicPnSupport.]()

##### CanIf_ClearTrcvWufFlagIndication [SWS_CANIF_00762] [

| Service Name | CanIf_ClearTrcvWufFlagIndication |  |
| --- | --- | --- |
| Syntax | void CanIf_ClearTrcvWufFlagIndication ( uint8 TransceiverId ) |  |
| Service ID [hex] | 0x20 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to a CAN transceiver, for which this function was called. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |

| Description | This service indicates that the transceiver has cleared the WufFlag referring to the corresponding CAN transceiver with the abstract CanIf TransceiverId. |
| --- | --- |
| Available via | CanIf_CanTrcv.h |

[SWS_CANIF_00757] [If CanIf_ClearTrcvWufFlagIndication() is called,

CanIf calls <User_ClearTrcvWufFlagIndication>().]()

Note: CanIf passes the delivered parameter TransceiverId to the upper layer mod- ule.

[SWS_CANIF_00805] [If parameter TransceiverId of CanIf_ClearTrcvWuf- FlagIndication() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_ClearTrcvWufFlagIndication() is called.]()

[SWS_CANIF_00806] [If CanIf was not initialized before calling CanIf_ClearTr- cvWufFlagIndication(), CanIf shall not execute notification, when CanIf_- ClearTrcvWufFlagIndication() is called.]()

Note: The call context of CanIf_ClearTrcvWufFlagIndication() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00808] [Configuration of CanIf_ClearTrcvWufFlagIndication (): This function shall be pre compile time configurable ON/OFF by the configuration parameter CanIfPublicPnSupport.]()

##### CanIf_CheckTrcvWakeFlagIndication [SWS_CANIF_00763] [

| Service Name | CanIf_CheckTrcvWakeFlagIndication |  |
| --- | --- | --- |
| Syntax | void CanIf_CheckTrcvWakeFlagIndication ( uint8 TransceiverId ) |  |
| Service ID [hex] | 0x21 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to a CAN transceiver, for which this function was called. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates that the check of the transceiver’s wake-up flag has been finished by the corresponding CAN transceiver with the abstract CanIf TransceiverId. This indication is used to cope with the asynchronous transceiver communication. |  |
| Available via | CanIf_CanTrcv.h |  |

[SWS_CANIF_00759] [If CanIf_CheckTrcvWakeFlagIndication() is called,

CanIf calls <User_CheckTrcvWakeFlagIndication>().]()

Note: CanIf passes the delivered parameter TransceiverId to the upper layer mod- ule.

[SWS_CANIF_00809] [If parameter TransceiverId of CanIf_CheckTrcvWake- FlagIndication() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_CheckTrcvWakeFlagIndication() is called.]()

[SWS_CANIF_00810] [If the CanIf was not initialized before calling CanIf_Check- TrcvWakeFlagIndication(), CanIf shall not execute notification, when CanIf_- CheckTrcvWakeFlagIndication() is called.]()

Note: The call context of CanIf_CheckTrcvWakeFlagIndication() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00812] [Configuration of CanIf_CheckTrcvWakeFlagIndication (): This function shall be pre compile time configurable ON/OFF by the configuration parameter CanIfPublicPnSupport.]()

##### CanIf_ControllerModeIndication [SWS_CANIF_00699] [

| Service Name | CanIf_ControllerModeIndication |  |
| --- | --- | --- |
| Syntax | void CanIf_ControllerModeIndication ( uint8 ControllerId, Can_ControllerStateType ControllerMode ) |  |
| Service ID [hex] | 0x17 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ControllerId | Abstract CanIf ControllerId which is assigned to a CAN controller, which state has been transitioned. |
|  | ControllerMode | Mode to which the CAN controller transitioned |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates a controller state transition referring to the corresponding CAN controller with the abstract CanIf ControllerId. |  |
| Available via | CanIf_Can.h |  |

Note: The callback service CanIf_ControllerModeIndication() is called by CanDrv and implemented in CanIf. It is called in case of a state transition notification of the CanDrv.

[SWS_CANIF_00700] [If parameter ControllerId of CanIf_ControllerMod- eIndication() has an invalid value, CanIf shall report development error code CANIF_E_PARAM_CONTROLLERID to the Det_ReportError service of the DET module, when CanIf_ControllerModeIndication() is called.]()

[SWS_CANIF_00702] [If CanIf was not initialized before calling CanIf_Con- trollerModeIndication(), CanIf shall not execute state transition notification, when CanIf_ControllerModeIndication() is called.]()

Note: The call context of CanIf_ControllerModeIndication() is either on inter- rupt level (interrupt mode) or on task level (polling mode).

##### CanIf_TrcvModeIndication [SWS_CANIF_00764] [

| Service Name | CanIf_TrcvModeIndication |  |
| --- | --- | --- |
| Syntax | void CanIf_TrcvModeIndication ( uint8 TransceiverId, CanTrcv_TrcvModeType TransceiverMode ) |  |
| Service ID [hex] | 0x22 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to a CAN transceiver, which state has been transitioned. |
|  | TransceiverMode | Mode to which the CAN transceiver transitioned |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates a transceiver state transition referring to the corresponding CAN transceiver with the abstract CanIf TransceiverId. |  |
| Available via | CanIf_CanTrcv.h |  |

]()

Note: The callback service CanIf_TrcvModeIndication() is called by CanDrv and implemented in CanIf. It is called in case of a state transition notification of the CanDrv.

[SWS_CANIF_00706] [If parameter TransceiverId of CanIf_TrcvModeIndica- tion() has an invalid value, CanIf shall report development error code CANIF_E_- PARAM_TRCV to the Det_ReportError service of the DET module, when CanIf_- TrcvModeIndication() is called.]()

[SWS_CANIF_00708] [If CanIf was not initialized before calling CanIf_TrcvMod- eIndication(), CanIf shall not execute state transition notification, when CanIf_- TrcvModeIndication() is called.]()

Note: The call context of CanIf_TrcvModeIndication() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00710] [Configuration of CanIf_TrcvModeIndication(): ID of the CAN Transceiver is published inside the configuration description of CanIf via pa- rameter CanIfTrcvId.]()

[SWS_CANIF_00730] [Configuration of CanIf_TrcvModeIndication(): If transceivers are not supported (CanIfTrcvDrvCfg is not configured, see CanIfTr- cvDrvCfg), CanIf_TrcvModeIndication() shall not be provided by CanIf.]()

##### CanIf_ControllerErrorStatePassive [SWS_CANIF_91008] [

| Service Name | CanIf_ControllerErrorStatePassive |  |
| --- | --- | --- |
| Syntax | void CanIf_ControllerErrorStatePassive ( uint8 ControllerId, uint16 RxErrorCounter, uint16 TxErrorCounter ) |  |
| Service ID [hex] | 0x4f |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller. |
|  | RxErrorCounter | Value of the Rx error counter |
|  | TxErrorCounter | Value of the Tx error counter |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | void | – |
| Description | The function derives the ErrorCounterTreshold from RxErrorCounter/ TxErrorCounter values and reports it to the IdsM as security event CANIF_SEV_ERRORSTATE_PASSIVE to the IdsM. It also prepares the context data for the respective security event. |  |
| Available via | CanIf_Can.h |  |

](RS_Ids_00810)

[SWS_CANIF_00919] [If parameter ControllerId of CanIf_Con- trollerErrorStatePassive() has an invalid value, the CanIf shall report development error code CANIF_E_PARAM_CONTROLLERID to the Det_ReportEr- ror service of the DET module when CanIf_ControllerErrorStatePassive() is called.](RS_Ids_00810)

##### CanIf_ErrorNotification [SWS_CANIF_91009] [

| Service Name | CanIf_ErrorNotification |  |
| --- | --- | --- |
| Syntax | void CanIf_ErrorNotification ( uint8 ControllerId, Can_ErrorType Can_ErrorType ) |  |
| Service ID [hex] | 0x50 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller. |
|  | Can_ErrorType | Reported CAN error |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | void | – |
| Description | The function shall derive the bus error source rx or tx from the parameter CanError and report the bus error as security event CANIF_SEV_TX_ERROR_DETECTED or CANIF_SEV_RX_ ERROR_DETECTED. It also prepares the context data for the respective security event. |  |
| Available via | CanIf_Can.h |  |

](RS_Ids_00810)

[SWS_CANIF_00920] [If parameter ControllerId of CanIf_ErrorNotifica- tion() has an invalid value, the CanIf shall report development error code CANIF_- E_PARAM_CONTROLLERID to the Det_ReportError service of the DET module, when CanIf_ErrorNotification() is called.](RS_Ids_00810)

[SWS_CANIF_00921] [If parameter CanError of CanIf_ErrorNotification() has an invalid value, the CanIf shall report development error code CANIF_E_- PARAM_CAN_ERROR to the Det_ReportError service of the DET module, when CanIf_ErrorNotification() is called.](RS_Ids_00810)

#### Scheduled functions

Note: CanIf does not have scheduled functions or needs some.

#### Expected interfaces

In this chapter all interfaces required from other modules are listed.

##### Mandatory interfaces

Note: This section defines all interfaces, which are required to fulfill the core function- ality of the module.

[SWS_CANIF_00040] [

| API Function | Header File | Description |
| --- | --- | --- |
| Can_GetControllerErrorState | Can.h | This service obtains the error state of the CAN controller. |
| Can_GetControllerRxErrorCounter | Can.h | Returns the Rx error counter for a CAN controller. This value might not be available for all CAN controllers, in which case E_NOT_OK would be returned. Please note that the value of the counter might not be correct at the moment the API returns it, because the Rx counter is handled asynchronously in hardware. Applications should not trust this value for any assumption about the current bus state. |
| Can_GetControllerTxErrorCounter | Can.h | Returns the Tx error counter for a CAN controller. This value might not be available for all CAN controllers, in which case E_NOT_OK would be returned. Please note that the value of the counter might not be correct at the moment the API returns it, because the Tx counter is handled asynchronously in hardware. Applications should not trust this value for any assumption about the current bus state. |
| Can_SetControllerMode | Can.h | This function performs software triggered state transitions of the CAN controller State machine. |
| Can_Write | Can.h | This function is called by CanIf to pass a CAN message to CanDrv for transmission. |
| Det_ReportRuntimeError | Det.h | Service to report runtime errors. If a callout has been configured then this callout shall be called. |
| SchM_Enter_CanIf_<ExclusiveArea> | SchM_<Mip>.h | Invokes the SchM_Enter function to enter a module local exclusive area. |
| SchM_Exit_CanIf_<ExclusiveArea> | SchM_<Mip>.h | Invokes the SchM_Exit function to exit an exclusive area. |

##### Optional interfaces

This section defines all interfaces, which are required to fulfill an optional functionality of the module.

[SWS_CANIF_00294] [

| API Function | Header File | Description |
| --- | --- | --- |
| Can_CheckWakeup | Can.h | This function checks if a wakeup has occurred for the given controller. |
| Can_SetBaudrate | Can.h | This service shall set the baud rate configuration of the CAN controller. Depending on necessary baud rate modifications the controller might have to reset. |
| CanNm_RxIndication | CanNm.h | Indication of a received PDU from a lower layer communication interface module. |
| CanNm_TxConfirmation | CanNm.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |
| CanSM_CheckTransceiverWakeFlag Indication | CanSM_CanIf.h | This callback function indicates the CanIf_Check TrcvWakeFlag API process end for the notified CAN Transceiver. |
| CanSM_ClearTrcvWufFlagIndication | CanSM_CanIf.h | This callback function shall indicate the CanIf_Clear TrcvWufFlag API process end for the notified CAN Transceiver. |
| CanSM_ConfirmPnAvailability | CanSM_CanIf.h | This callback function indicates that the transceiver is running in PN communication mode. |
| CanSM_ControllerBusOff | CanSM_CanIf.h | This callback function notifies the CanSM about a bus-off event on a certain CAN controller, which needs to be considered with the specified bus-off recovery handling for the impacted CAN network. |
| CanSM_ControllerModeIndication | CanSM_CanIf.h | This callback shall notify the CanSM module about a CAN controller mode change. |
| CanSM_TransceiverModeIndication | CanSM_CanIf.h | This callback shall notify the CanSM module about a CAN transceiver mode change. |
| CanTp_RxIndication | CanTp.h | Indication of a received PDU from a lower layer communication interface module. |
| CanTp_TxConfirmation | CanTp.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |
| CanTrcv_CheckWakeFlag | CanTrcv.h | Requests to check the status of the wakeup flag from the transceiver hardware. |
| CanTrcv_CheckWakeup | CanTrcv.h | Service is called by underlying CANIF in case a wake up interrupt is detected. |
| CanTrcv_GetBusWuReason | CanTrcv.h | Gets the wakeup reason for the Transceiver and returns it in parameter Reason. |
| CanTrcv_GetOpMode | CanTrcv.h | Gets the mode of the Transceiver and returns it in OpMode. |
| CanTrcv_SetOpMode | CanTrcv.h | Sets the mode of the Transceiver to the value Op Mode. |
| CanTrcv_SetWakeupMode | CanTrcv.h | Enables, disables or clears wake-up events of the Transceiver according to TrcvWakeupMode. |
| CanTSyn_RxIndication | CanTSyn.h | Indication of a received PDU from a lower layer communication interface module. |
| CanTSyn_TxConfirmation | CanTSyn.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |
| Det_ReportError | Det.h | Service to report development errors. |
| EcuM_ValidateWakeupEvent | EcuM.h | After wakeup, the ECU State Manager will stop the process during the WAKEUP VALIDATION state/ sequence to wait for validation of the wakeup event.This API service is used to indicate to the ECU Manager module that the wakeup events indicated in the sources parameter have been validated. |
| IdsM_SetSecurityEventWithContext Data | IdsM.h | This API is the application interface to report security events with context data to the IdsM. |
| J1939Nm_RxIndication | J1939Nm.h | Indication of a received PDU from a lower layer communication interface module. |
| J1939Nm_TxConfirmation | J1939Nm.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |
| J1939Tp_RxIndication | J1939Tp.h | Indication of a received PDU from a lower layer communication interface module. |
| J1939Tp_TxConfirmation | J1939Tp.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |
| Mirror_ReportCanFrame | Mirror.h | Reports a received or transmitted CAN frame. All received CAN frames that pass the hardware acceptance filter are reported, independent of the software filter configuration. Transmitted CAN frames are reported when the transmission is confirmed. |
| PduR_CanIfRxIndication | PduR_CanIf.h | Indication of a received PDU from a lower layer communication interface module. |
| PduR_CanIfTxConfirmation | PduR_CanIf.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |
| Xcp_CanIfRxIndication | Xcp.h | Indication of a received PDU from a lower layer communication interface module. |
| Xcp_CanIfTxConfirmation | Xcp.h | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |

##### Configurable interfaces

In this section all interfaces are listed, where the target function of any upper layer to be called has to be set up by configuration. These callback services are specified and implemented in the upper communication modules, which use CanIf according to the AUTOSAR BSW architecture. The specific callback notification is specified in the corresponding SWS document (see chapter 3 “Related documentation”).

As far the interface name is not specified to be mandatory, no callback is performed, if no API name is configured. This section describes only the content of notification of the callback, the call context inside CanIf and exact time by the call event.

<User_NotificationName> - This condition is applied for such interface services which will be implemented in the upper layer and called by CanIf. This condition displays the symbolic name of the functional group in a callback service in the corre- sponding upper layer module. Each upper layer module can define no, one or several callback services for the same functionality (i.e. transmit confirmation). The dispatch is ensured by the L-SDU ID.

The upper layer module provides the Service ID of the following functions.

###### <User_TriggerTransmit> [SWS_CANIF_00886] [

| Service Name | <User_TriggerTransmit> |  |
| --- | --- | --- |
| Syntax | Std_ReturnType <User_TriggerTransmit> ( PduIdType TxPduId, PduInfoType* PduInfoPtr ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant for different PduIds. Non reentrant for the same PduId. |  |
| Parameters (in) | TxPduId | ID of the SDU that is requested to be transmitted. |
| Parameters (inout) | PduInfoPtr | Contains a pointer to a buffer (SduDataPtr) to where the SDU data shall be copied, and the available buffer size in SduLengh. On return, the service will indicate the length of the copied SDU data in SduLength. |
| Parameters (out) | None |  |
| Return value | Std_ReturnType | E_OK: SDU has been copied and SduLength indicates the number of copied bytes. E_NOT_OK: No SDU data has been copied. PduInfoPtr must not be used since it may contain a NULL pointer or point to invalid data. |
| Description | Within this API, the upper layer module (called module) shall check whether the available data fits into the buffer size reported by PduInfoPtr->SduLength. If it fits, it shall copy its data into the buffer provided by PduInfoPtr->SduDataPtr and update the length of the actual copied data in PduInfoPtr->SduLength. If not, it returns E_NOT_OK without changing PduInfoPtr. |  |
| Available via | configurable |  |

Note: This callback service is called by CanIf and implemented in the corresponding upper layer module. It is called in case of a Trigger Transmit request of CanDrv.

Note: The call context of <User_TriggerTransmit>() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00888] [Configuration of <User_TriggerTransmit>(): The upper layer module, which provides the TriggerTransmit callback service, has to be con- figured by CanIfTxPduUserTxConfirmationUL (see CanIfTxPduUserTxCon- firmationUL). If no upper layer modules are configured, no TriggerTransmit call- back service is executed and therefore Trigger Transmit functionality is not supported for that PDU.]()

[SWS_CANIF_00889] [Configuration of <User_TriggerTransmit>(): The name of the API <User_TriggerTransmit>() which is called by CanIf shall be con- figured for CanIf by parameter CanIfTxPduUserTriggerTransmitName (see CanIfTxPduUserTriggerTransmitName).]()

Note: If CanIfTxPduTriggerTransmit is not specified or FALSE, no upper layer modules have to be configured for Trigger Transmit. Therefore, <User_Trigger- Transmit>() will not be called and CanIfTxPduUserTxConfirmationUL as well as CanIfTxPduUserTriggerTransmitName need not to be configured.

[SWS_CANIF_00890] [Configuration of <User_TriggerTransmit>(): If CanI- fTxPduUserTxConfirmationUL is set to PDUR, CanIfTxPduUserTrigger- TransmitName must be PduR_CanIfTriggerTransmit.]()

[SWS_CANIF_00891] [Configuration of <User_TriggerTransmit>(): If CanI- fTxPduUserTxConfirmationUL is set to CDD, the name of the API <User_Trig- gerTransmit>() has to be configured via parameter CanIfTxPduUserTrigger- TransmitName.]()

###### <User_TxConfirmation> [SWS_CANIF_00011] [

| Service Name | <User_TxConfirmation> |  |
| --- | --- | --- |
| Syntax | void <User_TxConfirmation> ( PduIdType TxPduId, Std_ReturnType result ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant for different PduIds. Non reentrant for the same PduId. |  |
| Parameters (in) | TxPduId | ID of the PDU that has been transmitted. |
|  | result | E_OK: The PDU was transmitted. E_NOT_OK: Transmission of the PDU failed. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | The lower layer communication interface module confirms the transmission of a PDU, or the failure to transmit a PDU. |  |
| Available via | configurable |  |

Note: This callback service is called by CanIf and implemented in the corresponding upper layer module. It is called in case of a transmit confirmation of CanDrv.

Note: This type of confirmation callback service is mainly designed for PduR, CanNm, and CanTp, but not exclusive.

Note: Parameter TxPduId is derived from <User> configuration.

Note: The call context of <User_TxConfirmation>() is either on interrupt level (interrupt mode) or on task level (polling mode).

[SWS_CANIF_00438] [Configuration of <User_TxConfirmation>(): The upper layer module, which provides this callback service, has to be configured by CanI- fTxPduUserTxConfirmationUL. If no upper layer modules are configured for trans- mit confirmation using <User_TxConfirmation>(), no transmit confirmation is ex- ecuted.]()

[SWS_CANIF_00542] [Configuration of <User_TxConfirmation>(): The name of the API <User_TxConfirmation>() which is called by CanIf shall be configured for CanIf by parameter CanIfTxPduUserTxConfirmationName.]()

Note: If transmit confirmations are not necessary or no upper layer modules are con- figured for transmit confirmations and thus <User_TxConfirmation>() shall not be called, CanIfTxPduUserTxConfirmationUL and CanIfTxPduUserTxConfir- mationName need not to be configured.

[SWS_CANIF_00439] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to PDUR, CanIfTxPduUserTxConfirmation- Name must be PduR_CanIfTxConfirmation.]()

[SWS_CANIF_00543] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to CAN_NM, CanIfTxPduUserTxConfirma- tionName must be CanNm_TxConfirmation.]()

Hint (Dependency to another module):

If at least one CanIf Tx L-SDU is configured with CanNm_TxConfirmation(), which means CanIfTxPduUserTxConfirmationUL equals CAN_NM, the CanNm config- uration parameter CANNM_IMMEDIATE_TXCONF_ENABLED must be set to FALSE (for CanNm related details see [4, Specification of CAN Network Management], [SWS_CANNM_00284]).

[SWS_CANIF_00858] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to J1939NM, CanIfTxPduUserTxConfirma- tionName must be J1939Nm_TxConfirmation.]()

[SWS_CANIF_00544] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to J1939TP, CanIfTxPduUserTxConfirma- tionName must be J1939Tp_TxConfirmation.]()

[SWS_CANIF_00550] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to CAN_TP, CanIfTxPduUserTxConfirma- tionName must be CanTp_TxConfirmation.]()

[SWS_CANIF_00556] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to XCP, CanIfTxPduUserTxConfirmation- Name must be Xcp_CanIfTxConfirmation.]()

[SWS_CANIF_00551] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to CDD, the name of the API <User_TxCon- firmation>() has to be configured via parameter CanIfTxPduUserTxConfir- mationName.]()

[SWS_CANIF_00879] [Configuration of <User_TxConfirmation>(): If CanIfTx- PduUserTxConfirmationUL is set to CAN_TSYN, CanIfTxPduUserTxConfirma- tionName must be CanTSyn_TxConfirmation.]()

###### <User_RxIndication> [SWS_CANIF_00012] [

| Service Name | <User_RxIndication> |  |
| --- | --- | --- |
| Syntax | void <User_RxIndication> ( PduIdType RxPduId, const PduInfoType* PduInfoPtr ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant for different PduIds. Non reentrant for the same PduId. |  |
| Parameters (in) | RxPduId | ID of the received PDU. |
|  | PduInfoPtr | Contains the length (SduLength) of the received PDU, a pointer to a buffer (SduDataPtr) containing the PDU, and the MetaData related to this PDU. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Indication of a received PDU from a lower layer communication interface module. |  |
| Available via | configurable |  |

](SRS_Can_01003)

Note: This service indicates a successful reception of an L-SDU to the upper layer module after passing all filters and validation checks.

Note: This callback service is called by CanIf and implemented in the configured upper layer module (e.g. PduR, CanNm, CanTp, etc.) if configured accordingly (see CanIfRxPduUserRxIndicationUL).

Note: Until <User_RxIndication>() returns, CanIf will not access <PduIn- foPtr>. The <PduInfoPtr> is only valid and can be used by upper layers, until the indication returns. CanIf guarantees that the number of configured bytes for this

<PduInfoPtr> is valid.

Note: The call context of <User_RxIndication>() is either on interrupt level (inter- rupt mode) or on task level (polling mode).

[SWS_CANIF_00441] [Configuration of <User_RxIndication>(): The upper layer module, which provides this callback service, has to be configured by CanIfRxPdu- UserRxIndicationUL.]()

[SWS_CANIF_00552] [Configuration of <User_RxIndication>(): The name of the API <User_RxIndication>() which will be called by CanIf shall be configured for CanIf by parameter CanIfRxPduUserRxIndicationName.]()

Note: If receive indications are not necessary or no upper layer modules are configured for receive indications and thus <User_RxIndication>() shall not be called, Can- IfRxPduUserRxIndicationUL and CanIfRxPduUserRxIndicationName need not to be configured.

[SWS_CANIF_00442] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to PDUR, CanIfRxPduUserRxIndicationName must be PduR_CanIfRxIndication.]()

[SWS_CANIF_00445] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to CAN_NM, CanIfRxPduUserRxIndicationName must be CanNm_RxIndication.]()

The value passed to CanNm via the API parameter CanNmRxPduId refers to the CanNm channel handle within the CanNm module (for CanNm related details see [4, Specifica- tion of CAN Network Management]).

[SWS_CANIF_00859] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to J1939NM, CanIfRxPduUserRxIndication- Name must be J1939Nm_RxIndication.]()

[SWS_CANIF_00448] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to CAN_TP, CanIfRxPduUserRxIndicationName must be CanTp_RxIndication.]()

[SWS_CANIF_00554] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to J1939TP, CanIfRxPduUserRxIndication- Name must be J1939Tp_RxIndication.]()

[SWS_CANIF_00555] [Configuration of <User_RxIndication>(): If CanIfRx- PduUserRxIndicationUL is set to XCP, CanIfRxPduUserRxIndicationName must be Xcp_CanIfRxIndication.]()

[SWS_CANIF_00557] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to CDD the name of the API has to be configured via parameter CanIfRxPduUserRxIndicationName.]()

[SWS_CANIF_00880] [Configuration of <User_RxIndication>(): If CanIfRxP- duUserRxIndicationUL is set to CAN_TSYN, CanIfRxPduUserRxIndication- Name must be CanTSyn_RxIndication.]()

###### <User_ValidateWakeupEvent> [SWS_CANIF_00532] [

| Service Name | <User_ValidateWakeupEvent> |  |
| --- | --- | --- |
| Syntax | void <User_ValidateWakeupEvent> ( EcuM_WakeupSourceType sources ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant (defined within providing upper layer module) |  |
| Parameters (in) | sources | Validated CAN wakeup events. Every CAN controller or CAN transceiver can be a separate wakeup source. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates if a wake up event initiated from the wake up source (CAN controller or transceiver) after a former request to the CAN Driver or CAN Transceiver Driver module is valid. |  |
| Available via | configurable |  |

Note: This callback service is mainly implemented in and used by the ECU State Man- ager module (see [13, Specification of ECU State Manager]).

Note: The CanIf calls this callback service. It is implemented by the configured up- per layer module. It is called only during the call of CanIf_CheckValidation() if a first CAN L-PDU reception event after a wake up event has been occurred at the corresponding CAN Controller.

Note: The call context of <User_ValidateWakeupEvent>() is either on interrupt level (interrupt mode) or on task level (polling mode).

Note: The callback service <User_ValidateWakeupEvent>() is in general re- entrant for multiple CAN Controller usage, but not for the same CAN Controller

[SWS_CANIF_00659] [Configuration of <User_ValidateWakeupEvent>(): If no validation is needed, this API can be omitted by disabling CanIfPublicWake- upCheckValidSupport.]()

[SWS_CANIF_00456] [Configuration of <User_ValidateWakeupEvent>(): The upper layer module which provides this callback service has to be configured by Can- IfDispatchUserValidateWakeupEventUL, but:

- If no upper layer modules are configured for wake up notification using <User_- ValidateWakeupEvent>(), no wake up notification needs to be configured. CanIfDispatchUserValidateWakeupEventUL needs not to be configured.

- If wake up is not supported (CanIfCtrlWakeupSupport and CanIfTr- cvWakeupSupport equal FALSE, CanIfDispatchUserValidateWakeu- pEventUL is not configurable.

[SWS_CANIF_00563] [Configuration of <User_ValidateWakeupEvent>():	If CanIfDispatchUserValidateWakeupEventUL is set to ECUM, CanIfDis- patchUserValidateWakeupEventName must	be	EcuM_ValidateWakeu- pEvent.]()

[SWS_CANIF_00564] [Configuration of <User_ValidateWakeupEvent>():	If CanIfDispatchUserValidateWakeupEventUL is set to CDD the name of the API has to be configured via parameter CanIfDispatchUserValidateWakeupEvent- Name.]()

###### <User_ControllerBusOff> [SWS_CANIF_00014] [

| Service Name | <User_ControllerBusOff> |  |
| --- | --- | --- |
| Syntax | void <User_ControllerBusOff> ( uint8 ControllerId ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant (defined within providing upper layer module) |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller, at which a BusOff occurred. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates a bus-off event to the corresponding upper layer module (mainly the CAN State Manager module). |  |
| Available via | configurable |  |

](SRS_Can_01029)

Note: This callback service is mainly implemented in and used by CanSm (see [3, Specification of CAN State Manager]).

Note: This callback service is called by CanIf and implemented by the configured up- per layer module. It is called in case of a BusOff notification via CanIf_Controller- BusOff() of the CanDrv. The delivered parameter ControllerId of the service CanIf_ControllerBusOff() is passed to the upper layer module.

Note: The call context of <User_ControllerBusOff>() is either on interrupt level (interrupt mode) or on task level (polling mode).

Note: The callback service <User_ControllerBusOff>() is in general re-entrant for multiple CAN Controller usage, but not for the same CAN Controller.

Note: Before re-initialization/restart during BusOff recovery is executed <User_Con- trollerBusOff>() is performed only once in case of multiple BusOff events at CAN Controller.

Configuration of <User_ControllerBusOff>()

[SWS_CANIF_00450] [Configuration of <User_ControllerBusOff>(): The upper layer module which provides this callback service has to be configured by CanIfDis- patchUserCtrlBusOffUL.]()

[SWS_CANIF_00558] [Configuration of <User_ControllerBusOff>(): The name of the API <User_ControllerBusOff>() which will be called by CanIf shall be configured for CanIf by parameter CanIfDispatchUserCtrlBusOffName.]()

[SWS_CANIF_00524] [Configuration of <User_ControllerBusOff>(): At least one upper layer module and hence an API of <User_ControllerBusOff>() has mandatorily to be configured, which CanIf can call in case of an occurred call of CanIf_ControllerBusOff().]()

[SWS_CANIF_00559] [Configuration of <User_ControllerBusOff>(): If Can- IfDispatchUserCtrlBusOffUL is set to CAN_SM, CanIfDispatchUserCtrlBu- sOffName must be CanSM_ControllerBusOff.]()

[SWS_CANIF_00560] [Configuration of <User_ControllerBusOff>(): If Can- IfDispatchUserCtrlBusOffUL is set to CDD the name of the API has to be config- ured via parameter CanIfDispatchUserCtrlBusOffName.]()

###### <User_ConfirmPnAvailability> [SWS_CANIF_00821] [

| Service Name | <User_ConfirmPnAvailability> |  |
| --- | --- | --- |
| Syntax | void <User_ConfirmPnAvailability> ( uint8 TransceiverId ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant (defined within providing upper layer module) |  |
| Parameters (in) | TransceiverId | Abstract CanIf TransceiverId, which is assigned to a CAN transceiver, which was checked for PN availability. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates that the CAN transceiver is running in PN communication mode. |  |
| Available via | configurable |  |

Note: This callback service is mainly implemented in and used by CanSm (see [3, Specification of CAN State Manager]).

Note: The call context of <User_ConfirmPnAvailability>() is either on interrupt level (interrupt mode) or on task level (polling mode).

Note: The callback service <User_ConfirmPnAvailability>() is in general re- entrant for multiple CAN Controller usage, but not for the same CAN Controller

[SWS_CANIF_00823] [Configuration of <User_ConfirmPnAvailability>(): The upper layer module, which is called (see [SWS_CANIF_00753]), has to be config- urable by CanIfDispatchUserConfirmPnAvailabilityUL if CanIfPublicPn- Support equals True.]()

[SWS_CANIF_00824]  [Configuration  of  <User_ConfirmPnAvailability>()

:  The name of <User_ConfirmPnAvailability>() shall be configurable by CanIfDispatchUserConfirmPnAvailabilityName if CanIfPublicPnSup- port equals True.]()

[SWS_CANIF_00825] [Configuration of <User_ConfirmPnAvailability>(): It shall be configurable by CanIfPublicPnSupport, if CanIf supports this service ( False: not supported, True: supported)]()

[SWS_CANIF_00826] [Configuration of <User_ConfirmPnAvailability>(): If CanIfDispatchUserConfirmPnAvailabilityUL is set to CAN_SM, CanIfDis- patchUserConfirmPnAvailabilityName must be CanSM_ConfirmPnAvail- ability.]()

[SWS_CANIF_00827] [Configuration of <User_ConfirmPnAvailability>(): If CanIfDispatchUserConfirmPnAvailabilityUL is set to CDD, the name of the service has to be configurable via parameter CanIfDispatchUserConfirmP- nAvailabilityName.]()

###### <User_ClearTrcvWufFlagIndication> [SWS_CANIF_00788] [

| Service Name | <User_ClearTrcvWufFlagIndication> |  |
| --- | --- | --- |
| Syntax | void <User_ClearTrcvWufFlagIndication> ( uint8 TransceiverId ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId, for which this function was called. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates that the CAN transceiver has cleared the WufFlag. This function is called in CanIf_ClearTrcvWufFlagIndication. |  |
| Available via | configurable |  |

Note: This callback service is mainly implemented in and used by CanSm (see [3, Specification of CAN State Manager]).

Note: The call context of <User_ClearTrcvWufFlagIndication>() is either on interrupt level (interrupt mode) or on task level (polling mode).

Note: The callback service <User_ClearTrcvWufFlagIndication>() is in gen- eral re-entrant for multiple CAN Controller usage, but not for the same CAN Controller

[SWS_CANIF_00794] [Configuration of <User_ClearTrcvWufFlagIndication> (): The upper layer module, which is called (see [SWS_CANIF_00757]), has to be configurable by CanIfDispatchUserClearTrcvWufFlagIndicationUL if Can- IfPublicPnSupport equals True.]()

[SWS_CANIF_00795] [Configuration of <User_ClearTrcvWufFlagIndication> (): The name of <User_ClearTrcvWufFlagIndication>() shall be config- urable by CanIfDispatchUserClearTrcvWufFlagIndicationName if CanIf- PublicPnSupport equals True.]()

[SWS_CANIF_00796] [Configuration of <User_ClearTrcvWufFlagIndication> (): It shall be configurable by CanIfPublicPnSupport, if CanIf supports this ser- vice (False: not supported, True: supported)]()

[SWS_CANIF_00797] [Configuration of <User_ClearTrcvWufFlagIndication> (): If CanIfDispatchUserClearTrcvWufFlagIndicationUL is set to CAN_SM, CanIfDispatchUserClearTrcvWufFlagIndicationName must be CanSM_- ClearTrcvWufFlagIndication.]()

[SWS_CANIF_00798] [Configuration of <User_ClearTrcvWufFlagIndication> (): If CanIfDispatchUserClearTrcvWufFlagIndicationUL is set to CDD, the name of the service has to be configurable via parameter CanIfDispatchUser- ClearTrcvWufFlagIndicationName.]()

###### <User_CheckTrcvWakeFlagIndication> [SWS_CANIF_00814] [

| Service Name | <User_CheckTrcvWakeFlagIndication> |  |
| --- | --- | --- |
| Syntax | void <User_CheckTrcvWakeFlagIndication> ( uint8 TransceiverId ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId, for which this function was called. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates that the wake up flag in the CAN transceiver is set. This function is called in CanIf_CheckTrcvWakeFlagIndication. |  |
| Available via | configurable |  |

Note: This callback service is mainly implemented in and used by CanSm (see [3, Specification of CAN State Manager]).

Note: The call context of <User_CheckTrcvWakeFlagIndication>() is either on interrupt level (interrupt mode) or on task level (polling mode).

Note: The callback service <User_CheckTrcvWakeFlagIndication>() is in gen- eral re-entrant for multiple CAN Controller usage, but not for the same CAN Controller

[SWS_CANIF_00800] [Configuration of <User_CheckTrcvWakeFlagIndica- tion>(): The upper layer module, which is called (see [SWS_CANIF_00759]), has to be configurable by CanIfDispatchUserCheckTrcvWakeFlagIndicationUL if CanIfPublicPnSupport equals True.]()

[SWS_CANIF_00801] [Configuration of <User_CheckTrcvWakeFlagIndica- tion>(): The name of <User_CheckTrcvWakeFlagIndication>() shall be con- figurable by CanIfDispatchUserCheckTrcvWakeFlagIndicationName if Can- IfPublicPnSupport equals True.]()

[SWS_CANIF_00802] [Configuration of <User_CheckTrcvWakeFlagIndica- tion>(): It shall be configurable by CanIfPublicPnSupport, if CanIf supports this service (False: not supported, True: supported)]()

[SWS_CANIF_00803] [Configuration of <User_CheckTrcvWakeFlagIndica- tion>(): If CanIfDispatchUserCheckTrcvWakeFlagIndicationUL is set to CAN_SM, CanIfDispatchUserCheckTrcvWakeFlagIndicationName must be CanSM_CheckTransceiverWakeFlagIndication.]()

[SWS_CANIF_00804] [Configuration of <User_CheckTrcvWakeFlagIndica- tion>(): If CanIfDispatchUserCheckTrcvWakeFlagIndicationUL is set to CDD, the name of the service has to be configurable via parameter CanIfDis- patchUserCheckTrcvWakeFlagIndicationName.]()

###### <User_ControllerModeIndication> [SWS_CANIF_00687] [

| Service Name | <User_ControllerModeIndication> |  |
| --- | --- | --- |
| Syntax | void <User_ControllerModeIndication> ( uint8 ControllerId, Can_ControllerStateType ControllerMode ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | ControllerId | Abstracted CanIf ControllerId which is assigned to a CAN controller, at which a controller state transition occurred. |
|  | ControllerMode | Notified CAN controller mode |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates a CAN controller state transition to the corresponding upper layer module (mainly the CAN State Manager module). |  |
| Available via | configurable |  |

Note: The upper layer module provides the Service ID.

Note: This callback service is mainly implemented in and used by CanSm (see [3, Specification of CAN State Manager]).

Note: The CanIf calls this callback service. It is implemented by the configured up- per layer module. It is called in case of a state transition notification via CanIf_- ControllerModeIndication() of the CanDrv.	The delivered parameter Con- trollerId of the service CanIf_ControllerModeIndication() is passed to the upper layer module. The delivered parameter ControllerMode of the service CanIf_ControllerModeIndication() is mapped to the appropriate parameter ControllerMode of <User_ControllerModeIndication>().

Note: For different upper layer users different service names shall be used.

Note: The call context of <User_ControllerModeIndication>() is on task level (polling mode).

Note: The callback service <User_ControllerModeIndication>() is in general re-entrant for multiple CAN Controller usage, but not for the same CAN Controller

[SWS_CANIF_00689] [Configuration of <User_ControllerModeIndication>()

: The upper layer module which provides this callback service has to be configured by

CanIfDispatchUserCtrlModeIndicationUL.]()

[SWS_CANIF_00690] [Configuration of <User_ControllerModeIndication>()

: The name of <User_ControllerModeIndication>() which is called by CanIf shall be configured for CanIf by parameter CanIfDispatchUserCtrlModeIndi- cationName. This is only necessary if state transition notifications are configured via CanIfDispatchUserCtrlModeIndicationUL.]()

[SWS_CANIF_00691] [Configuration of <User_ControllerModeIndication>()

: If CanIfDispatchUserCtrlModeIndicationUL is set to CAN_SM, CanIfDis- patchUserCtrlModeIndicationName must be CanSM_ControllerModeIndi- cation.]()

[SWS_CANIF_00692] [Configuration of <User_ControllerModeIndication>()

: If CanIfDispatchUserCtrlModeIndicationUL is set to CDD the name of the function has to be configured via parameter CanIfDispatchUserCtrlModeIndi- cationName.]()

###### <User_TrcvModeIndication> [SWS_CANIF_00693] [

| Service Name | <User_TrcvModeIndication> |  |
| --- | --- | --- |
| Syntax | void <User_TrcvModeIndication> ( uint8 TransceiverId, CanTrcv_TrcvModeType TransceiverMode ) |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | TransceiverId | Abstracted CanIf TransceiverId which is assigned to a CAN transceiver, at which a transceiver state transition occurred. |
|  | TransceiverMode | Notified CAN transceiver mode |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | This service indicates a CAN transceiver state transition to the corresponding upper layer module (mainly the CAN State Manager module). |  |
| Available via | configurable |  |

Note: The upper layer module provides the Service ID.

Note: This callback service is mainly implemented in and used by CanSm (see [3, Specification of CAN State Manager]).

Note: The CanIf calls this callback service. It is implemented by the configured upper layer module. It is called in case of a state transition notification via CanIf_TrcvMod- eIndication() of the CanTrcv. The delivered parameter Transceiver of the ser- vice CanIf_TrcvModeIndication() is mapped (as configured) to the appropriate parameter TransceiverId which will be passed to the upper layer module. The de- livered parameter TransceiverMode of the service CanIf_TrcvModeIndication () is mapped to the appropriate parameter TransceiverMode of <User_TrcvMod- eIndication>().

Note: For different upper layer users different service names shall be used.

[SWS_CANIF_00694] [Caveats of <User_TrcvModeIndication>():

- The CanTrcv must be initialized after Power ON.

- The call context is either on task level (polling mode).

- This callback service is in general re-entrant for multiple CAN Transceiver us- age, but not for the same CAN Transceiver.

[SWS_CANIF_00695] [Configuration of <User_TrcvModeIndication>(): The up- per layer module which provides this callback service has to be configured by Can- IfDispatchUserTrcvModeIndicationUL, but:

- If no upper layer modules are configured for transceiver mode indications using

<User_TrcvModeIndication>(), no transceiver mode indication needs to be configured. CanIfDispatchUserTrcvModeIndicationUL needs not to be configured.

- If transceivers are not supported (CanIfTrcvDrvCfg is not configured, Can- IfDispatchUserTrcvModeIndicationUL is not configurable.

If no upper layer modules are configured for state transition notifications using <User_TrcvModeIndication>(), no state transition notification needs to be con- figured.

[SWS_CANIF_00696] [Configuration of <User_TrcvModeIndication>(): The name of <User_TrcvModeIndication>() which will be called by CanIf shall be configured for CanIf by parameter CanIfDispatchUserTrcvModeIndication- Name. This is only necessary if state transition notifications are configured via Can- IfDispatchUserTrcvModeIndicationUL.]()

[SWS_CANIF_00697] [Configuration of <User_TrcvModeIndication>(): If CanIfDispatchUserTrcvModeIndicationUL is set to CAN_SM, CanIfDis- patchUserTrcvModeIndicationName must	be	CanSM_TransceiverMod- eIndication.]()

[SWS_CANIF_00698] [Configuration of <User_TrcvModeIndication>(): If Can- IfDispatchUserTrcvModeIndicationUL is set to CDD the name of the API has to be configured via parameter CanIfDispatchUserTrcvModeIndicationName.] ()

### Sequence diagrams

The following sequence diagrams show the interactions between CanIf and CanDrv.

«Peripheral»

CanController

CanIf_Transmit(Std_ReturnType, PduIdType, const PduInfoType*)

Can_Write(Std_ReturnType, Can_HwHandleType, const Can_PduType*)

alt CAN Controller

Copy L-PDU into CAN Hardware()

[CAN controller hardware object is free]

Copy L-PDU into CAN Hardware()

Can_Write()

[CAN controller hardware object is busy]

Can_Write()

Insert L-PDU in transmit buffer()

CanIf_Transmit()

«module» Can

«module» SchM

CanIf User

«module» CanIf

Transmit request (single CAN Driver)

Figure 9.1: Transmission request with a single CAN Driver

| Activity | Description |
| --- | --- |
| Transmission request | The upper layer initiates a transmit request via the service CanIf_Transmit(). The parameter CanTxPduId identifies the requested L-SDU. The service performs following steps: - validation of the input parameter - definition of the CAN Controller to be used The second parameter *PduInfoPtr is a pointer on the structure with transmit L-SDU related data such as SduLength and *SduDataPtr. |
| Start transmission | CanIf_Transmit() requests a transmission and calls the CanDrv service Can_Write() with corresponding processing of the HTH. |
| Hardware request | Can_Write() writes all L-PDU data in the CAN Hardware (if it is free) and sets the hardware request for transmission. |
| E_OK from Can_Write service | Can_Write() returns E_OK to CanIf_Transmit(). |
| CAN_BUSY from Can_Write service | If CanDrv detects, there are no free hardware objects available, it returns CAN_BUSY to CanIf. |
| Copying into the buffer | The L-PDU of the rejected transmit request will be inserted in the transmit buffer of CanIf until the next transmit confirmation. |
| E_OK from CanIf | CanIf_Transmit() returns E_OK to the upper layer. |

#### Transmit request (multiple CAN Drivers)

Figure 9.2: Transmission request with multiple CAN Drivers

First transmit request:

| Activity | Description |
| --- | --- |
| Transmission request A | The upper layer initiates a transmit request via the service CanIf_Transmit(). The parameter CanTxPduId identifies the requested L-SDU. The service performs following steps: - validation of the input parameter - definition of the CAN Controller to be used (here: Can_99_Ext1) The second parameter *PduInfoPtr is a pointer on the structure with transmit L-SDU related data such as SduLength and *SduDataPtr. |
| Start transmission | CanIf_Transmit() requests a transmission and calls the CanDrv Can_99_Ext1 service Can_Write_99_Ext1() with corresponding processing of the HTH. |
| Hardware request | Can_Write_99_Ext1() writes all L-PDU data in the CAN Hardware of Controller A (if it is free) and sets the hardware request for transmission. |
| E_OK from Can_Write service | Can_Write_99_Ext1() returns E_OK to CanIf_Transmit(). |
| CAN_BUSY from Can_Write service | If CanDrv Can_99_Ext1 detects, there are no free hardware objects available, it returns CAN_BUSY to CanIf. |
| Copying into the buffer | The L-PDU of the rejected transmit request will be inserted in the transmit buffers of CanIf until the next transmit confirmation. |
| E_OK from CanIf | CanIf_Transmit() returns E_OK to the upper layer. |

Second transmit request:

| Activity | Description |
| --- | --- |
| Transmission request B | The upper layer initiates a transmit request via the service CanIf_Transmit(). The parameter CanTxPduId identifies the requested L-SDU. The service performs following steps: - validation of the input parameter - definition of the CAN Controller to be used (here: Can_99_Ext2) The second parameter *PduInfoPtr is a pointer on the structure with transmit L-SDU related data such as SduLength and *SduDataPtr. |
| Start transmission | CanIf_Transmit() starts a transmission and calls the CanDrv Can_99_Ext2 service Can_Write_99_Ext2() with corresponding processing of the HTH. |
| Hardware request | Can_Write_99_Ext2() writes all L-PDU data in the CAN Hardware of Controller B (if it is free) and sets the hardware request for transmission. |
| E_OK from Can_Write service | Can_Write_99_Ext2() returns E_OK to CanIf_Transmit(). |
| CAN_BUSY from Can_Write service | If CanDrv Can_99_Ext2 detects, there are no free hardware objects available, it returns CAN_BUSY to CanIf. |
| Copying into the buffer | The L-PDU of the rejected transmit request will be inserted in the transmit buffers of CanIf until the next transmit confirmation. |
| E_OK from CanIf | CanIf_Transmit() returns E_OK to the upper layer. |

#### Transmit confirmation (interrupt mode)

Figure 9.3: Transmit confirmation interrupt driven

| Activity | Description |
| --- | --- |
| Transmit interrupt | The acknowledged CAN frame signals a successful transmission to the receiving CAN Controller and triggers the transmit interrupt. |
| Confirmation to CanIf | CanDrv calls the service CanIf_TxConfirmation(). The parameter CanTxPduId specifies the L-PDU previously sent by Can_Write(). CanDrv must store the all in HTHs pending L-PDU Ids in an array organized per HTH to avoid new search of the L-PDU ID for call of CanIf_TxConfirmation(). |
| Confirmation to upper layer | Calling of the corresponding upper layer confirmation service <User_TxConfirmation>(id, E_OK). It signals a successful L-SDU transmission to the upper layer. |

#### Transmit confirmation (polling mode)

Figure 9.4: Transmit confirmation polling driven

| Activity | Description |
| --- | --- |
| Cyclic Task CanDrv | The service Can_MainFunction_Write() is called by the BSW Scheduler. |
| Check for pending transmit confirmations | Can_MainFunction_Write() checks the underlying CAN Controller(s) about pending transmit confirmations of previously succeeded transmit events. |
| Transmit Confirmation | The acknowledged CAN frame signals a successful transmission to the sending CAN Controller. |
| Confirmation to CanIf | CanDrv calls the service CanIf_TxConfirmation(). The parameter CanTxPduId specifies the L-PDU previously sent by Can_Write(). CanDrv must store the all in HTHs pending L-PDU Ids in an array organized per HTH to avoid new search of the L-PDU ID for call of CanIf_TxConfirmation(). |
| Confirmation to upper layer | Calling of the corresponding upper layer confirmation service <User_TxConfirmation>(id, E_OK). It signals a successful L-SDU transmission to the upper layer. |

#### Transmit confirmation (with buffering)

«module»

SchM

«module»

CanIf

«module»

Can

«Peripheral»

CanController

Transmit Confirmation Interrupt()

CanIf_TxConfirmation(PduIdType)

check transmit buffers for other pending L-PDU()

alt Transmit Buffering

[Buffer is filled]

Can_Write(Std_ReturnType, Can_HwHandleType, const Can_PduType*)

Write L-PDU into CAN Hardware()

Write L-PDU into CAN Hardware()

Can_Write()

Remove L-PDU successfully requested for transmission from transmit buffer()

[Buffer is empty]

<User_TxConfirmation>(PduIdType, Std_ReturnType)

<User_TxConfirmation>()

CanIf_TxConfirmation()

Transmit Confirmation Interrupt()

CanIf User

Figure 9.5: Transmit confirmation with buffering

| Activity | Description |
| --- | --- |
| Transmit interrupt | Acknowledged CAN frame signals successful transmission to receiving CAN Controller and triggers transmit interrupt. |
| Confirmation to CanIf | CanDrv calls service CanIf_TxConfirmation(). Parameter CanTxPduId specifies the L-PDU previously transmitted by Can_Write(). CanDrv must store the all in HTHs pending L-PDU Ids in an array organized per HTH to avoid new search of the L-PDU ID for call of CanIf_TxConfirmation(). |
| Check of transmit buffers | The transmit buffers of CanIf checked, whether a pending L-PDU is stored or not. |
| Transmit request passed to CanDrv | In case of pending L-PDUs in the transmit buffers the highest priority order the latest L-PDU is requested for transmission by Can_Write(). It signals a successful L-PDU transmission to the upper layer. Thus Can_Write() can be called re-entrant. |
| Remove transmitted L-PDU from transmit buffers | The L-PDU pending for transmission is removed from the transmission buffers by CanIf. |
| Confirmation to the upper layer | Calling of the corresponding upper layer confirmation service <User_TxConfirmation>(id, E_OK). It signals a successful L-SDU transmission to the upper layer. |

#### Trigger Transmit Request

CanIf_Transmit(Std_ReturnType, PduIdType, const PduInfoType*)

Can_Write(Std_ReturnType,

Can_HwHandleType, const Can_PduType*)

alt Controller HW Status

[Controller HW object free]

CanIf_TriggerTransmit(Std_ReturnType, PduIdType, PduInfoType*)

<CanIfUser>_CanIfTriggerTransmit(Std_ReturnT e, PduIdType, PduInfoType*)

Copy L-Pdu into CAN hardware

Copy L-PDU into CAN hardware

CanIf_TriggerTransmit()

Can_Write()

Can_Write()

CanIf_Transmit()

[Controller HW object busy]

yp

«module» Can

«module» CanIf

CanIfUser

«peripheral» CanController

Figure 9.6: Trigger Transmit Request

| Activity | Description |
| --- | --- |
| Transmission request | The upper layer initiates a transmit request via the service CanIf_Transmit(). The parameter CanTxPduId identifies the requested L-SDU. The service performs following steps: - validation of the input parameter - definition of the CAN Controller to be used The second parameter *PduInfoPtr is a pointer to the structure with the size (SduLength) of the L-SDU to be transmitted. The actual SDU data has not been passed by the upper layer. Hence, the pointer *SduDataPtr points to NULL. |
| Start transmission | CanIf_Transmit() requests a transmission and calls the CanDrv service Can_Write() with corresponding processing of the HTH. |
| Trigger transmission | If the CAN hardware is free Can_Write() requests the SDU data from CanIf by its service CanIf_TriggerTransmit() passing the L-SDUs corresponding ID and a pointer to the CAN hardware’s buffer. CanIf forwards the trigger transmit request to the corresponding upper layer (CanIfUser). CanIf passes the buffer pointer received by CanDrv. The CanIfUser finally copies the SDU data to the buffer provided by CanIf (the CAN hardware buffer) and returns status and number of bytes effectively written. |
| E_OK from Can_Write() service | Can_Write() returns E_OK to CanIf_Transmit(). |
| CAN_BUSY from Can_Write() service | If CanDrv detects, there are no free hardware objects available, it returns CAN_BUSY to CanIf. |
| Queuing of transmission request | The Transmit Request for the L-PDU, which has been rejected by CanDrv, is queued by CanIf until the next transmit confirmation. |
| E_OK from CanIf | CanIf_Transmit() returns E_OK to the upper layer. |

#### Receive indication (interrupt mode)

CanIf User

«module»

CanIf

«module»

Can

«Peripheral»

CanController

Receive Interrupt()

Invalidation of hardware object() Invalidation of hardware object()

alt Temporary buffer usage

[Temp. buffer used = Data normalization necessary]

Copy received L-PDU into temporary buffer()

Copy received L-PDU into temporary buffer()

[Temp. buffer not used = Data normalization not necessary]

CanIf_RxIndication(const Can_HwType*, const PduInfoType*)

Software filtering (optional) and L-PDU assignment

[CAN L-PDU ID was found]: Data Length Check (optional)

<User_RxIndication>(PduIdType, const PduInfoType*)

alt Temporary buffer usage

[Temp. buffer used = Data normalization necessary]

Copy Data()

Copy Data()

[Temp. buffer not used = Data normalization not necessary]

Copy Data()

Copy Data()

<User_RxIndication>()

CanIf_RxIndication()

Validation of hardware object()

Validation of hardware object() Receive

Interrupt()

Figure 9.7: Receive indication interrupt driven

| Activity | Description |
| --- | --- |
| Receive Interrupt | The CAN Controller indicates a successful reception and triggers a receive interrupt. |
| Invalidation of CAN hardware object, provide CPU access to CAN mailbox | The CPU (CanDrv) get exclusive access rights to the CAN mailbox or at least to the corresponding hardware object, where new data were received. |

| Buffering, normalizing | The L-PDU is normalized and is buffered in the temporary buffer located in CanDrv. Each CanDrv owns such a temporary buffer for every Physical Channel only if normalizing of the data is necessary. |
| --- | --- |
| Indication to CanIf | The reception is indicated to CanIf by calling of CanIf_RxIndication(). The HRH specifies the CAN RAM Hardware Object and the corresponding CAN Controller, which contains the received L-PDU. The temporary buffer is referenced to CanIf by PduInfoPtr->SduDataPtr. |
| Software Filtering | The Software Filtering checks, whether the received L-PDU will be processed on a local ECU. If not, the received L-PDU is not indicated to upper layers. Further processing is suppressed. |
| Data Length Check | If the L-PDU is found, the Data Length of the received L-PDU is compared with the expected, statically configured one for the received L-PDU. |
| Receive Indication to the upper layer | The corresponding receive indication service of the upper layer is called. This signals a successful reception to the target upper layer. The parameter RxPduId specifies the L-SDU, the second parameter is the reference on the temporary buffer within the L-SDU. During is execution of this service the CAN hardware buffers must be unlocked for CPU access/locked for CAN Controller access. |
| Validation of CAN hardware object, allow access of CAN Controller to CAN mailbox | The CAN Controller get back exclusive access rights to the CAN mailbox or at least to the corresponding hardware object, where new data were already being copied into the upper layer buffer. |

#### Receive indication (polling mode)

«Peripheral»

CanController

BSW Scheduler

loop Cyclic Task of Interface

Can_MainFunction_Read()

Check for pending Rx indication()

alt Pending Rx indication

[Rx indication pending]

Invalidation of hardware object()

alt Temporary buffer usage

[Temp. buffer used = Data normalization necessary]

Copy received L-PDU into temporary buffer()

[Temp. buffer not used = Data normalization not necessary]

CanIf_RxIndication(const Can_HwType*, const PduInfoType*)

Software filtering (optional) and L-PDU assignment

«module»

CanIf

<User_RxIndication>(PduIdType, const PduInfoType*)

alt Temporary buffer usage

[Temp. buffer used = Data normalization necessary]

Copy data()

[Temp. buffer not used = Data normalization not necessary]

Copy data()

<User_RxIndication>()

CanIf_RxIndication()

Validation of hardware object()

[No Rx indication pending]

Can_MainFunction_Read()

CanIf User

«module»

Can

[CAN L-PDU ID was found]: Data Length Check (optional)

Figure 9.8: Receive indication polling driven

| Activity | Description |
| --- | --- |
| Cyclic Task CanDrv | The service Can_MainFunction_Read() is called by the BSW Scheduler. |
| Check for new received L-PDU | Can_MainFunction_Read() checks the underlying CAN Controller(s) about new received L-PDUs. |
| Invalidation of CAN hardware object, provide CPU access to CAN mailbox | In case of a new receive event the CPU (CanDrv) get exclusive access rights to the CAN mailbox or at least to the corresponding hardware object, where new data were received. |
| Buffering, normalizing | In case of a new receive event the L-PDU is normalized and is buffered in the temporary buffer located in CanDrv. Each CanDrv owns such a temporary buffer for every Physical Channel only if normalizing of the data is necessary. |
| Indication to CanIf | The reception is indicated to CanIf by calling of CanIf_RxIndication(). The HRH specifies the CAN RAM Hardware Object and the corresponding CAN Controller, which contains the received L-PDU. The temporary buffer is referenced to CanIf by PduInfoPtr->SduDataPtr. |
| Software Filtering | The Software Filtering checks, whether the received L-PDU will be processed on a local ECU. If not, the received L-PDU is not indicated to upper layers. Further processing is suppressed. |
| Data Length Check | If the L-PDU is found, the Data Length of the received L-PDU is compared with the expected, statically configured one for the received L-PDU. |
| Receive Indication to the upper layer | If configured, the corresponding receive indication service of the upper layer is called. This signals a successful reception to the target upper layer. The parameter RxPduId specifies the L-SDU, the second parameter is the reference on the temporary buffer within the L-SDU. During is execution of this service the CAN hardware buffers must be unlocked for CPU access/locked for CAN Controller access. |
| Validation of CAN hardware object, allow access of CAN Controller to CAN mailbox | The CAN Controller get back exclusive access rights to the CAN mailbox or at least to the corresponding hardware object, where new data were already being copied into the upper layer buffer. |

#### Read received data

CanIf User

«module»

Can

«Peripheral»

CanController

Receive Interrupt()

Invalidation of hardware object()

Invalidation of hardware object()

CanIf_RxIndication(const Can_HwType*, const PduInfoType*)

[L-PDU reception in BasicCAN]: Software filtering and L-PDU assignment()

[CAN L-PDU ID was found]:

Data Length Check()

Copy data to CANIF receive L-PDU buffer()

Copy data to CANIF receive L-PDU buffer()

Set Indication Flag()

<User_RxIndication>(PduIdType, const PduInfoType*)

<User_RxIndication>()

CanIf_RxIndication()

Validation of hardware object()

Validation of hardware object()

Receive Interrupt()

CanIf_ReadRxNotifStatus(CanIf_NotifStatusType, PduIdType)

Read Indication flag()

Reset Indication flag()

CanIf_ReadRxNotifStatus()

CanIf_ReadRxPduData(Std_ReturnType, PduIdType, PduInfoType**)

Read data from CANIF Rx buffer()

CanIf_ReadRxPduData()

«module» CanIf

«module» SchM

Figure 9.9: Read received data

| Activity | Description |
| --- | --- |
| Receive Interrupt | The CAN Controller indicates a successful reception and triggers a receive interrupt. |
| Invalidation of CAN hardware object, provide CPU access to CAN mailbox | The CPU (CanDrv) get exclusive access rights to the CAN mailbox or at least to the corresponding hardware object, where new data were received. |

| Buffering, normalizing | The L-PDU is normalized and is buffered in the temporary buffer located in CanDrv. Each CanDrv owns such a temporary buffer for every Physical Channel only if normalizing of the data is necessary. |
| --- | --- |
| Indication to CanIf | The reception is indicated to CanIf by calling of CanIf_RxIndication(). The HRH specifies the CAN RAM Hardware Object and the corresponding CAN Controller, which contains the received L-PDU. The temporary buffer is referenced to CanIf by PduInfoPtr->SduDataPtr. |
| Software Filtering | The Software Filtering checks, whether the received L-PDU will be processed on a local ECU. If not, the received L-PDU is not indicated to upper layers. Further processing is suppressed. |
| Data Length Check | If the L-PDU is found, the Data Length of the received L-PDU is compared with the expected, statically configured one for the received L-PDU. |
| Copy data | The data is copied out of the CAN hardware into the receive CAN L-PDU buffers in CanIf. During access the CAN hardware buffers must be unlocked for CPU access/locked for CAN Controller access. |
| Indication Flag | Set indication status flag for the received L-PDU in CanIf. |
| Receive Indication to the upper layer | The corresponding receive indication service of the upper layer is called. This signals a successful reception to the target upper layer. The parameter RxPduId specifies the L-SDU, the second parameter is the reference on the temporary buffer within the L-SDU. |
| Validation of CAN hardware object, allow access of CAN Controller to CAN mailbox | The CAN Controller get back exclusive access rights to the CAN mailbox or at least to the corresponding hardware object, where new data were already being copied into the upper layer buffer. |
| Read indication status | Times later the upper layer can read the indication status by call of CanIf_ReadRxNotifStatus(). This service can also be used for transmit L-PDUs. Then it return the confirmation status. |
| Reset indication status | Before CanIf_ReadRxNotifStatus() returns, the indication status is reset. |
| Read received data | Times later the upper layer can read the received data by call of CanIf_ReadRxPduData(). |
| Read CanIf Rx buffer | CanIf_ReadRxPduData() reads the data from CanIf Rx buffer. |
| E_OK from CanIf | If CanIf_ReadRxPduData() was successful, the request returns E_OK with valid PduInfoPtr. |

#### Start CAN network

CanIf User

«module»

CanIf

«module»

Can

«Peripheral»

CanController

loop Requesting CAN controller mode consecutively. If mode changed -> CanIf_ControllerModeIndication()

Can_MainFunction_Mode()

CanIf_SetControllerMode(Std_ReturnType, uint8, Can_ControllerStateType)

Can_SetControllerMode(Std_ReturnType, uint8, Can_ControllerStateType)

Disable Wakeup

interrupt, if supported()

request CAN controller mode transition to START()

alt CAN Controller Mode

[STOPPED]  CanIf_SetControllerMode returns with E_OK()

Can_SetControllerMode returns with E_OK()

CAN controller mode changes to START

CanIf_ControllerModeIndication(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>()

CanIf_ControllerModeIndication()

[STOPPED with direct indication]

CanIf_ControllerModeIndication(Controller, ControllerMode)

<User_ControllerModeIndication>(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>()

CanIf_ControllerModeIndication()

Can_SetControllerMode returns with E_OK()

CanIf_ControllerMode returns with E_OK()

[STARTED]

CanIf_ControllerModeIndication(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>()

CanIf_ControllerModeIndication()

Can_SetControllerMode returns with E_OK()

CanIf_SetControllerMode returns with E_OK()

[SLEEP]

Can_SetControllerMode returns with E_NOT_OK()

CanIf_SetControllerMode returns with E_NOT_OK()

Figure 9.10: Start CAN network

This sequence diagram resembles "Stop CAN network" or "Sleep CAN network".

| Activity | Description |
| --- | --- |
| Loop requesting CAN controller mode consecutively. | The Can_MainFunction_Mode() is triggered consecutively. It checks the HW if a controller mode has changed. If so, it is notified via a function call of CanIf_ControllerModeIndication (Controller, ControllerMode). |
| The upper layer requests " STARTED" mode of the desired CAN controller | The upper layer calls CanIf_SetControllerMode (ControllerId, CAN_CS_STARTED) to request STARTED mode for the requested CAN controller. |
| CanDrv disables wake up interrupts, if supported | This is only done in case of requesting "STARTED" mode. If " SLEEP" mode of CAN controller is requested, here the wake up interrupts are enabled. In case of "STOPPED", nothing happens. |
| CanDrv requests the CAN controller to transition into the requested mode ( CAN_CS_STARTED). | During function call Can_SetControllerMode(Controller, Can_ControllerStateType), the CanDrv enters the request into the hardware of the CAN controller. This may mean that the controller mode transitions directly, but it could mean that it takes a few milliseconds until the controller changes its state. It depends on the controllers. |
| The following reaction depends on the controller and its current operation mode |  |
| CAN controller was in STOPPED mode | The former request Can_SetControllerMode() returns and informs CanIf about a successful request which in turn returns the upper layer request CanIf_SetControllerMode(). The Can_MainFunction_Mode() detects the successful mode transition of the CAN controller and inform the CanIf asynchronously via CanIf_ControllerModeIndication (Controller, CAN_CS_STARTED). |
| CAN controller was in STOPPED mode and the CAN controller transitions very fast so that mode indication is called during transition request | During the former request Can_SetControllerMode() the function CanIf_ControllerModeIndication(Controller, CAN_CS_STARTED) is called to inform the CanIf directly about the successful mode transition. When CanIf_ControllerModeIndication(Controller, CAN_CS_STARTED) returned, the request Can_SetControllerMode() returns and informs CanIf about a successful request which in turn returns the upper layer request CanIf_SetControllerMode(). |
| CAN controller was in STARTED mode | During the former request Can_SetControllerMode() the function CanIf_ControllerModeIndication(Controller, CAN_CS_STARTED) is called to inform the CanIf directly about the successful mode transition (because the mode was already started). When CanIf_ControllerModeIndication (Controller, CAN_CS_STARTED) returned, the request Can_SetControllerMode() returns and informs CanIf about a successful request which in turn returns the upper layer request CanIf_SetControllerMode(). |
| CAN controller was in SLEEP mode | This transition is not allowed -> E_NOT_OK. |

«module»

Can

BusOff Detection()

Set CAN Controller to STOPPED mode, if necessary()

Set CAN Controller to STOPPED mode, if necessary()

CanIf_ControllerBusOff(uint8)

<User_ControllerBusOff>(uint8)

Reset transmit queue()

<User_ControllerBusOff>()

CanIf_ControllerBusOff()

BusOff Detection()

«Peripheral» CanController

«module» CanIf

<User_ControllerBusOff>

BusOff notification

Figure 9.11: BusOff notification

| Activity | Description |
| --- | --- |
| BusOff detection interrupt | The CAN controller signals a BusOff event. |
| Stop CAN controller | CAN controller is set to STOPPED mode by the CAN Driver, if necessary. |
| BusOff indication to CAN Interface | BusOff is notified to the CanIf by calling of CanIf_ControllerBusOff() |
| BusOff indication to upper layer (CanSM) | BusOff is notified to the upper layer by calling of <User_ControllerBusOff>() |

<User_ControllerBusOff>

«module»

CanIf

«module»

Can

«Peripheral»

CanController

loop Requesting CAN controller mode consecutively. If mode changed -> CanIf_ControllerModeIndication().

Can_MainFunction_Mode()

opt CAN controller

[BUSSOFF]

BusOff Detection()

Set CAN controller to STOPPED mode, if necessary() Set CAN controller to STOPPED mode, if necessary()

[STOPPED]

CanIf_ControllerBusOff(uint8)

Reset transmit queue()

<User_ControllerBusOff>(uint8)

<User_ControllerBusOff>()

CanIf_ControllerBusOff()

BusOff Detection()

CanIf_SetControllerMode(Std_ReturnType, uint8, Can_ControllerStateType)

Can_SetControllerMode(Std_ReturnType, uint8, Can_ControllerStateType)

Reset CAN controller, if necessary()

request CAN controller mode transition to START()

Can_SetContollerMode()

CanIf_SetControllerMode()

CanIf_ControllerModeIndication(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>(uint8, Can_ControllerStateType)

<User_ControllerModeIndication>()

CanIf_ControllerModeIndication()

BusOff recovery

Figure 9.12: BusOff recovery

| Activity | Description |
| --- | --- |
| BusOff detection interrupt | The CAN controller signals a BusOff event. |
| Stop CAN controller | CAN controller is set to STOPPED mode by the CanDrv, if necessary |
| BusOff indication to CanIf | BusOff is notified to the CanIf by calling of CanIf_ControllerBusOff(). The transmit buffers inside CanIf will be reset. |
| BusOff indication to upper layer | BusOff is notified to the upper layer by calling of <User_ControllerBusOff>() |
| Upper Layer (CanSM) initiates BusOff Recovery | After a time specified by the BusOff Recovery algorithm the Recovery process itself in initiated by CanIf_SetControllerMode(ControllerId, CAN_CS_STARTED). |
| Restart of CAN controller | The driver restarts the CAN controller by call of Can_SetControllerMode(Controller, CAN_CS_STARTED). |
| CAN controller started | CanDrv informs CanIf about the successful start by calling CanIf_ControllerModeIndication(). CanIf informs in turn upper layers about the mode change. |

### Configuration specification

In general, this chapter defines configuration parameters and their clustering into containers. For general information about the definition of containers and param- eters, refer to the [9, chapter 10.1 "Introduction to configuration specification" in SWS_BSWGeneral].

section 10.1 specifies the structure (containers) and the parameters of the CanIf.

#### Containers and configuration parameters

The following chapters summarize all configuration parameters. The detailed meanings of the parameters describe chapter 7 “Functional specification” and chapter 8 “API specification”.

The listed configuration items can be derived from a network description database, which is based on the EcuConfigurationTemplate. The configuration tool shall extract all information to configure the CanIf.

The CanIf has access to the CanDrv configuration data. All public CanDrv configura- tion data are described in [1, Specification of CAN Driver].

+container

+container

+subContainer

+subContainer

+subContainer

+container

+subContainer

+container

+container

+subContainer

lowerMultiplicity = 1 upperMultiplicity = *

lowerMultiplicity = 0 upperMultiplicity = *

CanIfTrcvCfg:  EcucParamConfContainerDef

CanIfTrcvDrvCfg:  EcucParamConfContainerDef

lowerMultiplicity = 1 upperMultiplicity = *

upperMultiplicity = * lowerMultiplicity = 1

CanIfCtrlDrvCfg:  EcucParamConfContainerDef

CanIfCtrlCfg: EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

lowerMultiplicity = 0 upperMultiplicity = *

CanIfDispatchCfg:  EcucParamConfContainerDef

CanIfHrhRangeCfg:  EcucParamConfContainerDef

CanIfTxPduCfg:  EcucParamConfContainerDef

+subContainer

+subContainer

lowerMultiplicity = 0 upperMultiplicity = *

lowerMultiplicity = 0 upperMultiplicity = *

CanIfRxPduCfg:  EcucParamConfContainerDef

lowerMultiplicity = 1 upperMultiplicity = 1

CanIfHrhCfg:  EcucParamConfContainerDef

+subContainer

CanIfInitCfg: EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

upperMultiplicity = 1 lowerMultiplicity = 1

+container

lowerMultiplicity = 0 upperMultiplicity = *

CanIfInitHohCfg:  EcucParamConfContainerDef

CanIfPublicCfg:  EcucParamConfContainerDef

CanIfHthCfg:  EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

CanIfPrivateCfg:  EcucParamConfContainerDef

CanIf: EcucModuleDef

Figure 10.1: Overview about CAN Interface configuration containers

##### CanIf

| Module SWS Item | ECUC_CanIf_00244 |  |
| --- | --- | --- |
| Module Name | CanIf |  |
| Module Description | This container includes all necessary configuration sub-containers according the CAN Interface configuration structure. |  |
| Post-Build Variant Support | true |  |
| Supported Config Variants | VARIANT-LINK-TIME, VARIANT-POST-BUILD, VARIANT-PRE- COMPILE |  |
| Included Containers |  |  |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfCtrlDrvCfg | 1..* | Configuration parameters for all the underlying CAN Driver modules are aggregated under this container. For each CAN Driver module a seperate instance of this container has to be provided. |
| CanIfDispatchCfg | 1 | Callback functions provided by upper layer modules of the CanIf. The callback functions defined in this container are common to all configured CAN Driver / CAN Transceiver Driver modules. |

| Container Name | Multiplicity | Scope / Dependency |
| --- | --- | --- |
| CanIfInitCfg | 1 | This container contains the init parameters of the CAN Interface. |
| CanIfPrivateCfg | 1 | This container contains the private configuration (parameters) of the CAN Interface. |
| CanIfPublicCfg | 1 | This container contains the public configuration (parameters) of the CAN Interface. |
| CanIfTrcvDrvCfg | 0..* | This container contains the configuration (parameters) of all addressed CAN transceivers by each underlying CAN Transceiver Driver module. For each CAN transceiver Driver a seperate instance of this container shall be provided. |

+container

+container

+container

+subContaine

+subContainer

+container

lowerMultiplicity = 0 upperMultiplicity = *

CanIfBufferCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+subContainer

CanIfRxPduCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+subContainer

CanIfTxPduCfg:  EcucParamConfContainerDef

+subContainer

lowerMultiplicity = 0 upperMultiplicity = *

lowerMultiplicity = 1 upperMultiplicity = 1

CanIfInitHohCfg:  EcucParamConfContainerDef

CanIfInitCfg: EcucParamConfContainerDef

upperMultiplicity = * lowerMultiplicity = 1

lowerMultiplicity = 1 upperMultiplicity = *

CanIfCtrlCfg:  EcucParamConfContainerDef

r

CanIfCtrlDrvCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+container

CanIfTrcvDrvCfg:  EcucParamConfContainerDef

CanIfDispatchCfg:  EcucParamConfContainerDef

+container

CanIfPrivateCfg:  EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 1

upperMultiplicity = 1 lowerMultiplicity = 0

CanIfPublicCfg:  EcucParamConfContainerDef

CanIf: EcucModuleDef

Figure 10.2: AR_EcucDef_CanIf

##### CanIfPrivateCfg

| SWS Item | [ECUC_CanIf_00245] |
| --- | --- |
| Container Name | CanIfPrivateCfg |
| Parent Container | CanIf |
| Description | This container contains the private configuration (parameters) of the CAN Interface. |
| Configuration Parameters |  |

| Name | CanIfFixedBuffer [ECUC_CanIf_00827] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPrivateCfg |  |  |
| Description | This parameter defines if the buffer element length shall be fixed to 8 Bytes for buffers to which only PDUs < 8 Bytes are assigned. TRUE: Minimum buffer element length is fixed to 8 Bytes. FALSE: Buffer element length depends on the configured length of the referenced global PDUs (see ECUC_EcuC_00078). |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfPrivateDataLengthCheck [ECUC_CanIf_00617] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPrivateCfg |  |  |
| Description | Selects whether Data Length Check is supported. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | true |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfPrivateSoftwareFilterType [ECUC_CanIf_00619] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPrivateCfg |  |  |
| Description | Selects the desired software filter mechanism for reception only. Each implemented software filtering method is identified by this enumeration number. Range: Types implemented software filtering methods |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | BINARY | Selects Binary Filter method. |  |
|  | INDEX | Selects Index Filter method. |  |
|  | LINEAR | Selects Linear Filter method. |  |
|  | TABLE | Selects Table Filter method. |  |
|  | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local dependency: BasicCAN reception must be enabled by referenced parameter CanHandleType of the CAN Driver module via CanIfHrhIdSymRef for at least one HRH. |  |  |

| Name | CanIfSupportTTCAN [ECUC_CanIf_00675] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPrivateCfg |  |  |
| Description | Defines whether TTCAN is supported. TRUE: TTCAN is supported. FALSE: TTCAN is not supported, only normal CAN communication is possible. |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfTTGeneral | 0..1 | CanIfTTGeneral is specified in the SWS TTCAN Interface and defines if and in which way TTCAN is supported. This container is only included and valid if TTCAN is supported by the controller, enabled (see CanIfSupportTTCAN, ECUC_CanIf_00675), and used. |

+container

upperMultiplicity = 1 lowerMultiplicity = 0

CanIf: EcucModuleDef

+parameter

defaultValue = True

CanIfPrivateDataLengthCheck:  EcucBooleanParamDef

CanIfPrivateSoftwareFilterType:  EcucEnumerationParamDef

CanIfPrivateCfg:  EcucParamConfContainerDef

+parameter

INDEX:

EcucEnumerationLiteralDef

TABLE:

EcucEnumerationLiteralDef

+literal

BINARY:

EcucEnumerationLiteralDef

LINEAR:

EcucEnumerationLiteralDef

+literal

+literal

+literal

+parameter

defaultValue = false lowerMultiplicity = 1 upperMultiplicity = 1

CanIfFixedBuffer:  EcucBooleanParamDef

Figure 10.3: AR_EcucDef_CanIfPrivateCfg

##### CanIfPublicCfg

| SWS Item | [ECUC_CanIf_00246] |
| --- | --- |
| Container Name | CanIfPublicCfg |
| Parent Container | CanIf |
| Description | This container contains the public configuration (parameters) of the CAN Interface. |
| Configuration Parameters |  |

| Name | CanIfBusMirroringSupport [ECUC_CanIf_00847] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enable support for Bus Mirroring. |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |

| Scope / Dependency | scope: local |
| --- | --- |

| Name | CanIfDevErrorDetect [ECUC_CanIf_00614] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Switches the development error detection and notification on or off. - true: detection and notification is enabled. - false: detection and notification is disabled. |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfEnableSecurityEventReporting [ECUC_CanIf_00848] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Switches the reporting of security events to the IdsM: - true: reporting is enabled. - false: reporting is disabled. Tags: atp.Status=draft |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfGlobalTimeSupport [ECUC_CanIf_00854] |
| --- | --- |
| Parent Container | CanIfPublicCfg |
| Description | Enables/Disables the Global Time APIs used when hardware timestamping is supported. Tags: atp.Status=draft |
| Multiplicity | 1 |
| Type Default Value | EcucBooleanParamDef |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfMetaDataSupport [ECUC_CanIf_00824] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enable support for dynamic ID handling using L-SDU MetaData. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicCddHeaderFile [ECUC_CanIf_00671] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Defines header files for callback functions which shall be included in case of CDDs. Range of characters is 1.. 32. |  |  |
| Multiplicity | 0..* |  |  |
| Type Default Value | EcucStringParamDef |  |  |
| Length | 1–32 |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |

| Scope / Dependency | scope: ECU |
| --- | --- |

| Name | CanIfPublicHandleTypeEnum [ECUC_CanIf_00742] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | This parameter is used to configure the Can_HwHandleType. The Can_HwHandleType represents the hardware object handles of a CAN hardware unit. For CAN hardware units with more than 255 HW objects the extended range shall be used (UINT16). |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | UINT16 |  |  |
|  | UINT8 |  |  |
|  | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: Can_HwHandleType |  |  |

| Name | CanIfPublicMultipleDrvSupport [ECUC_CanIf_00612] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Selects support for multiple CAN Drivers. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | true |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicPnSupport [ECUC_CanIf_00772] |
| --- | --- |
| Parent Container | CanIfPublicCfg |
| Description | Selects support of Partial Network features in CanIf. True: Enabled False: Disabled |
| Multiplicity | 1 |
| Type Default Value | EcucBooleanParamDef |
|  | false |
| Post-Build Variant Value | false |

| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
| --- | --- | --- | --- |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicReadRxPduDataApi [ECUC_CanIf_00607] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables / Disables the API CanIf_ReadRxPduData() for reading received L-SDU data. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicReadRxPduNotifyStatusApi [ECUC_CanIf_00608] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables and disables the API for reading the notification status of receive L-PDUs. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicReadTxPduNotifyStatusApi [ECUC_CanIf_00609] |
| --- | --- |
| Parent Container | CanIfPublicCfg |
| Description | Enables and disables the API for reading the notification status of transmit L-PDUs. True: Enabled False: Disabled |
| Multiplicity | 1 |
| Type Default Value | EcucBooleanParamDef |
|  | false |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicSetDynamicTxIdApi [ECUC_CanIf_00610] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables and disables the API for reconfiguration of the CAN Identifier for each Transmit L-PDU. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicTxBuffering [ECUC_CanIf_00618] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables and disables the buffering of transmit L-PDUs (rejected by the CanDrv) within the CAN Interface module. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfPublicTxConfirmPollingSupport [ECUC_CanIf_00733] |
| --- | --- |
| Parent Container | CanIfPublicCfg |
| Description | Configuration parameter to enable/disable the API to poll for Tx Confirmation state. |
| Multiplicity | 1 |
| Type Default Value | EcucBooleanParamDef |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local dependency: CAN State Manager module |  |  |

| Name | CanIfPublicWakeupCheckValidByNM [ECUC_CanIf_00741] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | If enabled, only NM messages shall validate a detected wake-up event in CanIf. If disabled, all received messages corresponding to a configured Rx PDU shall validate such a wake-up event. This parameter depends on CanIfPublicWakeupCheckValidSupport and shall only be configurable, if it is enabled. True: Enabled False: Disabled |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfPublicWakeupCheckValidSupport |  |  |

| Name | CanIfPublicWakeupCheckValidSupport [ECUC_CanIf_00611] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Selects support for wake up validation True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |

| Scope / Dependency | scope: ECU |
| --- | --- |

| Name | CanIfSetBaudrateApi [ECUC_CanIf_00838] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Configuration parameter to enable/disable the CanIf_SetBaudrate API to change the baud rate of a CAN Controller. If this parameter is set to true the CanIf_SetBaudrate API shall be supported. Otherwise the API is not supported. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTriggerTransmitSupport [ECUC_CanIf_00844] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables the CanIf_TriggerTransmit API at Pre-Compile-Time. Therefore, this parameter defines if there shall be support for trigger transmit transmissions. TRUE: Enabled FALSE: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | true |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxOfflineActiveSupport [ECUC_CanIf_00837] |
| --- | --- |
| Parent Container | CanIfPublicCfg |
| Description | Determines wether TxOffLineActive feature (see SWS_CANIF_00072) is supported by CanIf. True: Enabled False: Disabled |
| Multiplicity | 1 |
| Type Default Value | EcucBooleanParamDef |
|  | false |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfVersionInfoApi [ECUC_CanIf_00613] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables and disables the API for reading the version information about the CAN Interface. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfWakeupSupport [ECUC_CanIf_00843] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Enables the CanIf_CheckWakeup API at Pre-Compile-Time. Therefore, this parameter defines if there shall be support for wake-up. TRUE: Enabled FALSE: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | true |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfSecurityEventRefs | 0..1 | Container for the references to IdsMEvent elements representing the security events that the CanIf module shall report to the IdsM in case the coresponding security related event occurs (and if CanIfEnableSecurityEventReporting is set to "true"). The standardized security events in this container can be extended by vendor-specific security events. Tags: atp.Status=draft |

upperMultiplicity = 1 lowerMultiplicity = 1

defaultValue = false

+parameter

CanIfPublicReadRxPduNotifyStatusApi:  EcucBooleanParamDef

+parameter CanIfPublicReadTxPduNotifyStatusApi:

EcucBooleanParamDef

defaultValue = False

defaultValue = False

CanIfPublicSetDynamicTxIdApi:

+parameter	EcucBooleanParamDef

defaultValue = False

+parameter CanIfPublicWakeupCheckValidSupport:

EcucBooleanParamDef

defaultValue = False

+parameter

CanIfPublicMultipleDrvSupport:  EcucBooleanParamDef

defaultValue = True

+parameter

CanIfVersionInfoApi:

EcucBooleanParamDef

defaultValue = false

CanIfDevErrorDetect:  EcucBooleanParamDef

+parameter

defaultValue = false

+parameter CanIfPublicTxConfirmPollingSupport:

EcucBooleanParamDef

CanIfPublicTxBuffering:

+parameter	EcucBooleanParamDef

+parameter CanIfTriggerTransmitSupport:

EcucBooleanParamDef

defaultValue = False

defaultValue = true

+parameter

CanIfTxOfflineActiveSupport:

EcucBooleanParamDef

defaultValue = False

CanIfSetBaudrateApi:  EcucBooleanParamDef

+parameter

defaultValue = False lowerMultiplicity = 0 upperMultiplicity = 1

CanIfPublicWakeupCheckValidByNM:  EcucBooleanParamDef

+parameter

defaultValue = false lowerMultiplicity = 0 upperMultiplicity = 1

+parameter CanIfBusMirroringSupport:

EcucBooleanParamDef

defaultValue = false

+parameter

+parameter

CanIfPublicPnSupport:  EcucBooleanParamDef

CanIfPublicCddHeaderFile:

EcucStringParamDef

minLength = 1 maxLength = 32 lowerMultiplicity = 0 upperMultiplicity = *

defaultValue = false

+parameter

+parameter

CanIfMetaDataSupport:

EcucBooleanParamDef

defaultValue = false lowerMultiplicity = 0 upperMultiplicity = 1

+literal

+parameter

CanIfPublicHandleTypeEnum:  EcucEnumerationParamDef

UINT 8: EcucEnumerationLiteralDef

+literal

UINT16: EcucEnumerationLiteralDef

+parameter CanIfEnableSecurityEventReporting:

EcucBooleanParamDef

defaultValue = false

+parameter

CanIfGlobalTimeSupport:  EcucBooleanParamDef

+subContainer

CanIfSecurityEventRefs:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfPublicCfg:  EcucParamConfContainerDef

+parameter  CanIfPublicReadRxPduDataApi:

EcucBooleanParamDef

defaultValue = true

CanIfWakeupSupport:  EcucBooleanParamDef

Figure 10.4: AR_EcucDef_CanIfPublicCfg

##### CanIfInitCfg

| SWS Item | [ECUC_CanIf_00247] |
| --- | --- |
| Container Name | CanIfInitCfg |

| Parent Container | CanIf |
| --- | --- |
| Description | This container contains the init parameters of the CAN Interface. |
| Configuration Parameters |  |

| Name | CanIfInitCfgSet [ECUC_CanIf_00623] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfInitCfg |  |  |
| Description | Selects the CAN Interface specific configuration setup. This type of the external data structure shall contain the post build initialization data for the CAN Interface for all underlying CAN Dirvers. constant to CanIf_ConfigType |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucStringParamDef |  |  |
| Length | 1–32 |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfMaxBufferSize [ECUC_CanIf_00828] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfInitCfg |  |  |
| Description | Maximum total size of all Tx buffers. This parameter is needed only in case of post-build loadable implementation using static memory allocation. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 18446744073709551615 |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfMaxRxPduCfg [ECUC_CanIf_00830] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfInitCfg |  |  |
| Description | Maximum number of Pdus. This parameter is needed only in case of post-build loadable implementation using static memory allocation. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 18446744073709551615 |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfMaxTxPduCfg [ECUC_CanIf_00829] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfInitCfg |  |  |
| Description | Maximum number of Pdus. This parameter is needed only in case of post-build loadable implementation using static memory allocation. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 18446744073709551615 |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfBufferCfg | 0..* | This container contains the Txbuffer configuration. Multiple buffers with different sizes could be configured. If CanIfBufferSize (ECUC_CanIf_00834) equals 0, the CanIf Tx L-PDU only refers via this CanIfBufferCfg the corresponding CanIfHthCfg. |
| CanIfInitHohCfg | 0..* | This container contains the references to the configuration setup of each underlying CAN Driver. |
| CanIfRxPduCfg | 0..* | This container contains the configuration (parameters) of each receive CAN L-PDU. The SHORT-NAME of "CanIfRxPduConfig" container itself represents the symolic name of Receive L-PDU. This L-SDU produces a meta data item of type CAN_ID_32. |
| CanIfTxPduCfg | 0..* | This container contains the configuration (parameters) of a transmit CAN L-PDU. It has to be configured as often as a transmit CAN L-PDU is needed. The SHORT-NAME of "CanIfTxPduConfig" container represents the symolic name of Transmit L-PDU. This L-SDU consumes a meta data item of type CAN_ID_32. |

+container

+parameter

+parameter

+parameter

+parameter

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfMaxRxPduCfg:  EcucIntegerParamDef

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfMaxTxPduCfg:  EcucIntegerParamDef

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfMaxBufferSize:  EcucIntegerParamDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfTxPduCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+subContainer

CanIfInitHohCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+subContainer

CanIfRxPduCfg:  EcucParamConfContainerDef

+subContainer

minLength = 1 maxLength = 32

lowerMultiplicity = 1 upperMultiplicity = 1

CanIfInitCfgSet: EcucStringParamDef

CanIfInitCfg: EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

CanIf: EcucModuleDef

Figure 10.5: AR_EcucDef_CanIfInitCfg

##### CanIfTxPduCfg

| SWS Item | [ECUC_CanIf_00248] |
| --- | --- |
| Container Name | CanIfTxPduCfg |
| Parent Container | CanIfInitCfg |
| Description | This container contains the configuration (parameters) of a transmit CAN L-PDU. It has to be configured as often as a transmit CAN L-PDU is needed. The SHORT-NAME of "CanIfTxPduConfig" container represents the symolic name of Transmit L-PDU. This L-SDU consumes a meta data item of type CAN_ID_32. |
| Post-Build Variant Multiplicity | true |

| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
| --- | --- | --- | --- |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Configuration Parameters |  |  |  |

| Name | CanIfTxPduCanId [ECUC_CanIf_00592] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | CAN Identifier of transmit CAN L-PDUs used by the CAN Driver for CAN L-PDU transmission. Range: 11 Bit For Standard CAN Identifier ... 29 Bit For Extended CAN identifier The CAN Identifier may be omitted for dynamic transmit L-PDUs. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduCanIdMask [ECUC_CanIf_00823] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Identifier mask which denotes relevant bits in the CAN Identifier. This parameter may be used to keep parts of the CAN Identifier of dynamic transmit L-PDUs static. Range: 11 bits for Standard CAN Identifier, 29 bits for Extended CAN Identifier. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 3758096383 |  |  |
|  | 3758096383 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |

| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
| --- | --- | --- | --- |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduCanIdType [ECUC_CanIf_00590] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Type of CAN Identifier of the transmit CAN L-PDU used by the CAN Driver module for CAN L-PDU transmission. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | EXTENDED_CAN | CAN frame with extended identifier (29 bits) |  |
|  | EXTENDED_FD_CAN | CAN FD frame with extended identifier (29 bits) |  |
|  | STANDARD_CAN | CAN frame with standard identifier (11 bits) |  |
|  | STANDARD_FD_CAN | CAN FD frame with standard identifier (11 bits) |  |
|  | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduId [ECUC_CanIf_00591] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | ECU wide unique, symbolic handle for transmit CAN L-SDU. Range: 0..max. number of CantTxPduIds |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |  |
| Range Default Value | 0 .. 4294967295 |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduPnFilterPdu [ECUC_CanIf_00773] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | If CanIfPublicPnFilterSupport is enabled, by this parameter PDUs could be configured which will pass the CanIfPnFilter. If there is no CanIfTxPduPnFilterPdu configured per controller, the corresponding controller applies no CanIfPnFilter. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local dependency: This parameter shall only be configurable if CanIfPublicPnSupport equals True. |  |  |

| Name | CanIfTxPduReadNotifyStatus [ECUC_CanIf_00589] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Enables and disables transmit confirmation for each transmit CAN L-SDU for reading its notification status. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local dependency: CanIfPublicReadTxPduNotifyStatusApi must be enabled. |  |  |

| Name | CanIfTxPduTriggerTransmit [ECUC_CanIf_00840] |
| --- | --- |
| Parent Container | CanIfTxPduCfg |
| Description | Determines if or if not CanIf shall use the trigger transmit API for this PDU. |
| Multiplicity | 0..1 |
| Type Default Value | EcucBooleanParamDef |
|  | false |

| Post-Build Variant Value | true |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU dependency: If CanIfTxPduTriggerTransmit is TRUE then CanIfTxPduUserTxConfirmationUL has to be either PDUR or CDD and CanIfTxPduUserTriggerTransmitName has to be specified accordingly. |  |  |

| Name | CanIfTxPduTruncation [ECUC_CanIf_00845] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Enables/disables truncation of PDUs that exceed the configured size. |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | true |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduType [ECUC_CanIf_00593] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Defines the type of each transmit CAN L-PDU. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | DYNAMIC | CAN ID is defined at runtime. |  |
|  | STATIC | CAN ID is defined at compile-time. |  |
|  | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduUserTriggerTransmitName [ECUC_CanIf_00842] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | This parameter defines the name of the <User_TriggerTransmit>. This parameter depends on the parameter CanIfTxPduUserTxConfirmationUL. If CanIfTxPduUserTxConfirmationUL equals CAN_TP, CAN_NM, PDUR, XCP, CAN_TSYN, J1939NM or J1939TP, the name of the <User_TriggerTransmit> is fixed. If CanIfTxPduUserTxConfirmationUL equals CDD, the name of the <User_TxConfirmation> is selectable. Please be aware that this parameter depends on the same parameter as CanIfTxPduUserTxConfirmationName. It shall be clear which upper layer is responsible for that PDU. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucFunctionNameDef |  |  |
| Length | 1–32 |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfTxPduUserTriggerTransmitName requires CanIfTxPduUserTxConfirmationUL to be either PDUR or CDD. |  |  |

| Name | CanIfTxPduUserTxConfirmationName [ECUC_CanIf_00528] |
| --- | --- |
| Parent Container | CanIfTxPduCfg |
| Description | This parameter defines the name of the <User_TxConfirmation>. This parameter depends on the parameter CanIfTxPduUserTxConfirmationUL. If CanIfTxPduUserTxConfirmationUL equals CAN_TP, CAN_NM, PDUR, XCP, CAN_TSYN, J1939NM or J1939TP, the name of the <User_TxConfirmation> is fixed. If CanIfTxPduUserTxConfirmationUL equals CDD, the name of the <User_TxConfirmation> is selectable. |
| Multiplicity | 0..1 |
| Type Default Value | EcucFunctionNameDef |
| Length | 1–32 |
| Regular Expression |  |
| Post-Build Variant Multiplicity | false |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduUserTxConfirmationUL [ECUC_CanIf_00527] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | This parameter defines the upper layer (UL) module to which the confirmation of the successfully transmitted CanTxPduId has to be routed via the <User_TxConfirmation>. This <User_TxConfirmation> has to be invoked when the confirmation of the configured CanTxPduId will be received by a Tx confirmation event from the CAN Driver module. If no upper layer (UL) module is configured, no <User_TxConfirmation> has to be called in case of a Tx confirmation event of the CanTxPduId from the CAN Driver module. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Multiplicity | CAN_NM | CAN NM |  |
|  | CAN_TP | CAN TP |  |
|  | CAN_TSYN | Global Time Synchronization over CAN |  |
|  | CDD | Complex Driver |  |
|  | J1939NM | J1939Nm |  |
|  | J1939TP | J1939Tp |  |
|  | PDUR | PDU Router |  |
|  | XCP | Extended Calibration Protocol |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduBufferRef [ECUC_CanIf_00831] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Configurable reference to a CanIf buffer configuration. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanIfBufferCfg |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTxPduRef [ECUC_CanIf_00603] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTxPduCfg |  |  |
| Description | Reference to the "global" Pdu structure to allow harmonization of handle IDs in the COM-Stack. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to Pdu |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfTTTxFrame Triggering | 0..1 | CanIfTTTxFrameTriggering is specified in the SWS TTCAN Interface and defines Frame trigger for TTCAN transmission. This container is only included and valid if TTCAN is supported by the controller, enabled (see CanIfSupportTTCAN, ECUC_CanIf_00675), and a joblist is used. |

+parameter

+destination

+literal

+literal

+literal

+parameter

+literal

+literal

+literal

+literal

+literal

+parameter

+literal

+parameter

+parameter

+parameter

+parameter

+destination

+reference

+literal

+parameter

+literal

+parameter

+parameter

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1 maxLength = 32

defaultValue = false lowerMultiplicity = 0 upperMultiplicity = 1

CanIfTxPduUserTriggerTransmitName:  EcucFunctionNameDef

CanIfTxPduTriggerTransmit:  EcucBooleanParamDef

STATIC: EcucEnumerationLiteralDef

DYNAMIC: EcucEnumerationLiteralDef

CanIfTxPduType:  EcucEnumerationParamDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfTxPduBufferRef:  EcucReferenceDef

CanIfBufferCfg:  EcucParamConfContainerDef

min = 0

max = 536870911

lowerMultiplicity = 0 upperMultiplicity = 1

defaultValue = false lowerMultiplicity = 0 upperMultiplicity = 1

CanIfTxPduCanId:  EcucIntegerParamDef

+parameter

CanIfTxPduPnFilterPdu:  EcucBooleanParamDef

min = 0

max = 3758096383

defaultValue = 3758096383 lowerMultiplicity = 0 upperMultiplicity = 1

defaultValue = true lowerMultiplicity = 1

upperMultiplicity = 1

CanIfTxPduTruncation:  EcucBooleanParamDef

CanIfTxPduCanIdMask:  EcucIntegerParamDef

+parameter

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1 maxLength = 32

symbolicNameValue = true min = 0

max = 4294967295

EcucFunctionNameDef

CanIfTxPduUserTxConfirmationName:

CanIfTxPduId:  EcucIntegerParamDef

EXTENDED_FD_CAN: EcucEnumerationLiteralDef

+literal

STANDARD_FD_CAN: EcucEnumerationLiteralDef

EXTENDED_CAN: EcucEnumerationLiteralDef

+literal

STANDARD_CAN: EcucEnumerationLiteralDef

CanIfTxPduCanIdType:  EcucEnumerationParamDef

CAN_TSYN: EcucEnumerationLiteralDef

J1939NM: EcucEnumerationLiteralDef

CDD: EcucEnumerationLiteralDef

XCP: EcucEnumerationLiteralDef

J1939TP: EcucEnumerationLiteralDef

CAN_NM: EcucEnumerationLiteralDef

lowerMultiplicity = 0 upperMultiplicity = 1

PDUR: EcucEnumerationLiteralDef

CAN_TP: EcucEnumerationLiteralDef

+literal

CanIfTxPduUserTxConfirmationUL:  EcucEnumerationParamDef

+reference

lowerMultiplicity = 0 upperMultiplicity = *

CanIfTxPduRef:  EcucReferenceDef

defaultValue = false

lowerMultiplicity = 0 upperMultiplicity = *

Pdu:  EcucParamConfContainerDef

CanIfTxPduReadNotifyStatus:  EcucBooleanParamDef

CanIfTxPduCfg:  EcucParamConfContainerDef

Figure 10.6: AR_EcucDef_CanIfTxPduCfg

##### CanIfRxPduCfg

| SWS Item | [ECUC_CanIf_00249] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfRxPduCfg |  |  |
| Parent Container | CanIfInitCfg |  |  |
| Description | This container contains the configuration (parameters) of each receive CAN L-PDU. The SHORT-NAME of "CanIfRxPduConfig" container itself represents the symolic name of Receive L-PDU. This L-SDU produces a meta data item of type CAN_ID_32. |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Configuration Parameters |  |  |  |

| Name | CanIfRxPduCanId [ECUC_CanIf_00598] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | CAN Identifier of Receive CAN L-PDUs used by the CAN Interface. Exa: Software Filtering. This parameter is used if exactly one Can Identifier is assigned to the Pdu. If a range is assigned then the CanIfRxPduCanIdRange parameter shall be used. Range: 11 Bit For Standard CAN Identifier ... 29 Bit For Extended CAN identifier |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfRxPduCanIdMask [ECUC_CanIf_00822] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | Identifier mask which denotes relevant bits in the CAN Identifier. This parameter defines a CAN Identifier range in an alternative way to CanIfRxPduCanIdRange. It identifies the bits of the configured CAN Identifier that must match the received CAN Identifier. Range: 11 bits for Standard CAN Identifier, 29 bits for Extended CAN Identifier. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
|  | 536870911 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfRxPduCanIdType [ECUC_CanIf_00596] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | CAN Identifier of receive CAN L-PDUs used by the CAN Driver for CAN L-PDU reception. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | EXTENDED_CAN | CAN 2.0 or CAN FD frame with extended identifier (29 bits) |  |
|  | EXTENDED_FD_CAN | CAN FD frame with extended identifier (29 bits) |  |
|  | EXTENDED_NO_FD_CA N | CAN 2.0 frame with extended identifier (29 bits) |  |
|  | STANDARD_CAN | CAN 2.0 or CAN FD frame with standard identifier (11 bits) |  |
|  | STANDARD_FD_CAN | CAN FD frame with standard identifier (11 bits) |  |
|  | STANDARD_NO_FD_CA N | CAN 2.0 frame with standard identifier (11 bits) |  |
|  | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfRxPduDataLength [ECUC_CanIf_00599] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | Data length of the received CAN L-PDUs used by the CAN Interface. This information is used for Data Length Check. Additionally it might specify the valid bits in case of the discrete DLC for CAN FD L-PDUs > 8 bytes. The data area size of a CAN L-PDU can have a range from 0 to 64 bytes. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 64 |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU dependency: If CanIfRxPduDataLength > 8 then CanIfRxPduCanIdType must not be STANDARD_NO_FD_CAN or EXTENDED_NO_FD_CAN |  |  |

| Name | CanIfRxPduDataLengthCheck [ECUC_CanIf_00846] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | This parameter switches the message specific data length check. True: Data length check will be executed during the reception of this PDU. False: No data length check will be executed during the reception of this PDU. |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | true |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfRxPduId [ECUC_CanIf_00597] |  |
| --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |
| Description | ECU wide unique, symbolic handle for receive CAN L-SDU. It shall fulfill ANSI/AUTOSAR definitions for constant defines. Range: 0..max. number of defined CanRxPduIds |  |
| Multiplicity | 1 |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |
| Range Default Value | 0 .. 4294967295 |  |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfRxPduReadData [ECUC_CanIf_00600] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | Enables and disables the Rx buffering for reading of received L-SDU data. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfRxPduReadNotifyStatus [ECUC_CanIf_00595] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | Enables and disables receive indication for each receive CAN L-SDU for reading its notification status. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local dependency: CanIfPublicReadRxPduNotifyStatusApi must be enabled. |  |  |

| Name | CanIfRxPduUserRxIndicationName [ECUC_CanIf_00530] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | This parameter defines the name of the <User_RxIndication>. This parameter depends on the parameter CanIfRxPduUserRxIndicationUL. If CanIfRxPduUserRxIndicationUL equals CAN_TP, CAN_NM, PDUR, XCP, CAN_TSYN, J1939NM or J1939TP, the name of the <User_RxIndication> is fixed. If CanIfRxPduUserRxIndicationUL equals CDD, the name of the <User_RxIndication> is selectable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucFunctionNameDef |  |  |
| Length | 1–32 |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfRxPduUserRxIndicationUL [ECUC_CanIf_00529] |  |
| --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |
| Description | This parameter defines the upper layer (UL) module to which the indication of the successfully received CANRXPDUID has to be routed via <User_RxIndication>. This <User_RxIndication> has to be invoked when the indication of the configured CANRXPDUID will be received by an Rx indication event from the CAN Driver module. If no upper layer (UL) module is configured, no <User_RxIndication> has to be called in case of an Rx indication event of the CANRXPDUID from the CAN Driver module. |  |
| Multiplicity | 0..1 |  |
| Type | EcucEnumerationParamDef |  |
| Range Post-Build Variant Multiplicity | CAN_NM | CAN NM |
|  | CAN_TP | CAN TP |
|  | CAN_TSYN | Global Time Synchronization over CAN |
|  | CDD | Complex Driver |
|  | J1939NM | J1939Nm |
|  | J1939TP | J1939Tp |
|  | PDUR | PDU Router |
|  | XCP | Extended Calibration Protocol |
|  | false |  |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfRxPduHrhIdRef [ECUC_CanIf_00602] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | The HRH to which Rx L-PDU belongs to, is referred through this parameter. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanIfHrhCfg |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local dependency: This information has to be derived from the CAN Driver configuration. |  |  |

| Name | CanIfRxPduRef [ECUC_CanIf_00601] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCfg |  |  |
| Description | Reference to the "global" Pdu structure to allow harmonization of handle IDs in the COM-Stack. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to Pdu |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfRxPduCanIdRange | 0..1 | Optional container that allows to map a range of CAN Ids to one PduId. |
| CanIfTTRxFrame Triggering | 0..1 | CanIfTTRxFrameTriggering is specified in the SWS TTCAN Interface and defines Frame trigger for TTCAN reception. This container is only included and valid if TTCAN is supported by the controller, enabled (see CanIfSupportTTCAN, ECUC_CanIf_00675), and a joblist is used for reception. |

lowerMultiplicity = 1 upperMultiplicity = 1

CanIfInitCfg: EcucParamConfContainerDef

CanIfRxPduCfg:  EcucParamConfContainerDef

+subContainer

lowerMultiplicity = 0 upperMultiplicity = *

+reference

defaultValue = False

CanIfRxPduReadNotifyStatus:  EcucBooleanParamDef

CanIfRxPduHrhIdRef:  +de EcucReferenceDef

+parameter

stination

CanIfHrhCfg:

EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+reference

requiresSymbolicNameValue = true

CanIfHrhIdSymRef: EcucReferenceDef

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1

maxLength = 32

CanIfRxPduUserRxIndicationName:  EcucFunctionNameDef

+parameter

+reference

lowerMultiplicity = 0 upperMultiplicity = *

Pdu:  EcucParamConfContainerDef

CanIfRxPduRef:  EcucReferenceDef

+destination

upperMultiplicity = 1 lowerMultiplicity = 0

CanIfRxPduUserRxIndicationUL:  EcucEnumerationParamDef

CAN_TP: EcucEnumerationLiteralDef

+literal

CDD: EcucEnumerationLiteralDef

+literal

+literal

CAN_NM: EcucEnumerationLiteralDef

J1939TP: EcucEnumerationLiteralDef

+literal

+parameter

PDUR: EcucEnumerationLiteralDef

+literal

XCP: EcucEnumerationLiteralDef

+literal

J1939NM: EcucEnumerationLiteralDef

+literal

CAN_TSYN: EcucEnumerationLiteralDef

+literal

EXTENDED_CAN: EcucEnumerationLiteralDef

CanIfRxPduCanIdType:  EcucEnumerationParamDef

+literal

STANDARD_CAN: EcucEnumerationLiteralDef

+literal

+literal

+parameter

STANDARD_FD_CAN: EcucEnumerationLiteralDef

EXTENDED_FD_CAN: EcucEnumerationLiteralDef

+literal

EXTENDED_NO_FD_CAN: EcucEnumerationLiteralDef

+literal

STANDARD_NO_FD_CAN: EcucEnumerationLiteralDef

+literal

+parameter

CanIfRxPduDataLengthCheck:  EcucBooleanParamDef

symbolicNameValue = true upperMultiplicity = 1

lowerMultiplicity = 1 min = 0

max = 4294967295

CanIfRxPduId:  EcucIntegerParamDef

defaultValue = true

+parameter

min = 0 max = 64

CanIfRxPduDataLength:  EcucIntegerParamDef

+parameter

min = 0

max = 536870911

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfRxPduCanId:  EcucIntegerParamDef

defaultValue = false

CanIfRxPduReadData: EcucBooleanParamDef

+parameter

+parameter

CanIfRxPduCanIdRange:

E

CanIfRxPduCanIdRangeUpperCanId:

Figure 10.7: AR_EcucDef_CanIfRxPduCfg

##### CanIfRxPduCanIdRange

| SWS Item | [ECUC_CanIf_00743] |
| --- | --- |
| Container Name | CanIfRxPduCanIdRange |
| Parent Container | CanIfRxPduCfg |
| Description | Optional container that allows to map a range of CAN Ids to one PduId. |
| Configuration Parameters |  |

| Name | CanIfRxPduCanIdRangeLowerCanId [ECUC_CanIf_00745] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCanIdRange |  |  |
| Description | Lower CAN Identifier of a receive CAN L-PDU for identifier range definition, in which all CAN Ids are mapped to one PduId. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfRxPduCanIdRangeUpperCanId [ECUC_CanIf_00744] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfRxPduCanIdRange |  |  |
| Description | Upper CAN Identifier of a receive CAN L-PDU for identifier range definition, in which all CAN Ids are mapped to one PduId. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

No Included Containers

##### CanIfDispatchCfg

| SWS Item | [ECUC_CanIf_00250] |
| --- | --- |
| Container Name | CanIfDispatchCfg |
| Parent Container | CanIf |

| Description | Callback functions provided by upper layer modules of the CanIf. The callback functions defined in this container are common to all configured CAN Driver / CAN Transceiver Driver modules. |
| --- | --- |
| Configuration Parameters |  |

| Name | CanIfDispatchUserCheckTrcvWakeFlagIndicationName [ECUC_CanIf_00791] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the name of <User_CheckTrcvWakeFlagIndication>. If CanIfDispatchUserCheckTrcvWakeFlagIndicationUL equals CAN_SM the name of <User_CheckTrcvWakeFlagIndication> is fixed. If it equals CDD, the name is selectable. If CanIfPublicPnSupport equals False, this parameter shall not be configurable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucFunctionNameDef |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserCheckTrcvWakeFlagIndicationUL, CanIfPublicPnSupport |  |  |

| Name | CanIfDispatchUserCheckTrcvWakeFlagIndicationUL [ECUC_CanIf_00792] |  |
| --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |
| Description | This parameter defines the upper layer module to which the CheckTrcvWakeFlagIndication from the Driver modules have to be routed. If CanIfPublicPnSupport equals False, this parameter shall not be configurable. |  |
| Multiplicity | 0..1 |  |
| Type | EcucEnumerationParamDef |  |
| Range Post-Build Variant Multiplicity | CAN_SM | CAN State Manager |
|  | CDD | Complex Driver |
|  | false |  |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfPublicPnSupport |  |  |

| Name | CanIfDispatchUserClearTrcvWufFlagIndicationName [ECUC_CanIf_00789] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the name of <User_ClearTrcvWufFlagIndication>. If CanIfDispatchUserClearTrcvWufFlagIndicationUL equals CAN_SM the name of <User_ClearTrcvWufFlagIndication> is fixed. If it equals CDD, the name is selectable. If CanIfPublicPnSupport equals False, this parameter shall not be configurable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucFunctionNameDef |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserClearTrcvWufFlagIndicationUL, CanIfPublicPnSupport |  |  |

| Name | CanIfDispatchUserClearTrcvWufFlagIndicationUL [ECUC_CanIf_00790] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the upper layer module to which the ClearTrcvWufFlagIndication from the Driver modules have to be routed. If CanIfPublicPnSupport equals False, this parameter shall not be configurable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Multiplicity | CAN_SM | CAN State Manager |  |
|  | CDD | Complex Driver |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfPublicPnSupport |  |  |

| Name | CanIfDispatchUserConfirmPnAvailabilityName [ECUC_CanIf_00819] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the name of <User_ConfirmPnAvailability>. If CanIfDispatchUserConfirmPnAvailabilityUL equals CAN_SM the name of <User_ConfirmPnAvailability> is fixed. If it equals CDD, the name is selectable. If CanIfPublicPnSupport equals False, this parameter shall not be configurable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucFunctionNameDef |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |

| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
| --- | --- | --- | --- |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserConfirmPnAvailabilityUL, CanIfPublicPnSupport |  |  |

| Name | CanIfDispatchUserConfirmPnAvailabilityUL [ECUC_CanIf_00820] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the upper layer module to which the ConfirmPnAvailability notification from the Driver modules have to be routed. If CanIfPublicPnSupport equals False, this parameter shall not be configurable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Multiplicity | CAN_SM | CAN State Manager |  |
|  | CDD | Complex Driver |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfPublicPnSupport |  |  |

| Name | CanIfDispatchUserCtrlBusOffName [ECUC_CanIf_00525] |
| --- | --- |
| Parent Container | CanIfDispatchCfg |
| Description | This parameter defines the name of <User_ControllerBusOff>. This parameter depends on the parameter CanIfDispatchUserCtrlBusOffUL. If CanIfDispatchUserCtrlBusOffUL equals CAN_SM the name of <User_ControllerBusOff> is fixed. If CanIfDispatchUserCtrlBusOffUL equals CDD, the name of <User_ControllerBusOff> is selectable. |
| Multiplicity | 0..1 |
| Type Default Value | EcucFunctionNameDef |
| Length | 1–32 |
| Regular Expression |  |
| Post-Build Variant Multiplicity | false |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserCtrlBusOffUL |  |  |

| Name | CanIfDispatchUserCtrlBusOffUL [ECUC_CanIf_00547] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the upper layer (UL) module to which the notifications of all ControllerBusOff events from the CAN Driver modules have to be routed via <User_ControllerBusOff>. There is no possibility to configure no upper layer (UL) module as the provider of <User_ControllerBusOff>. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | CAN_SM | CAN State Manager |  |
|  | CDD | Complex Driver |  |
|  | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfDispatchUserCtrlModeIndicationName [ECUC_CanIf_00683] |
| --- | --- |
| Parent Container | CanIfDispatchCfg |
| Description | This parameter defines the name of <User_ControllerModeIndication>. This parameter depends on the parameter CanIfDispatchUserCtrlModeIndicationUL. If CanIfDispatchUserCtrlModeIndicationUL equals CAN_SM the name of <User_ControllerModeIndication> is fixed. If CanIfDispatchUserCtrlModeIndicationUL equals CDD, the name of <User_ControllerModeIndication> is selectable. |
| Multiplicity | 0..1 |
| Type Default Value | EcucFunctionNameDef |
| Length | 1–32 |
| Regular Expression |  |

| Post-Build Variant Multiplicity | false |  |  |
| --- | --- | --- | --- |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserCtrlModeIndicationUL |  |  |

| Name | CanIfDispatchUserCtrlModeIndicationUL [ECUC_CanIf_00684] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the upper layer (UL) module to which the notifications of all ControllerTransition events from the CAN Driver modules have to be routed via <User_ControllerModeIndication>. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | CAN_SM | CAN State Manager |  |
|  | CDD | Complex Driver |  |
|  | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfDispatchUserTrcvModeIndicationName [ECUC_CanIf_00685] |
| --- | --- |
| Parent Container | CanIfDispatchCfg |
| Description | This parameter defines the name of <User_TrcvModeIndication>. This parameter depends on the parameter CanIfDispatchUserTrcvModeIndicationUL. If CanIfDispatchUserTrcvModeIndicationUL equals CAN_SM the name of <User_TrcvModeIndication> is fixed. If CanIfDispatchUserTrcvModeIndicationUL equals CDD, the name of <User_TrcvModeIndication> is selectable. |
| Multiplicity | 0..1 |
| Type Default Value | EcucFunctionNameDef |
| Length | 1–32 |
| Regular Expression |  |

| Post-Build Variant Multiplicity | false |  |  |
| --- | --- | --- | --- |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserTrcvModeIndicationUL |  |  |

| Name | CanIfDispatchUserTrcvModeIndicationUL [ECUC_CanIf_00686] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the upper layer (UL) module to which the notifications of all TransceiverTransition events from the CAN Transceiver Driver modules have to be routed via <User_TrcvModeIndication>. If no UL module is configured, no upper layer callback function will be called. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Multiplicity | CAN_SM | CAN State Manager |  |
|  | CDD | Complex Driver |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfDispatchUserValidateWakeupEventName [ECUC_CanIf_00531] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the name of <User_ValidateWakeupEvent>. This parameter depends on the parameter CanIfDispatchUserValidateWakeupEventUL. If CanIfDispatchUserValidateWakeupEventUL equals ECUM, the name of <User_ValidateWakeupEvent> is fixed. If CanIfDispatchUserValidateWakeupEventUL equals CDD, the name of <User_ValidateWakeupEvent> is selectable. |  |  |
| Multiplicity | 0..1 |  |  |
| Type Default Value | EcucFunctionNameDef |  |  |
| Length | 1–32 |  |  |
| Regular Expression |  |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: CanIfDispatchUserValidateWakeupEventUL |  |  |

| Name | CanIfDispatchUserValidateWakeupEventUL [ECUC_CanIf_00549] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfDispatchCfg |  |  |
| Description | This parameter defines the upper layer (UL) module to which the notifications about positive former requested wake up sources have to be routed via <User_ValidateWakeupEvent>. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Multiplicity | CDD | Complex Driver |  |
|  | ECUM | ECU State Manager |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |

| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
| --- | --- | --- | --- |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

+parameter

+parameter

+literal

+parameter

+literal

+parameter

+literal

+literal

+parameter

+literal

+parameter

+literal

+literal

+literal

+parameter

+literal

+parameter

+literal

+literal

+parameter

+literal

+literal

CDD: EcucEnumerationLiteralDef

lowerMultiplicity = 0 upperMultiplicity = 1

CAN_SM: EcucEnumerationLiteralDef

+literal

CanIfDispatchUserConfirmPnAvailabilityUL:  EcucEnumerationParamDef

CDD: EcucEnumerationLiteralDef

lowerMultiplicity = 0 upperMultiplicity = 1

+parameter

CAN_SM: EcucEnumerationLiteralDef

CanIfDispatchUserCheckTrcvWakeFlagIndicationUL:  EcucEnumerationParamDef

CDD: EcucEnumerationLiteralDef

lowerMultiplicity = 0 upperMultiplicity = 1

CAN_SM: EcucEnumerationLiteralDef

CanIfDispatchUserClearTrcvWufFlagIndicationUL:  EcucEnumerationParamDef

lowerMultiplicity = 0 upperMultiplicity = 1

+parameter

CanIfDispatchUserCheckTrcvWakeFlagIndicationName:  EcucFunctionNameDef

lowerMultiplicity = 0 upperMultiplicity = 1

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfDispatchUserClearTrcvWufFlagIndicationName:  EcucFunctionNameDef

+parameter

CanIfDispatchUserConfirmPnAvailabilityName:  EcucFunctionNameDef

CDD: EcucEnumerationLiteralDef

lowerMultiplicity = 0 upperMultiplicity = 1

CAN_SM: EcucEnumerationLiteralDef

CanIfDispatchUserTrcvModeIndicationUL:  EcucEnumerationParamDef

CDD: EcucEnumerationLiteralDef

lowerMultiplicity = 1 upperMultiplicity = 1

+parameter

CAN_SM: EcucEnumerationLiteralDef

CanIfDispatchUserCtrlModeIndicationUL:  EcucEnumerationParamDef

+parameter

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1 maxLength = 32

CanIfDispatchUserTrcvModeIndicationName:  EcucFunctionNameDef

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1

maxLength = 32

CanIfDispatchUserCtrlModeIndicationName:  EcucFunctionNameDef

CDD: EcucEnumerationLiteralDef

lowerMultiplicity = 0 upperMultiplicity = 1

ECUM: EcucEnumerationLiteralDef

CanIfDispatchUserValidateWakeupEventUL:  EcucEnumerationParamDef

CDD: EcucEnumerationLiteralDef

CAN_SM: EcucEnumerationLiteralDef

CanIfDispatchUserCtrlBusOffUL:  EcucEnumerationParamDef

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1 maxLength = 32

CanIfDispatchUserValidateWakeupEventName:  EcucFunctionNameDef

lowerMultiplicity = 0 upperMultiplicity = 1 minLength = 1

maxLength = 32

CanIfDispatchCfg:  EcucParamConfContainerDef

CanIfDispatchUserCtrlBusOffName:  EcucFunctionNameDef

Figure 10.8: AR_EcucDef_CanIfDispatchCfg

##### CanIfCtrlCfg

| SWS Item | [ECUC_CanIf_00546] |
| --- | --- |

| Container Name | CanIfCtrlCfg |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfCtrlDrvCfg |  |  |
| Description | This container contains the configuration (parameters) of an adressed CAN controller by an underlying CAN Driver module. This container is configurable per CAN controller. |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE, VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
|  |  | – |  |
| Configuration Parameters |  |  |  |

| Name | CanIfCtrlId [ECUC_CanIf_00647] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfCtrlCfg |  |  |
| Description | This parameter abstracts from the CAN Driver specific parameter Controller. Each controller of all connected CAN Driver modules shall be assigned to one specific ControllerId of the CanIf. Range: 0..number of configured controllers of all CAN Driver modules |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |  |
| Range Default Value | 0 .. 255 |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfCtrlWakeupSupport [ECUC_CanIf_00637] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfCtrlCfg |  |  |
| Description | This parameter defines if a respective controller of the referenced CAN Driver modules is queriable for wake up events. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfCtrlCanCtrlRef [ECUC_CanIf_00636] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfCtrlCfg |  |  |
| Description | This parameter references to the logical handle of the underlying CAN controller from the CAN Driver module to be served by the CAN Interface module. The following parameters of CanController config container shall be referenced by this link: CanControllerId, CanWakeupSourceRef Range: 0..max. number of underlying supported CAN controllers |  |  |
| Multiplicity | 1 |  |  |
| Type | Symbolic name reference to CanController |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: amount of CAN controllers |  |  |

No Included Containers

+container

+subContainer

+reference

+destination

+parameter

+parameter

defaultValue = False

CanIfCtrlWakeupSupport:  EcucBooleanParamDef

upperMultiplicity = 1 lowerMultiplicity = 1

symbolicNameValue = true min = 0

max = 255

CanControllerId:  EcucIntegerParamDef

min = 0 max = 255

symbolicNameValue = true

+parameter

CanIfCtrlId: EcucIntegerParamDef

upperMultiplicity = * lowerMultiplicity = 1

requiresSymbolicNameValue = true

upperMultiplicity = * lowerMultiplicity = 1

CanController:  EcucParamConfContainerDef

CanIfCtrlCanCtrlRef: EcucReferenceDef

CanIfCtrlCfg:  EcucParamConfContainerDef

lowerMultiplicity = 1 upperMultiplicity = *

CanIfCtrlDrvCfg:  EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

CanIf: EcucModuleDef

Figure 10.9: AR_EcucDef_CanIfCtrlCfg

##### CanIfCtrlDrvCfg

| SWS Item | [ECUC_CanIf_00253] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfCtrlDrvCfg |  |  |
| Parent Container | CanIf |  |  |
| Description | Configuration parameters for all the underlying CAN Driver modules are aggregated under this container. For each CAN Driver module a seperate instance of this container has to be provided. |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE, VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
|  |  | – |  |
| Configuration Parameters |  |  |  |

| Name | CanIfCtrlDrvInitHohConfigRef [ECUC_CanIf_00642] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfCtrlDrvCfg |  |  |
| Description | Reference to the Init Hoh Configuration |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanIfInitHohCfg |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfCtrlDrvNameRef [ECUC_CanIf_00638] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfCtrlDrvCfg |  |  |
| Description | CAN Interface Driver Reference. This reference can be used to get any information (Ex. Driver Name, Vendor ID) from the CAN driver. The CAN Driver name can be derived from the ShortName of the CAN driver module. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanGeneral |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfCtrlCfg | 1..* | This container contains the configuration (parameters) of an adressed CAN controller by an underlying CAN Driver module. This container is configurable per CAN controller. |

upperMultiplicity = 1 lowerMultiplicity = 0

CanIf: EcucModuleDef

+destination

upperMultiplicity = 1 lowerMultiplicity = 1

CanIfCtrlDrvNameRef:  EcucReferenceDef

CanGeneral:  EcucParamConfContainerDef

+container

+reference

+reference

lowerMultiplicity = 1 upperMultiplicity = *

CanIfCtrlDrvCfg:  EcucParamConfContainerDef

+destination

lowerMultiplicity = 0 upperMultiplicity = *

CanIfCtrlDrvInitHohConfigRef:  EcucReferenceDef

CanIfInitHohCfg:  EcucParamConfContainerDef

Figure 10.10: AR_EcucDef_CanIfCtrlDrvCfg

##### CanIfTrcvDrvCfg

| SWS Item | [ECUC_CanIf_00273] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfTrcvDrvCfg |  |  |
| Parent Container | CanIf |  |  |
| Description | This container contains the configuration (parameters) of all addressed CAN transceivers by each underlying CAN Transceiver Driver module. For each CAN transceiver Driver a seperate instance of this container shall be provided. |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE, VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
|  |  | – |  |
| Configuration Parameters |  |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfTrcvCfg | 1..* | This container contains the configuration (parameters) of one addressed CAN transceiver by the underlying CAN Transceiver Driver module. For each CAN transceiver a seperate instance of this container has to be provided. |

+container

+subContainer

lowerMultiplicity = 1 upperMultiplicity = *

lowerMultiplicity = 0 upperMultiplicity = *

CanIfTrcvCfg:  EcucParamConfContainerDef

CanIfTrcvDrvCfg:  EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

CanIf: EcucModuleDef

Figure 10.11: AR_EcucDef_CanIfTrcvDrvCfg

##### CanIfTrcvCfg

| SWS Item | [ECUC_CanIf_00587] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfTrcvCfg |  |  |
| Parent Container | CanIfTrcvDrvCfg |  |  |
| Description | This container contains the configuration (parameters) of one addressed CAN transceiver by the underlying CAN Transceiver Driver module. For each CAN transceiver a seperate instance of this container has to be provided. |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE, VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
|  |  | – |  |
| Configuration Parameters |  |  |  |

| Name | CanIfTrcvId [ECUC_CanIf_00654] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTrcvCfg |  |  |
| Description | This parameter abstracts from the CAN Transceiver Driver specific parameter Transceiver. Each transceiver of all connected CAN Transceiver Driver modules shall be assigned to one specific TransceiverId of the CanIf. Range: 0..number of configured transceivers of all CAN Transceiver Driver modules |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |  |
| Range Default Value | 0 .. 255 |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTrcvWakeupSupport [ECUC_CanIf_00606] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfTrcvCfg |  |  |
| Description | This parameter defines if a respective transceiver of the referenced CAN Transceiver Driver modules is queriable for wake up events. True: Enabled False: Disabled |  |  |
| Multiplicity | 1 |  |  |
| Type Default Value | EcucBooleanParamDef |  |  |
|  | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfTrcvCanTrcvRef [ECUC_CanIf_00605] |
| --- | --- |
| Parent Container | CanIfTrcvCfg |
| Description | This parameter references to the logical handle of the underlying CAN transceiver from the CAN transceiver driver module to be served by the CAN Interface module. Range: 0..max. number of underlying supported CAN transceivers |
| Multiplicity | 1 |
| Type | Symbolic name reference to CanTrcvChannel |
| Post-Build Variant Value | false |

| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
| --- | --- | --- | --- |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: ECU dependency: amount of CAN transceivers |  |  |

No Included Containers

+container

+subContainer

+parameter

+parameter

+reference

+destination

+reference

symbolicNameValue = true max = 255

lowerMultiplicity = 0 upperMultiplicity = 1

requiresSymbolicNameValue = true

CanTrcvChannelId:  EcucIntegerParamDef

CanTrcvWakeupSourceRef:  EcucReferenceDef

+parameter

upperMultiplicity = * lowerMultiplicity = 1

lowerMultiplicity = 1 upperMultiplicity = 1

requiresSymbolicNameValue = true

CanTrcvChannel: EcucParamConfContainerDef

CanIfTrcvCanTrcvRef: EcucReferenceDef

min = 0 max = 255

symbolicNameValue = true

CanIfTrcvId: EcucIntegerParamDef

defaultValue = false

lowerMultiplicity = 1 upperMultiplicity = *

CanIfTrcvWakeupSupport:  EcucBooleanParamDef

CanIfTrcvCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfTrcvDrvCfg:  EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

CanIf: EcucModuleDef

Figure 10.12: AR_EcucDef_CanIfTrcvCfg

##### CanIfInitHohCfg

| SWS Item | [ECUC_CanIf_00257] |
| --- | --- |
| Container Name | CanIfInitHohCfg |
| Parent Container | CanIfInitCfg |
| Description | This container contains the references to the configuration setup of each underlying CAN Driver. |
| Post-Build Variant Multiplicity | false |

| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE, VARIANT-LINK-TIME, VARIANT-POST-BUILD |
| --- | --- | --- | --- |
|  |  | – |  |
|  |  | – |  |
| Configuration Parameters |  |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfHrhCfg | 0..* | This container contains configuration parameters for each hardware receive object (HRH). |
| CanIfHthCfg | 0..* | This container contains parameters related to each HTH. |

+subContainer

+subContainer

lowerMultiplicity = 0 upperMultiplicity = *

CanIfHthCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

+subContainer

lowerMultiplicity = 0 upperMultiplicity = *

CanIfHrhCfg:  EcucParamConfContainerDef

CanIfInitHohCfg:  EcucParamConfContainerDef

lowerMultiplicity = 1 upperMultiplicity = 1

CanIfInitCfg: EcucParamConfContainerDef

Figure 10.13: AR_EcucDef_CanIfInitHohCfg

##### CanIfHthCfg

| SWS Item | [ECUC_CanIf_00258] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfHthCfg |  |  |
| Parent Container | CanIfInitHohCfg |  |  |
| Description | This container contains parameters related to each HTH. |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Configuration Parameters |  |  |  |

| Name | CanIfHthCanCtrlIdRef [ECUC_CanIf_00625] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHthCfg |  |  |
| Description | Reference to controller Id to which the HTH belongs to. A controller can contain one or more HTHs. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanIfCtrlCfg |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfHthIdSymRef [ECUC_CanIf_00627] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHthCfg |  |  |
| Description | The parameter refers to a particular HTH object in the CanDrv configuration (see CanHardwareObject ECUC_Can_00324). CanIf receives the following information of the CanDrv module by this reference: - CanHandleType (see ECUC_Can_00323) - CanObjectId (see ECUC_Can_00326) |  |  |
| Multiplicity | 1 |  |  |
| Type | Symbolic name reference to CanHardwareObject |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

+subContainer

+reference

+destination

+reference

+destination

+parameter

upperMultiplicity = 1 lowerMultiplicity = 1

symbolicNameValue = true min = 0

max = 65535

CanHandleType:  EcucEnumerationParamDef

CanObjectId: EcucIntegerParamDef

+parameter

upperMultiplicity = * lowerMultiplicity = 1

requiresSymbolicNameValue = true

CanHardwareObject:  EcucParamConfContainerDef

CanIfHthIdSymRef: EcucReferenceDef

lowerMultiplicity = 0 upperMultiplicity = *

upperMultiplicity = * lowerMultiplicity = 1

CanIfHthCanCtrlIdRef:  EcucReferenceDef

CanIfCtrlCfg:  EcucParamConfContainerDef

CanIfHthCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfInitHohCfg:  EcucParamConfContainerDef

Figure 10.14: AR_EcucDef_CanIfHthCfg

##### CanIfHrhCfg

| SWS Item | [ECUC_CanIf_00259] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfHrhCfg |  |  |
| Parent Container | CanIfInitHohCfg |  |  |
| Description | This container contains configuration parameters for each hardware receive object (HRH). |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Configuration Parameters |  |  |  |

| Name | CanIfHrhSoftwareFilter [ECUC_CanIf_00632] |
| --- | --- |
| Parent Container | CanIfHrhCfg |
| Description | Selects the hardware receive objects by using the HRH range/list from CAN Driver configuration to define, for which HRH a software filtering has to be performed at during receive processing. True: Software filtering is enabled False: Software filtering is enabled |
| Multiplicity | 1 |
| Type Default Value | EcucBooleanParamDef |
|  | true |

| Post-Build Variant Value | false |  |  |
| --- | --- | --- | --- |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME, VARIANT-POST-BUILD |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfHrhCanCtrlIdRef [ECUC_CanIf_00631] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHrhCfg |  |  |
| Description | Reference to controller Id to which the HRH belongs to. A controller can contain one or more HRHs. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanIfCtrlCfg |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Name | CanIfHrhIdSymRef [ECUC_CanIf_00634] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHrhCfg |  |  |
| Description | The parameter refers to a particular HRH object in the CanDrv configuration (see CanHardwareObject ECUC_Can_00324). CanIf receives the following information of the CanDrv module by this reference: - CanHandleType (see ECUC_Can_00323) - CanObjectId (see ECUC_Can_00326) |  |  |
| Multiplicity | 1 |  |  |
| Type | Symbolic name reference to CanHardwareObject |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: ECU |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| CanIfHrhRangeCfg | 0..* | Defines the parameters required for configurating multiple CANID ranges for a given same HRH. |

+subContainer

+parameter

upperMultiplicity = 1 lowerMultiplicity = 1

symbolicNameValue = true min = 0

max = 65535

CanObjectId: EcucIntegerParamDef

CanHandleType:  EcucEnumerationParamDef

lowerMultiplicity = 0 upperMultiplicity = *

+parameter

CanIfHrhRangeCfg:  EcucParamConfContainerDef

defaultValue = True

CanIfHrhSoftwareFilter:  EcucBooleanParamDef

upperMultiplicity = * lowerMultiplicity = 1

requiresSymbolicNameValue = true

+subContainer

CanHardwareObject:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfHrhCfg:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfInitHohCfg:  EcucParamConfContainerDef

+reference

CanIfHrhIdSymRef: EcucReferenceDef

+reference

+destination

+parameter

upperMultiplicity = * lowerMultiplicity = 1

CanIfHrhCanCtrlIdRef:  EcucReferenceDef

CanIfCtrlCfg:  EcucParamConfContainerDef

+destination

Figure 10.15: AR_EcucDef_CanIfHrhCfg

##### CanIfHrhRangeCfg

| SWS Item | [ECUC_CanIf_00628] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfHrhRangeCfg |  |  |
| Parent Container | CanIfHrhCfg |  |  |
| Description | Defines the parameters required for configurating multiple CANID ranges for a given same HRH. |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Configuration Parameters |  |  |  |

| Name | CanIfHrhRangeBaseId [ECUC_CanIf_00825] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHrhRangeCfg |  |  |
| Description | CAN Identifier used as base value in combination with CanIfHrhRangeMask for a masked ID range in which all CAN Ids shall pass the software filtering. The size of this parameter is limited by CanIfHrhRangeRxPduRangeCanIdType. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfHrhRangeMask [ECUC_CanIf_00826] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHrhRangeCfg |  |  |
| Description | Used as mask value in combination with CanIfHrhRangeBaseId for a masked ID range in which all CAN Ids shall pass the software filtering. The size of this parameter is limited by CanIfHrhRangeRxPduRangeCanIdType. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfHrhRangeRxPduLowerCanId [ECUC_CanIf_00629] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHrhRangeCfg |  |  |
| Description | Lower CAN Identifier of a receive CAN L-PDU for identifier range definition, in which all CAN Ids shall pass the software filtering. |  |  |
| Multiplicity | 0..1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 536870911 |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfHrhRangeRxPduRangeCanIdType [ECUC_CanIf_00644] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfHrhRangeCfg |  |  |
| Description | Specifies whether a configured Range of CAN Ids shall only consider standard CAN Ids or extended CAN Ids. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range Post-Build Variant Value | EXTENDED | All the CANIDs are of type extended only (29 bit). |  |
|  | STANDARD | All the CANIDs are of type standard only (11bit). |  |
|  | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| Name | CanIfHrhRangeRxPduUpperCanId [ECUC_CanIf_00630] |  |
| --- | --- | --- |
| Parent Container | CanIfHrhRangeCfg |  |
| Description | Upper CAN Identifier of a receive CAN L-PDU for identifier range definition, in which all CAN Ids shall pass the software filtering. |  |
| Multiplicity | 0..1 |  |
| Type | EcucIntegerParamDef |  |
| Range Default Value | 0 .. 536870911 |  |
| Post-Build Variant Multiplicity | true |  |

| Post-Build Variant Value | true |  |  |
| --- | --- | --- | --- |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

No Included Containers

+subContainer

+parameter

+literal

+literal

upperMultiplicity = 1 lowerMultiplicity = 0 min = 0

max = 536870911

CanIfHrhRangeMask:  EcucIntegerParamDef

+parameter

upperMultiplicity = 1 lowerMultiplicity = 0 min = 0

max = 536870911

CanIfHrhRangeBaseId:  EcucIntegerParamDef

+parameter

EXTENDED:

EcucEnumerationLiteralDef

CanIfHrhRangeRxPduRangeCanIdType:  EcucEnumerationParamDef

STANDARD:

EcucEnumerationLiteralDef

+parameter

upperMultiplicity = 1 lowerMultiplicity = 0 min = 0

max = 536870911

CanIfHrhRangeRxPduLowerCanId:  EcucIntegerParamDef

lowerMultiplicity = 0 upperMultiplicity = *

+parameter

min = 0

max = 536870911

lowerMultiplicity = 0 upperMultiplicity = 1

CanIfHrhRangeCfg:  EcucParamConfContainerDef

CanIfHrhRangeRxPduUpperCanId:  EcucIntegerParamDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfHrhCfg:  EcucParamConfContainerDef

Figure 10.16: AR_EcucDef_CanIfHrhRangeCfg

##### CanIfBufferCfg

| SWS Item | [ECUC_CanIf_00832] |
| --- | --- |
| Container Name | CanIfBufferCfg |
| Parent Container | CanIfInitCfg |

| Description | This container contains the Txbuffer configuration. Multiple buffers with different sizes could be configured. If CanIfBufferSize (ECUC_CanIf_00834) equals 0, the CanIf Tx L-PDU only refers via this CanIfBufferCfg the corresponding CanIfHthCfg. |  |  |
| --- | --- | --- | --- |
| Post-Build Variant Multiplicity | true |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Configuration Parameters |  |  |  |

| Name | CanIfBufferSize [ECUC_CanIf_00834] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfBufferCfg |  |  |
| Description | This parameter defines the number of CanIf Tx L-PDUs which can be buffered in one Txbuffer. If this value equals 0, the CanIf does not perform Txbuffering for the CanIf Tx L-PDUs which are assigned to this Txbuffer. If CanIfPublicTxBuffering equals False, this parameter equals 0 for all TxBuffer. If the CanHandleType of the referred HTH equals FULL, this parameter equals 0 for this TxBuffer. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range Default Value | 0 .. 255 |  |  |
|  | 0 |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local dependency: CanIfPublicTxBuffering, CanHandleType |  |  |

| Name | CanIfBufferHthRef [ECUC_CanIf_00833] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfBufferCfg |  |  |
| Description | Reference to HTH, that defines the hardware object or the pool of hardware objects configured for transmission. All the CanIf Tx L-PDUs refer via the CanIfBufferCfg and this parameter to the HTHs if TxBuffering is enabled, or not. Each HTH shall not be assigned to more than one buffer. |  |  |
| Multiplicity | 1 |  |  |
| Type | Reference to CanIfHthCfg |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | VARIANT-PRE-COMPILE |
|  |  | X | VARIANT-LINK-TIME |
|  |  | X | VARIANT-POST-BUILD |

| Scope / Dependency | scope: local |
| --- | --- |

No Included Containers

+reference

+parameter

min = 0 max = 255

defaultValue = 0

CanIfBufferSize:  EcucIntegerParamDef

lowerMultiplicity = 0 upperMultiplicity = *

CanIfBufferCfg:  EcucParamConfContainerDef

+destination

lowerMultiplicity = 0 upperMultiplicity = *

CanIfHthCfg:  EcucParamConfContainerDef

CanIfBufferHthRef:  EcucReferenceDef

Figure 10.17: AR_EcucDef_CanIfBufferCfg

##### CanIfSecurityEventRefs

| SWS Item | [ECUC_CanIf_00849] |  |  |
| --- | --- | --- | --- |
| Container Name | CanIfSecurityEventRefs |  |  |
| Parent Container | CanIfPublicCfg |  |  |
| Description | Container for the references to IdsMEvent elements representing the security events that the CanIf module shall report to the IdsM in case the coresponding security related event occurs (and if CanIfEnableSecurityEventReporting is set to "true"). The standardized security events in this container can be extended by vendor-specific security events. Tags: atp.Status=draft |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Configuration Parameters |  |  |  |

| Name | CANIF_SEV_ERRORSTATE_BUSOFF [ECUC_CanIf_00853] |
| --- | --- |
| Parent Container | CanIfSecurityEventRefs |
| Description | The CAN controller transitioned to state busoff. Tags: atp.Status=draft |
| Multiplicity | 0..1 |
| Type | Symbolic name reference to IdsMEvent |
| Post-Build Variant Multiplicity | false |
| Post-Build Variant Value | false |

| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
| --- | --- | --- | --- |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CANIF_SEV_ERRORSTATE_PASSIVE [ECUC_CanIf_00852] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfSecurityEventRefs |  |  |
| Description | A reception related error was detected. Depending on the context data this could indicate suspicious CAN activity. Tags: atp.Status=draft |  |  |
| Multiplicity | 0..1 |  |  |
| Type | Symbolic name reference to IdsMEvent |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
| --- | --- | --- | --- |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

| Name | CANIF_SEV_TX_ERROR_DETECTED [ECUC_CanIf_00850] |  |  |
| --- | --- | --- | --- |
| Parent Container | CanIfSecurityEventRefs |  |  |
| Description | A transmission related error was detected. Depending on the context data this could indicate suspicious CAN activity. Tags: atp.Status=draft |  |  |
| Multiplicity | 0..1 |  |  |
| Type | Symbolic name reference to IdsMEvent |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Value Configuration Class | Pre-compile time Link time Post-build time | X | All Variants |
|  |  | – |  |
|  |  | – |  |
| Scope / Dependency | scope: local |  |  |

No Included Containers

CanIfPublicCfg:  EcucParamConfContainerDef

+parameter CanIfEnableSecurityEventReporting:

EcucBooleanParamDef

upperMultiplicity = 1 lowerMultiplicity = 1

defaultValue = false

+subContainer

CanIfSecurityEventRefs:  EcucParamConfContainerDef

+reference

CANIF_SEV_TX_ERROR_DETECTED:

EcucReferenceDef

+destination

IdsMEvent:  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = 1

lowerMultiplicity = 0 upperMultiplicity = 1

requiresSymbolicNameValue = true

lowerMultiplicity = 1 upperMultiplicity = 65535

+reference

CANIF_SEV_RX_ERROR_DETECTED:

EcucReferenceDef

+destination

lowerMultiplicity = 0 upperMultiplicity = 1

requiresSymbolicNameValue = true

+reference

CANIF_SEV_ERRORSTATE_PASSIVE:

EcucReferenceDef

+destination

lowerMultiplicity = 0 upperMultiplicity = 1

requiresSymbolicNameValue = true

+reference

CANIF_SEV_ERRORSTATE_BUSOFF:

EcucReferenceDef

+destination

lowerMultiplicity = 0 upperMultiplicity = 1

requiresSymbolicNameValue = true

Figure 10.18: AR_EcucDef_CanIfSecurityEventRefs

(from IdsM)

## A - Not applicable requirements

[SWS_CANIF_00999] [These requirements are not applicable to this specifica-tion.(

| SRS_BSW_00159 | SRS_BSW_00167 | SRS_BSW_00170 | SRS_BSW_00416 | SRS_BSW_00168 |
| --- | --- | --- | --- | --- |
| SRS_BSW_00423 | SRS_BSW_00424 | SRS_BSW_00425 | SRS_-BSW_00426 | SRS_BSW_00427 |
| SRS_BSW_00428 | SRS_BSW_00429 | SRS_BSW_00432 | SRS_BSW_00433 | SRS_BSW_00336 |
| SRS_BSW_00417 | SRS_BSW_00164 | SRS_BSW_00007 | SRS_BSW_00307 | SRS_BSW_00373 |
| SRS_BSW_00328 | SRS_BSW_00378 | SRS_BSW_00306 | SRS_BSW_00308 | SRS_BSW_00309 |
| SRS_BSW_00330 | SRS_BSW_00172 | SRS_BSW_00010 | SRS_BSW_00341 | SRS_BSW_00334 |
| SRS_Can_01139 | SRS_Can_01014 |  |  |  |

)